| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0307 |
| --- | --- |

| Title: | CE15-related: palette index map scan order constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Xiang Li, Shan Liu, and Xiaozhong Xu | Tel: Email: | jingye@tencent.com xlxiangli@tencent.com shanl@tencent.com xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, the constraints on palette mode index map scan order are proposed. In VTM 2.0.1, there are non-square coding blocks. Some coding blocks have large height/width or width/height ratios. For these blocks, it is proposed to use one scan order. The proposed method reports negligible loss compared to CE15.2 and reduced complexity.

# Introduction

In CE15.2 [1], for each color index map, two scan orders are used, one is horizontal traverse scan and the other is vertical traverse scan. In VVC [2], there are quad-tree, binary-tree and ternary-tree block partitioning schemes, thus there are non-square coding blocks. It is observed that for those coding blocks with a ratio of height / width or width / height greater than or equal to a threshold, one scan order is sufficient. In this proposal, a threshold equal to 8 is tested. For these blocks, a scan order starting along the longer side of the block is used. For example, a 64x8 block will use horizontal traverse scan.

# Simulation results

The simulation is performed using VTM 2.0.1, under VTM configuration. The software is implemented on top of CE15.2. The mandatory common test conditions (CTC) as specified in the JVET-K1010 [3] are followed. Additional screen content testing materials as specified in CE8 [4] and CE15 [1] are also evaluated. Results are reported in the following table:

Table I: simulation results of threshold equal to 8.

# Conclusion

In this contribution, constraints on palette index map scan order are tested. It is reported that by adding the scan order constraints, the coding performance of the palette mode is preserved, and the complexity is reduced as well.

# References

[1] Y.-C. Sun, Y.-H. Chao, X. Xu, “Description of Core Experiment 15 (CE15): Palette mode”, 	JVET-K1035, Ljubljana, SL, July 2018

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2)”, ISO/IEC JTC1/SC29/WG11 JVET-K1001, July 2018.

[3]	F.Bossen, J. Boyce, X. Li, V. Sergin, and K. Suhring, ”JVET common test conditions and software reference configurations", JVET-K1010, 11th JVET meeting, July 2018.

[4] X. Xu, K. Müller, L. Wang, “Description of Core Experiment 8 (CE8): Current Picture Referencing”, JVET-K1028, Ljubljana, SL, July 2018

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).