| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0312-v1 |
| --- | --- |

| Title: | CE9.2.14: Interpolation filters in DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The memory bandwidth requirement of DMVR mode would be higher than regular inter mode at decoder side if 8-tap interpolation filter is used after MV refinement. In this contribution, 4-tap and 6-tap filters are proposed for DMVR final interpolation to reduce the memory bandwidth. It is claimed that when using 4-tap filters for the final motion compensation, the memory bandwidth of DMVR can be kept the same as regular inter mode. It is reported -1.61% and -1.64% average luma BD-rate difference using 2-tap and 4-tap filters for the motion search in DMVR. When using 6-tap filters for both motion search and final motion compensation, it is reported -1.63% average luma BD-rate difference compared with BMS-2.1-rc1 with VTM configurations.

# Introduction

Decoder side motion vector refinement (DMVR) techniques were proposed to derive motion information at decoder side. With DMVR, an initial MV pair for a block is first identified at decoder. Then the MV pair is refined within a search range. However, due to the introduction of search range, the memory bandwidth requirement for DMVR mode is higher than conventional inter mode.

# Proposed method

Figure 1 shows the memory access for interpolation in motion compensation (MC) of a conventional inter mode. For a W×H block, a (W + Tap_MC − 1) × (H + Tap_MC − 1) block needs to be fetched into memory in a generic case, where Tap_MC represents the tap of interpolation filter.

Figure 1 Memory access for interpolation in motion compensation

Figure 2 shows the memory access for a W×H block in DMVR mode, where SR represents the search range during MV refinement. From the figure, totally a (W + 2 × SR + Tap_MC − 1) × (H + 2 × SR + Tap_MC − 1) block needs to fetched into memory in a generic case if the same interpolation filter is used in both MV refinement process and final MC interpolation. Clearly, it requires higher memory bandwidth than that of conventional inter mode, especially when SR is not small.

Figure 2 Memory access for a block in DMVR

For example, the default settings in BMS-2.1 [5] have Tap_MC = 8 and SR = 2. Then the bandwidth for the conventional inter block is (W + 7) × (H + 7). When Tap_MC = 8, then the bandwidth for a DMVR block is (W + 11) × (H + 11).

In this contribution, 4-tap and 6-tap filters are proposed for the final motion compensation. When Tap_MC = 6, then the bandwidth for a DMVR block is (W + 9) × (H + 9). When Tap_MC = 4, then the bandwidth for a DMVR block is (W + 7) × (H + 7), which is the same as the conventional inter block. In addition, bilinear filters and 4-tap filters are proposed for the DMVR motion search. The 4-tap filters are DCT-based interpolation filters (DCT-IF), and the 6-tap filter is reused from BIO in BMS-2.1 [5]. The filter coefficients are listed in Table 1.

Table 1 Proposed filter coefficients

| phase | 4-tap | 6-tap |
| --- | --- | --- |
| 0/16 | {0, 64, 0, 0}/64 | {0, 0, 64, 0, 0, 0}/64 |
| 1/16 | {-2, 63, 4, -1}/64 | {1, -3, 64, 4, -2, 0}/64 |
| 2/16 | {-4, 62, 8, -2}/64 | {1, -6, 62, 9, -3, 1}/64 |
| 3/16 | {-5, 59, 13, -3}/64 | {2, -8, 60, 14, -5, 1}/64 |
| 4/16 | {-6, 56, 18, -4}/64 | {2, -9, 57, 19, -7, 2}/64 |
| 5/16 | {-6, 52, 23, -5}/64 | {3, -10, 53, 24, -8, 2}/64 |
| 6/16 | {-7, 48, 28, -5}/64 | {3, -11, 50, 29, -9, 2}/64 |
| 7/16 | {-7, 44, 33, -6}/64 | {3, -11, 44, 35, -10, 3}/64 |
| 8/16 | {-7, 39, 39, -7}/64 | {1, -7, 38, 38, -7, 1}/64 |
| 9/16 | {-6, 33, 44, -7}/64 | {3, -10, 35, 44, -11, 3}/64 |
| 10/16 | {-5, 28, 48, -7}/64 | {2, -9, 29, 50, -11, 3}/64 |
| 11/16 | {-5, 23, 52, -6}/64 | {2, -8, 24, 53, -10, 3}/64 |
| 12/16 | {-4, 18, 56, -6}/64 | {2, -7, 19, 57, -9, 2}/64 |
| 13/16 | {-3, 13, 59, -5}/64 | {1, -5, 14, 60, -8, 2}/64 |
| 14/16 | {-2, 8, 62, -4}/64 | {1, -3, 9, 62, -6, 1}/64 |
| 15/16 | {-1, 4, 63, -2}/64 | {0, -2, 4, 64, -3, 1}/64 |

For final motion compensation of a DMVR mode, if the MV refinement process does not modify its integer parts, then the original 8-tap filter is used. Otherwise, 4-tap/6-tap filter is used. In this way, one can take advantage of the 8-tap filter but does not need to fetch extra samples thus it does not increase the memory bandwidth.

# Simulation results

The proposed method is implemented on top of BMS-2.1-rc1, using VTM configurations with DMVR tool on.

## Test with bilinear search filter and 4-tap final motion compensation filter

Reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + proposed bilinear search filter + 4-tap final MC filter

It is observed from the results that the proposed method leads to -1.61% and -0.01% luma BD-rate difference on average compared with anchor without and with DMVR, respectively.

## Test with 4-tap search filter and 4-tap final motion compensation filter

Reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + proposed 4-tap search filter + 4-tap final MC filter

It is observed from the results that the proposed method leads to -1.64% and -0.01% luma BD-rate difference on average compared with anchor without and with DMVR, respectively.

## Test with 6-tap search filter and 6-tap final motion compensation filter

Reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + proposed 6-tap search filter + 6-tap final MC filter

It is observed from the results that the proposed method leads to -1.63% and 0.00% luma BD-rate difference on average compared with anchor without and with DMVR, respectively.

# Conclusions

In this contribution, a memory bandwidth reduction method is proposed for DMVR by using shorter filters. With the proposed method 4-tap filters in the final motion compensation, the memory bandwidth requirement by DMVR is kept the same as regular inter modes. It shows -1.61% and -1.64% average luma BD-rate difference with 2-tap and 4-tap motion search filters, respectively. When using 6-tap filters for both motion search and final motion compensation, it shows -1.63% average luma BD-rate difference. It is proposed to adopt the proposed method.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010.

- “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation,” S. Esenlik, Y.-W. Chen, F. Chen, JVET-K1029.

- “CE9-related: Memory bandwidth reduction for DMVR,” M. Xu, X. Li, S. Liu, JVET-K0288.

- “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, JVET-K1001.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS