| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0313-v1 |
| --- | --- |

| Title: | CE1-related: Non-square virtual pipeline data unit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Virtual pipeline data units (VPDUs) are defined in JVET-K0556 as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time; different stages process different VPDUs simultaneously. The VPDU size is roughly proportional to the buffer size in most pipeline stages. In this contribution, the VPDU is extended to cover non-square regions. It is reported that 0.22% luma BD-rate reduction is achieved on average over BMS-2.1 with VTM configurations for RA configuration.

# Introduction

Virtual pipeline data units (VPDUs) are defined in JVET-K0556 [3] as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time; different stages process different VPDUs simultaneously. The VPDU size is roughly proportional to the buffer size in most pipeline stages. Several constraints in JVET-K0556 [3] are tested in CE1 sub test 2.

In VTM-2.0.1 [4], the maximum TT size for inter frames is set to 64, which may degrade the coding performance.

# Proposed method

In this contribution, non-square VPDU is proposed. Each VPDU contains the same number of pixels so that can be processed by the pipeline engines, but it does not necessarily to have a square shape. A non-square can either be part of a CU, or can contain one or multiple CUs. The following constraints are set for the non-square VPDUs.

- For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.

- For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

With these conditions, the TT split for the 128x128 block may be implemented in VPDU, thus the maximum TT size for inter frames can be larger than 64. In this contribution, 128 is used as the maximum TT size for inter frames. The proposed method has no effect on intra slices.

# Simulation results

During the development of BMS-2.1, a bug regarding TT split is identified and fixed. To evaluate the result without influenced by the bug, BMS-2.1 with VTM configuration is used as the anchor.

Reference: BMS-2.1 (with VTM configuration)

Test: BMS-2.1 (with VTM configuration) + proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1 VTM cfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.47% | -0.26% | 121% | 101% |
| Class A2 | -0.37% | -0.21% | -0.26% | 116% | 101% |
| Class B | -0.21% | -0.17% | -0.13% | 116% | 105% |
| Class C | -0.05% | -0.07% | 0.03% | 108% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.21% | -0.14% | 115% | 103% |
| Class D | -0.03% | 0.00% | 0.02% | 109% | 108% |
| Class F (optional) | -0.04% | 0.06% | 0.06% | 112% | 103% |

The result shows 0.22% coding gain on average for luma in RA, compared with BMS-2.1 with VTM configuration. It is also reported 15% and 3% complexity increase for encoding and decoding, respectively.

# Conclusions

In this contribution, non-square VPDU is proposed. It is reported that 0.22% luma BD-rate reduction is achieved on average over BMS-2.1 with VTM configuration, for RA configuration.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010.

- “Description of Core Experiment 1 (CE1): Partitioning,” J. Ma, F. Le Léannec, M. W. Park, JVET-K1021.

- “CE1-related: CE1-related: Constraint for binary and ternary partitions,” C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, JVET-K0556.

- “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, JVET-K1001.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS