| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0314-v1 |
| --- | --- |

| Title: | CE9-related: Constrained intra prediction with decoder side motion vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Decoder side motion vector derivation (DMVD) techniques were proposed to derive motion information at decoder side after parsing stage. As intra prediction may depend on the results of its spatial neighbor’s full reconstruction, it cannot be started until the MV derivation process and the final reconstruction has finished for the previous block, which leads to serious latency in hardware pipeline. It is reported that the issue can be resolved with the method proposed in this contribution at the cost of about 0.27% luma BD-rate increase for BMS-2.1-rc1 with VTM configuration and DMVR tool on.

# Introduction

Decoder side motion vector derivation (DMVD) techniques were proposed to derive motion information at decoder side after parsing stage so that a lot of bits can be saved for motion vector (MV). These methods, such as DMVR, showed good coding performance and were studied in CE9 [2]. However, such methods lead to high dependency between the intra prediction of the current block and the final reconstruction of the previous block, which leads to pipeline latency issue in hardware design.

# Problem statement

In BMS-2.1 [5], constrained_intra_prediction_flag is signaled in the PPS, which takes value of 0 or 1. When it is equal to 1, the reconstruction samples of inter blocks are prohibited to be used as the reference in the intra prediction of a neighboring block. When it is equal to 0, such prediction is allowed.

With constrained_intra_prediction_flag equal to 0, a hypothetical decoder implementation for an intra prediction may contain three pipeline stages, as shown in Figure 1.

Figure 1 hypothetical pipeline for intra prediction

When DMVD is turned on, the decoding of an intra block may require an additional pipeline stage for the DMVD, as shown in Figure 2. Since motion search is involved in DMVD and it is quite time-consuming, it would significantly increase the pipeline latency for the intra prediction.

Figure 2 hypothetical pipeline for intra prediction with DMVD

# Proposed method

To resolve the high latency caused by the interaction of intra prediction and DMVR, it is proposed that constrained_intra_prediction_flag may be equal to 0, 1, or 2. When it is equal to 0, the intra prediction reference may come from intra or inter coded neighboring blocks. When it is equal to 1, the intra prediction reference may only come from intra coded neighboring blocks. When it is equal to 2, the intra prediction may come from intra or inter coded neighboring blocks where DMVR is not applicable.

In BMS-2.1-rc1, DMVR mode is a special merge candidate and there is no specific signaling for DMVR. Moreover, it is possible that the final MV of a block is unchanged after MV refinement. Unfortunately, it is difficult to distinguish whether MV_final would be different from MV_init without actual MV refinement. Therefore, DMVR for a neighboring block CU_neib is considered applicable if all the following conditions are true in the current implementation.

- CU_neib is in merge mode

- CU_neib is not affine

- CU_neib has two valid but different reference frames

# Simulation results

The proposed method is implemented on top of BMS-2.1-rc1 with VTM configurations and DMVR on. The methods were tested with constrained_intra_prediction_flag equal to 0, 1, and 2.

## Test with constrained_intra_prediction_flag=0

Reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + constrained_intra_prediction_flag=0

In BMS-2.1-rc1, the default value for constrained_intra_prediction_flag is 0. Thus the result of this test is the same as the second reference except for the marginal difference in the signaling of constrained_intra_prediction_flag.

## Test with constrained_intra_prediction_flag=1

Reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + constrained_intra_prediction_flag=1

When the intra prediction dependency with inter blocks is completely removed, it shows 2.15% loss on average for luma compared with BMS-2.1-rc1, and 3.84% loss compared with DMVR on.

## Test with constrained_intra_prediction_flag=2

Reference: BMS-2.1-rc1 (VTM configuration)

Second reference: BMS-2.1-rc1 (VTM configuration) + DMVR

Test: BMS-2.1-rc1 (VTM configuration) + DMVR + constrained_intra_prediction_flag=2

When only the dependency between the intra block and the DMVR coded block is removed, it still achieves 1.36% coding gain for BMS-2.1-rc1 on average for luma. When compared with DMVR on, the average relative loss on luma introduced by the proposed method with constrained_intra_prediction_flag=2 is 0.27%.

# Conclusions

An issue in the interaction of intra prediction and decoder side motion vector derivation methods is identified in this contribution, i.e., the MV refinement process imposes dependency between the intra prediction of the current block and the final reconstruction of the neighboring DMVR coded block which leads to serious delay issue in hardware pipeline design. A method to break such dependency is proposed. Simulations shows that the latency issue is solved with around 0.27% luma BD-rate increase compared with the case when this feature is disabled. It is proposed to adopt this method.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010.

- “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation,” S. Esenlik, Y.-W. Chen, F. Chen, JVET-K1029.

- “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, JVET-K1001.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS