| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0316-v2 |
| --- | --- |

| Title: | CE7-related: Reduced context models for transform coefficients coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Gao, Xiang Li, and Shan Liu | Tel: Email: | maxwellgao@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution proposes to reduce the context models for transform coefficients by sharing the context models among the transform coefficients. It is reported that the proposed method can save 12 context models in total, while keeping similar coding performance to VTM2.0.1. More specifically, the BD rate increase of the proposed method is reported as 0.02%, 0.02%, 0.02% for AI, RA and LD configurations, respectively. Another method is also proposed on the top of the first method, which can save 54 context models and the BD-Rate increase is reported as 0.05%, 0.04% and -0.04% for AI, RA and LD, respectively.

Introduction

In [1], an alternative approach for entropy coding of transform coefficient levels was proposed. In this method, the coding of transform coefficients is separated into four scan passes. In the first pass, three syntax elements sig_coeff_flag, par_level_flag and rem_abs_gt1_flag are coded; in the second pass, the syntax element rem_abs_gt2_flag is coded; in the third pass, the remaining values (referred as abs_remainder) of coefficient levels are invoked, if necessary; in the fourth pass, the sign information is coded.

Let absLevel1[x][y] represent the partially reconstructed absolute levels for coefficient at position (x, y) after the first pass, d represent the diagonal position of the current coefficient (d = x + y), numSig represent the number of non-zero coefficients in local template and sumAbs1 represent the sum of partially reconstructed absolute level absLevel1[x][y] for coefficients covered by local template. The template used is illustrated in Figure 1.

In the context modeling process, the context model index for sig_coeff_flag is determined as follows:

- If the current colour component represents the luma component, the context model index is set to

ctxIdSig = 18 * max(0, state−1) + min( sumAbs1, 5 ) + ( d < 2 ? 12 : ( d < 5 ? 6 : 0 ) )

- Otherwise, (the current colour component is the chroma component), the context index is set to

ctxIdSig = 12 * max(0, state−1) + min( sumAbs1, 5 ) + ( d < 2 ? 6 : 0 ) )

In the context modeling process, the context model index for par_level_flag is determined as follows:

- If the current scan position is equal to the position of the last non-zero level (as indicted by the transmitted x and y coordinates), ctxIdPar is set equal to 0;

- Otherwise, if the current colour component is the luma component, the context index is set to

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ( d == 0 ? 15 : ( d < 3 ? 10 : ( d < 10 ? 5 : 0 ) ) )

- Otherwise (the current colour component is the chroma component), the context index is set to

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ( d == 0 ? 5 : 0 )

The context indexes ctxIdGt1 and ctxIdGt2 for rem_abs_gt1_flag and rem_abs_gt2_flag are determined in the same way as the context index ctxIdPar:

ctxIdGt1 = ctxIdPar

ctxIdGt2 = ctxIdPar

Figure 1 Illustration of the local template.

Reduced context models for transform coefficients

For transform coefficients with d = 0, it is proposed to share the context models among the transform coefficients with sumAbs1 – numSig = 0 and sumAbs1 – numSig = 1, since the transform coefficients at DC position usually have large absolute magnitude.

For transform coefficients with d >= 10, it is proposed to share the context models among the transform coefficients with sumAbs1 – numSig = 3 and sumAbs1 – numSig = 4, since the transform coefficients within high frequency region usually have small absolute magnitude.

For Chroma component, it is proposed to share the context models among the transform coefficients with sumAbs1 – numSig = 3 and sumAbs1 – numSig = 4 in regardless of d, since the absolute magnitude of the transform coefficients in Chroma component is usually small.

The context model index for par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag in [1] can be listed as follows:

Luma:

| Diagonal position | Context model index |
| --- | --- |
| d=0 | 16, 17, 18, 19, 20 |
| d<3 | 11, 12, 13, 14, 15 |
| d<10 | 6, 7, 8, 9, 10 |
| d>=10 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |

Chroma:

| Diagonal position | Context model index |
| --- | --- |
| d=0 | 6, 7, 8, 9, 10 |
| d>0 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |

While in the proposed method, the pair of context models labelled in yellow are combined into one context model. So, for each syntax element (par_level_flag, rem_abs_gt1_flag or rem_abs_gt2_flag), the proposed method saves 4 context models. Therefore, it saves 12 context models in total.

In the second method, the context models used for par_level_flag, rem_abs_gt1_flag or rem_abs_gt2_flag is further reduced on the top of the first method by sharing the context models for transform coefficients of Luma in with 0 < d < 3 and 3<= d < 10 and transform coefficients of Chroma. In addition, the context models used for sig_coeff_flag is also reduced by sharing the context models for transform coefficients of Luma and Chroma.

More specifically, when coding par_level_flag, rem_abs_gt1_flag or rem_abs_gt2_flag, for transform coefficients with 0 < d < 3, it is proposed to share the context models among the transform coefficients with sumAbs1 – numSig = 1 and sumAbs1 – numSig = 2; for transform coefficients with 3<= d < 10, it is proposed to share the context models among the transform coefficients with sumAbs1 – numSig = 2 and sumAbs1 – numSig = 3; for transform coefficients in Chroma, it is proposed to share the context models among the transform coefficients with 2 <= sumAbs1 – numSig <= 4 in regardless of d. So the context models that are combined is shown in the following table labeled in yellow. So, it can save 8 context models for each syntax element, and can save 24 in total.

Luma:

| Diagonal position | Context model index |
| --- | --- |
| d=0 | 16, 17, 18, 19, 20 |
| d<3 | 11, 12, 13, 14, 15 |
| d<10 | 6, 7, 8, 9, 10 |
| d>=10 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |

Chroma:

| Diagonal position | Context model index |
| --- | --- |
| d=0 | 6, 7, 8, 9, 10 |
| d>0 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |

When coding sig_coeff_flag, for transform coefficients with 0 <= d < 2 or 2<= d < 5, it is proposed to share the context models among the transform coefficients with 1<= sumAbs1 <= 3; for transform coefficients with d >= 5, it is proposed to share the context models among the transform coefficients with 2 <= sumAbs1 <= 4; for transform coefficients in Chroma, it is proposed to share the context models among the transform coefficients with 3<= sumAbs1 <= 5 in regardless of d. As shown in the following table, the context models that are labeled in yellow are combined together. So, it save 10 context models for each state, and can save 30 in total.

Luma:

| Diagonal position | Context model index |
| --- | --- |
| d<2 | 12, 13, 14, 15, 16, 17 |
| d<5 | 6, 7, 8, 9, 10, 11 |
| d>=5 | 0, 1, 2, 3, 4, 5 |

Chroma:

| Diagonal position | Context model index |
| --- | --- |
| d<2 | 6, 7, 8, 9, 10, 11 |
| d>=2 | 0, 1, 2, 3, 4, 5 |

Therefore, the second method can save 54 context models in total for sig_coeff_flag, par_level_flag, rem_abs_gt1_flag and rem_abs_gt2_flag.

Simulation results

The proposed method is implemented on the top of VTM2.0.1. The common test conditions defined in [2] is used in the simulation. Table 1 and Table 2 shows the BD-rate of the first proposed method and the second proposed method relative to VTM2.0.1, respectively.

Table. 1. The BD-rate of the first proposed method relative to VTM2.0.1.

Table. 2. The BD-rate of the second proposed method relative to VTM2.0.1.

Conclusion

This contribution proposes to share the context models among the transform coefficients to reduce the context models. The first proposed method can save 12 context models in total, and the BD rate of the proposed method is 0.02%, 0.02%, 0.02% for AI, RA and LD configurations, respectively. The second method is proposed on the top of the first method, which can save 54 context models and the BD-Rate increase is reported as 0.05%, 0.04% and -0.04% for AI, RA and LD, respectively. It is proposed to adopt this method.

References

[1] H. Schwarz, T. Nguyen, D.Marpe, T. Wiegand, “Non-CE7: Alternative Entropy Coding for Dependent Quantization”, ISO/IEC JTC1/SC29/WG11 JVET-K0072, July 2018.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, July. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).