| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0318 |
| --- | --- |

| Title: | CE4 - ranking based spatial merge candidate list for inter prediction (Test CE4.4.14) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, and Shan Liu 661 Bryant St, Palo Alto, CA 94301 USA | Tel: Email: | +1-650-798-3300 guichunli@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

This contribution reports the test results of ranking based spatial merge candidate list for inter prediction, which was proposed in JVET-K0304. In test CE4.4.14, it was reported that there are 0.12% and 0.24% BD-rate gain on random access and low delay B configuration, respectively.

# Introduction

In current Versatile Video Coding Test Model (VTM-2.0.1), the positions of spatial motion vector predictors for inter merge candidates candidates are as depicted in Figure 1.

Figure 1 - Merge Candidates Currently used in VTM-2.0.1

A0 is the left-bottom candidate, A1 is the left candidate, B0 is the above-right candidate, B1 is the above candidate, and B2 is the above-left candidate.

In inter merge mode, the order of spatial merge candidates is {A1, B1, B0, A0, B2}.

# Proposed Method in JVET-K0304

In the proposed method in JVET-K0304, before constructing the merge candidate list, spatial neighboring blocks are scanned in a defined range, and the statistics of all available spatial merge candidates in the range are collected.

A histogram of the motion information is constructed, based on the count of neighboring blocks with same motion information.

In proposed method, each scanned spatial candidate block is in minimum block size (4x4 luma samples in VTM), such block will be referred to as min-block in this document. As depicted in Figure 3, the scanning range of spatial candidates includes min-blocks which are immediately adjacent to the current block (including top-left position), M min-blocks from the bottom-left neighboring block downwards, and N min-blocks from the above-right neighboring block to the right. The scanning order implemented is from bottom most position in the range to top-left corner, and then to the right most position. Since this method is ranking based, a change of initial scanning order doesn’t affect the end result, as long as all candidates in the search range are checked.

The value of M is related to the current block height H (in luma pixels). Where M = 2*H/4.

The value of N is determined by the current block width W (in luma pixels). Where N = 2*W/4.

Figure 2 – Scanning of Spatial candidates adjacent to current block and extended to more positions on left and above

After the scanning process, a histogram of neighboring blocks’ motion information is generated. Each histogram bin corresponds to an individual motion information from all the available spatial neighbors. Then all the motion information with non-zero counts in the histogram are sorted in descending order.

The motion information entries on the sorted list with largest counts will be added at the beginning of the merge candidate list, until the maximum allowed number of candidates is reached.

Since ATMVP was adopted and was included in VTM-2.0.1, a harmonization with ATMVP is applied in this test. The harmonization method used is to add ATMVP to the 4th position on the final candidate list, if there’s more than 4 candidate available on the sorted candidate list.

In case the number of candidates on the sorted list is smaller than max merge candidates, the remaining non-spatial merge candidates will be derived in the original order as in current VTM until the candidate list is fully constructed.

The maximum number of potential candidates is no more than the number of 4x4 minimum blocks, but the total number depends on the number of duplicated motion vector values, which cannot be accurately estimated. Only the upper bound of the potential candidate and the lower bound of candidate comparison are provided in the complexity analysis table.

The following complexity analysis includes spatial, temporal, combined bi-prediction candidates, and ATMVP is excluded as it doesn’t prune against regular non-sub-block candidates.

Table 3- Complexity analysis for CE4.4.14

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | <129+1 | >129 | 0 | 1 | 0 | 129+1 | sorting |

# Simulation Results

The proposed method is implemented on top of VTM-2.0.1. Simulation tests were performed with common test condition. VTM-2.0.1 with common test condition is used as anchor.

In this core experiment, VTM-2.0.1 with common test condition is used as anchor. Common test condition is also used for test CE4.4.14 with no additional conditions.

## Simulation result of test CE4.4.14

In test CE4.4.14, it was reportedly observed that there are 0.12% and 0.24% BD-rate gain on random access and low delay B configuration, respectively.

Table 5 - Test result of CE4.4.10.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.06% | -0.10% | 101% | 102% |
| Class A2 | 0.00% | -0.06% | -0.09% | 101% | 103% |
| Class B | -0.19% | -0.22% | -0.21% | 102% | 104% |
| Class C | -0.15% | -0.16% | -0.16% | 106% | 114% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.14% | -0.15% | 103% | 106% |
| Class D | 0.01% | -0.09% | -0.10% | 106% | 115% |
| Class F (optional) | -0.07% | -0.18% | -0.20% | 104% | 110% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.23% | -0.26% | 100% | 103% |
| Class C | -0.12% | -0.32% | -0.35% | 101% | 105% |
| Class E | -0.51% | -0.35% | -0.48% | 102% | 106% |
| Overall | -0.24% | -0.29% | -0.35% | 101% | 104% |
| Class D | -0.03% | 0.09% | -0.66% | 105% | 108% |
| Class F (optional) | -0.14% | 0.52% | 0.41% | 102% | 106% |

# Conclusion

It was reported that the ranking based spatial merge candidate list for inter prediction provides moderate gain in both Random Access and Low Delay B configurations. It’s proposed to adopt the proposed method in VVC and VTM software.

# References

[1] G. Li, X. Xu, X. Li, S. Liu, “CE4-related: ranking based spatial merge candidate list for inter prediction”, JVET-K0304, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[4] VTM-2.0.1 software: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-2.0.1

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).