| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0328 |
| --- | --- |

| Title: | CE7-related: modified binarization for reduced bin-to-bit ratio |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 frank@bossentech.com |
| Source: | Sharp Labs America, Inc. |  |  |

_____________________________

# Abstract

A modified coefficient level coding method is proposed. It is asserted that it results in fewer bins on average, as observed by a reduction of CABAC padding bytes. Average RD performances compared to VTM 2.0.1 are −0.25% / −0.16% / −0.29% / −0.21% for AI / RA / LB / LP configurations. Larger improvements, up to about 3%, are observed for cases where a lot of padding occurs in VTM.

# Problem Statement

It has been reported in issue #89 that a significant number of padding bytes are inserted in several test cases. The insertion of such padding bytes is the consequence of a high bin-to-bit ratio, which is undesirable. Modifications of coefficient level coding from JVET-K0072 are the source of the high bin-to-bit ratio.

# Proposal

The binarization of coefficient levels is modified such as to use fewer bins for small level values:

| Level | Significance flag | >1 flag | >2 flag | Parity flag |
| --- | --- | --- | --- | --- |
| 0 | 0 | Not coded | Not coded | Not coded |
| 1 | 1 | 0 | Not coded | Not coded |
| 2 | 1 | 1 | 0 | 1 |
| 3 | 1 | 1 | 0 | 0 |
| 4+2n | 1 | 1 | 1 | 1 |
| 5+2n | 1 | 1 | 1 | 0 |

More specifically level 1 is coded using two context-coded bins instead of previously three.

It should be noted that the worst case number of bins in unchanged (4 context-coded bins per coefficient). Furthermore the coding of the >2 flags is included into the same coding loop as other context-coded bins, thereby reducing the number of loops.

# Results

Results were obtained by implementing the proposed modification in the BMS-2.0.1 software and using the VTM configuration.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.02% | -0.05% | 102% | 103% |
| Class A2 | -1.07% | -0.63% | -0.74% | 102% | 103% |
| Class B | -0.08% | -0.02% | 0.06% | 105% | 106% |
| Class C | -0.06% | -0.15% | 0.06% | 97% | 101% |
| Class E | -0.02% | -0.13% | -0.10% | 103% | 104% |
| Overall | -0.25% | -0.17% | -0.12% | 102% | 104% |
| Class D | -0.02% | 0.00% | -0.27% | 100% | 101% |
| Class F (optional) | 0.08% | -0.07% | -0.11% | 104% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.03% | 0.00% | 105% | 104% |
| Class A2 | -0.28% | -0.33% | -0.37% | 104% | 105% |
| Class B | -0.19% | -0.29% | -0.11% | 98% | 102% |
| Class C | -0.08% | -0.16% | 0.12% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.20% | -0.08% | 100% | 102% |
| Class D | -0.06% | -0.26% | 0.21% | 97% | 100% |
| Class F (optional) | -0.04% | -0.20% | -0.31% | 98% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.30% | -0.46% | 100% | 100% |
| Class C | -0.16% | -0.09% | -0.20% | 101% | 103% |
| Class E | -0.36% | 0.32% | -0.20% | 101% | 103% |
| Overall | -0.29% | -0.08% | -0.31% | 101% | 102% |
| Class D | -0.11% | 0.69% | -0.09% | 99% | 100% |
| Class F (optional) | -0.03% | 0.14% | -0.17% | 102% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | 0.02% | 0.25% | 105% | 106% |
| Class C | -0.20% | -0.11% | -0.07% | 98% | 108% |
| Class E | -0.05% | 0.26% | 0.00% | 86% | 100% |
| Overall | -0.21% | 0.04% | 0.08% | 98% | 105% |
| Class D | -0.07% | 0.18% | -0.01% | 97% | 101% |
| Class F (optional) | -0.04% | -0.43% | -0.41% | 103% | 105% |

Outliers (with a 0.5% difference or larger) include the following test points:

- DaylightRoad2 AI -2.96%

- DaylightRoad2 RA -0.51%

- BQTerrace LB -0.94%

- KristenAndSara LB -0.58%

- BQTerrace LP -0.77%

The significant improvement in several test cases can be attributed to the avoidance of CABAC padding bytes, as fewer bins are coded.

The general improvement in coding efficiency can be attributed to the more efficient binarization.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).