| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0347 |
| --- | --- |

| Title: | CE13: De-blocking filter disabled across face discontinuities (Test 4.1.a) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JEVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Jian-Liang Lin, Hung-Chih Lin, Shen-Kai Chang, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, jl.lin, hungchih.lin, shenkai.chang, yc.zhang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 4.1.a in CE13: HEC with de-blocking filter disabled across face discontinuities. In terms of end-to-end WS-PSNR-Y/U/V, the experimental results reportedly demonstrate that HEC with de-blocking filter disabled across face discontinuities achieves an average -0.26%/-0.46%/-0.49% and -0.57%/-0.01%/0.11% BD-rate saving compared to the HEC and PHEC, respectively. In addition, the proposed method noticeably enhance subjective quality by alleviating the seam artifacts compared to HEC.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of the de-blocking filter disabled across face discontinuities proposed in JVET-K0142 [2] implemented on the hybrid equi-angular cubemap (HEC) projection [3].

The de-blocking filter in a conventional video coding system is used to remove undesired artifacts and enhance encoding quality. For a 360° video, the layout for a specific projection format usually has one or multiple discontinuous edges. Applying the de-blocking filter on these discontinuous edges directly may result in poor visual quality and coding efficiency because the filtered pixels across discontinuous edges are jointly processed (filtered). To address this problem, a 360°-based de-blocking filtering method is proposed such that the discontinuous edge can be properly processed.

# Proposed Method

### The proposed 360°-based de-blocking filtering method is a straightforward approach that disables the de-blocking filtering process when the filtered pixels cross the discontinuous edge as illustrated in Fig. 1.

Fig. 1. The de-blocking filtering is disabled when the filtered pixels cross the discontinuous edge

# Experimental Results

The test of the proposed de-blocking filtering disabled across face discontinuities is conducted on the HEC layout. The software is implemented on 360Lib-7.0 [4] and BMS-2.0.1 (with fix BMS ticket #78 and #79) [5] followed by the JVET common test conditions and evaluation procedures for 360° video [6] with VTM-2.0 [7] encoder configurations. The resolution of each face is set to 1280x1280, which yields the frame resolution is 3840x2560. As compared to HEC and PHEC, the BD-rate performance, in terms of end-to-end WS-PSNR, provided by disabling de-blocking filter across face discontinuities are listed in Table I and Table II, respectively.

Table I. BD-rate performance for Test 4.1.a compared to HEC

|  | Proposed |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| SkateboardInLot | -0.20% | -0.59% | -0.73% |
| ChairLift | -0.28% | -0.17% | -0.12% |
| KiteFlite | -0.19% | -0.16% | -0.24% |
| Harbor | -0.67% | -0.95% | -1.40% |
| Trolley | -0.17% | -0.53% | -0.21% |
| GasLamp | -0.23% | -0.69% | -0.50% |
| Balboa | -0.25% | -0.01% | -0.12% |
| Broadway | -0.29% | 0.08% | -0.21% |
| Landing2 | -0.22% | -1.53% | -1.16% |
| BranCastle2 | -0.06% | -0.09% | -0.19% |
| Class S1 | -0.29% | -0.51% | -0.53% |
| Class S2 | -0.20% | -0.39% | -0.42% |
| Overall | -0.26% | -0.46% | -0.49% |

Table II. BD-rate performance for Test 4.1.a compared to PHEC

|  | Proposed |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| SkateboardInLot | -0.17% | 0.41% | 0.42% |
| ChairLift | -0.40% | 0.07% | 0.16% |
| KiteFlite | -0.64% | -0.04% | 0.14% |
| Harbor | -0.64% | 0.55% | 0.44% |
| Trolley | -0.83% | -0.05% | -0.13% |
| GasLamp | -0.71% | -0.32% | -0.04% |
| Balboa | -0.37% | 0.02% | 0.35% |
| Broadway | -0.36% | 0.18% | 0.35% |
| Landing2 | -0.71% | -0.51% | -0.33% |
| BranCastle2 | -0.87% | -0.45% | -0.23% |
| Class S1 | -0.56% | 0.10% | 0.17% |
| Class S2 | -0.58% | -0.19% | 0.04% |
| Overall | -0.57% | -0.01% | 0.11% |

# Subjective Quality Testing

In Fig. 2 to Fig. 7, the cropped viewport images of HEC and HEC with de-blocking filter disabled across face discontinuities (Test 4.1.a) are demonstrated (under VTM-2.0 with QP = 37) in order to evaluate the seam artifacts appeared at the discontinuous edge.

Fig. 2 The subjective quality of Balboa frame 28 (HEC)

Fig. 3 The subjective quality of Balboa frame 28 (Test 4.1.a)

Fig. 4 The subjective quality of SkateboardInLot frame 8 (HEC)

Fig. 5 The subjective quality of SkateboardInLot frame 8 (Test 4.1.a)

Fig. 6 The subjective quality of ChairliftRide frame 4 (HEC)

Fig. 7 The subjective quality of ChairliftRide frame 4 (Test 4.1.a)

# Conclusion

In this contribution, the performance of the proposed de-blocking filter disabled across face discontinuities is investigated. The concept of the proposed method aims to avoid improper edge filtering at the discontinuous edge. As compared to the HEC, the proposed method can remarkably reduce the visible seam artifacts with slightly improved compression efficiency.

# References

- P. Hanhart, J.-L. Lin, and C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1033, July. 2018, Ljubljana, SI.

- S.-Y. Lin, L. Liu, C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang and C.-C. Ju, “AHG8: 360°-based in-loop filters for cubemap projection,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0142, July 2018, Ljubljana, SI.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang and C.-C. Ju,”CE13: Modified Cubemap Projection in JVET-J0019 (Test 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0131, July 2018, Ljubljana, SI.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VTM Benchmark Set, https://vcgit.hhi.fraunhofer.de/JVET-K-CE13/VVCSoftware_BMS

- P. Hanhart, J. Boyce, and K. Choi, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1012, July. 2018, Ljubljana, SI.

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).