| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0352-v1 |
| --- | --- |

| Title: | CE13: Intra prediction and in-loop filters using spherical neighbors, and geometry padding (Test 7.8.a) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng-Hsuan Shih, Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Shen-Kai Chang, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {ch.shih, Sheng-Yen.Lin, Lin.Liu, jl.lin, shenkai.chang, yc.zhang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 7.8.a in CE13: Intra prediction using spherical neighbors, geometry padding of reference pictures, and all in-loop filters using spherical neighbors. The simulation results reportedly show that, in terms of end-to-end WS-PSNR-Y/U/V, Test 7.8.a achieves the average -1.98%/-2.51%/-2.19% and -1.67%/-2.95%/-2.78% BD-rate saving compared to PHEC and HEC, respectively, while the seam artifacts caused by face discontinuities are also noticeably reduced.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of Test7.8.a, which is the combination of Intra prediction using spherical neighbors, geometry padding of reference pictures, and all in-loop filters using spherical neighbors.

# Experiment results

The test is conducted on 360Lib-7.0 [2] and BMS-2.0.1 (with fix BMS ticket #78 and #79) [3] followed by the JVET common test conditions (CTC) and evaluation procedures [4] under the VTM-2.0 [5] configuration setting. Following the CTC, the frame size is set to 3840x2560 with the face size of 1280x1280. Compared to PHEC and HEC, Table I demonstrates the average BD-rate performance in terms of end-to-end WS-PSNR for Test 7.8.a.

Table I BD-rate performance of Test 7.8.a

|  | Anchor: PHEC |  |  | Anchor: HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -3.06% | -3.26% | -2.66% | -3.09% | -4.22% | -3.76% |
| ChairLift | -3.35% | -3.40% | -3.12% | -3.23% | -3.63% | -3.40% |
| KiteFlite | -0.72% | -0.48% | -0.44% | -0.28% | -0.61% | -0.81% |
| Harbor | -0.74% | -0.70% | -0.25% | -0.77% | -2.19% | -2.09% |
| Trolley | -1.08% | -1.06% | -1.07% | -0.43% | -1.54% | -1.15% |
| GasLamp | -0.95% | -0.78% | -0.75% | -0.47% | -1.15% | -1.22% |
| Balboa | -2.74% | -4.21% | -3.21% | -2.62% | -4.24% | -3.67% |
| Broadway | -2.08% | -3.18% | -2.90% | -2.01% | -3.29% | -3.45% |
| Landing2 | -2.24% | -4.56% | -3.75% | -1.74% | -5.54% | -4.56% |
| BranCastle2 | -2.86% | -3.43% | -3.70% | -2.06% | -3.08% | -3.66% |
| Class S1 | -1.65% | -1.61% | -1.38% | -1.38% | -2.22% | -2.07% |
| Class S2 | -2.48% | -3.85% | -3.39% | -2.11% | -4.04% | -3.83% |
| Overall | -1.98% | -2.51% | -2.19% | -1.67% | -2.95% | -2.78% |

# Subjective quality testing

In Fig. 1 to Fig. 9, the resulted viewport images of HEC, PHEC, and Test 7.8.a are demonstrated (under VTM-2.0 encoder configuration with QP = 37) to evaluate the seam artifacts appeared at face discontinuities.

Fig. 1 The cropped image of Balboa frame 21 (HEC)

Fig. 2 The cropped image of Balboa frame 21 (PHEC)

Fig. 3 The cropped image of Balboa frame 21 (CE13.7.8.a)

Fig. 4 The cropped image of Broadway frame 84 (HEC)

Fig. 5 The cropped image of Broadway frame 84 (PHEC)

Fig. 6 The cropped image of Broadway frame 84 (CE13.7.8.a)

Fig. 7 The cropped image of Landing2 frame 104 (HEC)

Fig. 8 The cropped image of Landing2 frame 104 (PHEC)

Fig. 9 The cropped image of Landing2 frame 104 (CE13.7.8.a)

# Conclusion

In this contribution, the performance of Test 7.8.a is investigated. The experimental data reports that Test 7.8.a, in terms of end-to-end WS-PSNR-Y/U/V, achieves -1.98%/-2.51%/-2.19% BD-rate saving, while the visual seam artifacts are also noticeably reduced.

# Introduction

- Philippe Hanhart, Jian-Liang Lin, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-K1033, July 2018, Ljubljana, SI.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VVC software Benchmark Set, https://vcgit.hhi.fraunhofer.de/JVET-K-CE13/VVCSoftware_BMS/

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).