| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0357-v1 |
| --- | --- |

| Title: | CE14 related: Adaptive colour space clipping filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In the video coding, the samples on the YCbCr space that are transformed from the RGB space by a standard such as ITU-R BT.709 are coded. Since not all samples of the three-dimensional YCbCr space are included into original RGB space, there are some possibilities that samples outside original RGB space can be generated by coding loss. In this contribution, a loop filter called an adaptive colour space clipping filter that corrects the samples outside the color space has been proposed. It was implemented on BMS-2.01 software, and two methods that are clipping of all YCbCr values and only CbCr values have been tested with VTM configuration. As experimental results, on the RA condition, the average gain of all YCbCr clipping were -0.20%/-0.95%/-1.50% for Y/Cb/Cr respectively and the average encoding and decoding times were 100%/110%. The average gain of clipping of only CbCr are -1.07%/-1.57% for Cb/Cr respectively, the average encoding and decoding times were 100%/104%. There were high chroma gains up to 11% with several specific sequences, FoodMarket4, Campfire (Class A1), ShowGirl2 and Cosmos1 (HDR-B).

Introduction

In JEM software, the adaptive clipping [1] by max and min values on YCbCr space is adopted. It is known that there is mainly a gain of luminance. In this contribution, we focused on the color space and studied the clipping for chroma.

As a colour space for video coding currently such as ITU-R BT.709 [2] is generally used. The RGB space transformed to the YCbCr space is a parallelepiped as shown on Fig. 1. Certainly, not all values in YCbCr space are used. An idea of recovering sample values out of the range of the YCbCr space by coding loss has been proposed by Li et al. [3]. In this contribution, considering applying their idea to a realistic video coding system, we would like to propose the followings.

- Integer transformation and inverse transformation

- Clipping on RGB space

- Support to 4:2:0 format

- Adaptive switching at slice level

- Clipping of all YCbCr and clipping only CbCr

Figure 1  RGB space transformed to YCbCr space

In this contribution, the proposal of the color space clipping filter is shown in Section 2, and the experiment result is shown in Section 3. Section 4 summarizes the contents of this contribution. The appendix shows the specification document.

Colour space clipping filter

In this section, we would like to explain the proposed method introduced in the previous section. Specific specifications are shown in the appendix.

- Integer transformation and inverse transformation

Most color space conversion standards are defined by real number arithmetic. In order to use it for video coding, we define a specification of integer transformation and inverse transform for colour space. The color space clipping filter can be executed with 32-bit integer arithmetic in the coding loop.

- Clipping on RGB space

The pixel value is transformed into the YCbCr space into the RGB space, and when the value is out of the range, clipping processing is performed within each range of R, G and B. When the pixel value is clipped, the value is inverse-transformed into the YCbCr space to replace the original pixel value.

- Support for 420 format

Ideally, after up-sampling the Cb and Cr signals and converting them to the 444 format, they should be transformed into the RGB space and inversely transformed into the YCbCr space. However, the complexity of these processes is remarkably high. Therefore, we would like to propose a simplified method. In the case of transformation of luma samples, use the nearest neighbor Cb and Cr pixels and in the case of transformation of chroma samples, use the average value of the corresponding the luma 2x2 block.

- Adaptive switching at slice level

Actually, the pixel values of the input video signal are not always within the range of the standard, and the values after clipping are not always close to the pixel values of the input signal. Therefore, if color space clipping processing is fixedly performed, the PSNR with respect to the input signal may be worse. Therefore, we design a one-bit flag on the slice header to switch whether or not to perform color space clipping at the slice level.

- Clipping of all YCbCr and clipping CbCr only

As a result of the experiment, the gain on Cb and Cr are high. Further, considering the processing complexity, clipping CbCr can be realized by 1/3 processing of all YCbCr. Therefore, we would like to show the result of the method of clipping CbCr only, too.

Experimental results

The proposed method has been implemented on BMS-2.01 software and the simulation based on VTM configuration, JVET-K1010 [5] for SDR sequences and additionally JVET-K1011 [6] for HDR sequences with only fixed QP condition have been conducted. When HDR-A sequences that are used ITU-R BT.2100 are encoded, according to VUI information, the transform coefficients are changed.

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As summary of experimental results, mainly there were on average about 1% Cb and Cr gains totally and there were up to 11% gains with several following sequences.

Table 1  Results of high gain sequences

|  |  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-2.01 |  |  |  |  |  |
|  |  | YCbCr clipping |  |  | CbCr clipping |  |  |
|  |  | Y | U | V | Y | U | V |
| Class A1 | FoodMarket4 | -0.1% | -3.6% | -3.4% | 0.0% | -3.5% | -3.4% |
|  | Campfire | -1.3% | -2.9% | -10.1% | -0.1% | -3.3% | -11.2% |
| HDR-B | ShowGirl2 | -0.4% | -5.4% | -3.6% | 0.0% | -5.3% | -3.1% |
|  | Cosmos1 | -0.3% | -10.8% | -6.0% | 0.0% | -10.8% | -6.1% |

The decoding times seems to increase according to the computational complexity. Since they depend on the implementation and this contribution does not use any special speed up technologies, there will be some possibilities of improvement.

In the following sub-sections, the summary of experimental results are shown. The detailed results are included in the attached Excel files.

Results of YCbCr clipping

Table 2  Summary of results for SDR (YCbCr)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -1.19% | -2.78% | 100% | 106% |
| Class A2 | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Class B | -0.11% | -0.55% | -0.74% | 100% | 103% |
| Class C | -0.06% | -0.41% | -0.24% | 100% | 103% |
| Class E | -0.02% | -0.09% | 0.03% | 101% | 101% |
| Overall | -0.07% | -0.46% | -0.72% | 100% | 102% |
| Class D | -0.14% | -0.42% | -0.45% | 99% | 103% |
| Class F (optional) | -0.11% | -0.07% | 0.41% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -2.19% | -5.25% | 100% | 120% |
| Class A2 | 0.01% | -0.07% | -0.03% | 101% | 102% |
| Class B | -0.26% | -1.04% | -1.09% | 100% | 113% |
| Class C | -0.07% | -0.56% | -0.28% | 100% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.95% | -1.50% | 100% | 110% |
| Class D | -0.35% | -0.56% | -0.61% | 100% | 108% |
| Class F (optional) | -0.02% | -0.03% | 0.01% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | -1.99% | -1.29% | 100% | 112% |
| Class C | -0.09% | -1.23% | -0.64% | 101% | 107% |
| Class E | -0.04% | -0.46% | 0.12% | 100% | 100% |
| Overall | -0.15% | -1.36% | -0.72% | 100% | 107% |
| Class D | -0.29% | -0.40% | -0.92% | 101% | 107% |
| Class F (optional) | -0.12% | -0.20% | -0.06% | 102% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -1.73% | -1.52% | 100% | 112% |
| Class C | -0.14% | -1.39% | -0.81% | 101% | 108% |
| Class E | -0.05% | -0.57% | -0.23% | 100% | 101% |
| Overall | -0.20% | -1.33% | -0.96% | 100% | 108% |
| Class D | -0.27% | -0.57% | -1.61% | 101% | 108% |
| Class F (optional) | -0.03% | -0.16% | -0.11% | 108% | 100% |

Table 3  Summary of results for HDR (YCbCr)

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM2.0 WCG_EXT=1 |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.1% | -0.1% |
|  | FlyingBirds |  |  |  | 0.0% | 0.0% | 0.0% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | -0.1% | -0.6% |
|  | SunsetBeach |  |  |  | 0.0% | -0.1% | -0.5% |
| HDR-B | Market | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | SunRise | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | ShowGirl2 | -0.3% | -3.0% | -2.8% | -0.4% | -4.1% | -4.2% |
|  | BalloonFestival | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% |
|  | Hurdles | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Starting | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Cosmos1 | -0.2% | -4.9% | -3.8% | -0.2% | -5.6% | -4.3% |
|  | Average HDR-A |  |  |  | 0.0% | -0.1% | -0.3% |
|  | Average HDR-B | -0.1% | -1.1% | -0.9% | -0.1% | -1.4% | -1.2% |
|  | Average all |  |  |  | -0.1% | -0.9% | -0.9% |
|  | Enc Time[%] | 100% |  |  |  |  |  |
|  | Dec Time[%] | 102% |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |
|  |  | Over VTM2.0 WCG_EXT=1 |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | 0.0% | 0.2% |
|  | FlyingBirds |  |  |  | 0.0% | 0.0% | 0.0% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | -0.1% | -0.5% |
|  | SunsetBeach |  |  |  | -0.1% | -0.4% | -1.8% |
| HDR-B | Market | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | SunRise | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | ShowGirl2 | -0.3% | -4.0% | -2.7% | -0.4% | -5.4% | -3.6% |
|  | BalloonFestival | 0.0% | -0.3% | 0.2% | 0.0% | -0.5% | 0.2% |
|  | Hurdles | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Starting | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Cosmos1 | -0.2% | -9.6% | -5.8% | -0.3% | -10.8% | -6.0% |
|  | Average HDR-A |  |  |  | 0.0% | -0.1% | -0.5% |
|  | Average HDR-B | -0.1% | -2.0% | -1.2% | -0.1% | -2.4% | -1.3% |
|  | Average all |  |  |  | -0.1% | -1.6% | -1.0% |
|  | Enc Time[%] | 98% |  |  |  |  |  |
|  | Dec Time[%] | 108% |  |  |  |  |  |

Results of CbCr clipping

Table 4  Summary of results for SDR (CbCr)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -1.01% | -3.36% | 100% | 100% |
| Class A2 | 0.00% | -0.04% | -0.01% | 100% | 99% |
| Class B | 0.00% | -0.58% | -0.83% | 100% | 99% |
| Class C | 0.00% | -0.56% | -0.27% | 100% | 100% |
| Class E | 0.00% | -0.15% | 0.03% | 100% | 100% |
| Overall | 0.00% | -0.49% | -0.85% | 100% | 100% |
| Class D | 0.00% | -0.60% | -0.62% | 99% | 101% |
| Class F (optional) | 0.00% | -0.07% | 0.00% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -2.40% | -5.67% | 100% | 107% |
| Class A2 | 0.02% | -0.04% | -0.06% | 101% | 101% |
| Class B | 0.01% | -1.11% | -1.14% | 100% | 104% |
| Class C | 0.02% | -0.79% | -0.18% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -1.07% | -1.57% | 100% | 104% |
| Class D | 0.03% | -0.89% | -0.96% | 100% | 103% |
| Class F (optional) | 0.01% | -0.17% | -0.15% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -1.93% | -1.54% | 100% | 104% |
| Class C | 0.01% | -1.54% | -0.56% | 101% | 103% |
| Class E | -0.06% | -0.46% | -0.65% | 100% | 100% |
| Overall | -0.01% | -1.43% | -0.99% | 100% | 102% |
| Class D | -0.02% | -1.10% | -1.63% | 101% | 102% |
| Class F (optional) | 0.01% | -0.22% | -0.71% | 102% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.01 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -1.85% | -1.19% | 100% | 104% |
| Class C | -0.04% | -1.58% | -0.43% | 101% | 103% |
| Class E | -0.02% | -0.60% | -0.14% | 100% | 100% |
| Overall | -0.02% | -1.45% | -0.67% | 100% | 103% |
| Class D | 0.04% | -1.31% | -1.80% | 101% | 104% |
| Class F (optional) | -0.01% | -0.50% | -1.22% | 108% | 100% |

Table 5  Summary of results for HDR (CbCr)

|  |  | All Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM2.0.1 WCG_EXT=1 |  |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |  |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.1% | -0.1% |  |
|  | FlyingBirds |  |  |  | 0.0% | 0.0% | 0.0% |  |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | -0.1% | -0.6% |  |
|  | SunsetBeach |  |  |  | 0.0% | -0.1% | -0.5% |  |
| HDR-B | Market | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | SunRise | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | ShowGirl2 | 0.0% | -3.0% | -2.8% | 0.0% | -4.1% | -4.2% |  |
|  | BalloonFestival | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | 0.0% |  |
|  | Hurdles | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | Starting | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |
|  | Cosmos1 | 0.0% | -5.1% | -4.1% | 0.0% | -5.9% | -4.7% |  |
|  | Average HDR-A |  |  |  | 0.0% | -0.1% | -0.3% |  |
|  | Average HDR-B | 0.0% | -1.2% | -1.0% | 0.0% | -1.5% | -1.3% |  |
|  | Average all |  |  |  | 0.0% | -0.9% | -0.9% |  |
|  | Enc Time[%] | 100% |  |  |  |  |  |  |
|  | Dec Time[%] | 100% |  |  |  |  |  |  |
|  |  | BD-rate |  |  |  |  |  |  |
|  |  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU |  | psnrV |
| HDR-A | DayStreet |  |  |  | 0.0% | -0.1% |  | 0.0% |
|  | FlyingBirds |  |  |  | 0.0% | 0.0% |  | 0.0% |
|  | PeopleInShoppingCenter |  |  |  | 0.0% | -0.1% |  | -0.6% |
|  | SunsetBeach |  |  |  | 0.0% | -0.5% |  | -1.7% |
| HDR-B | Market | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |
|  | SunRise | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |
|  | ShowGirl2 | 0.0% | -3.9% | -2.4% | 0.0% | -5.3% |  | -3.1% |
|  | BalloonFestival | 0.0% | -0.1% | 0.2% | 0.0% | -0.3% |  | 0.1% |
|  | Hurdles | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |
|  | Starting | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |
|  | Cosmos1 | 0.0% | -9.5% | -5.9% | 0.0% | -10.8% |  | -6.1% |
|  | Average HDR-A |  |  |  | 0.0% | -0.2% |  | -0.6% |
|  | Average HDR-B | 0.0% | -1.9% | -1.2% | 0.0% | -2.3% |  | -1.3% |
|  | Average all |  |  |  | 0.0% | -1.5% |  | -1.0% |
|  | Enc Time[%] | 98% |  |  |  |  |  |  |
|  | Dec Time[%] | 103% |  |  |  |  |  |  |

Conclusion

In this contribution, the adaptive colour space clipping filter has been proposed. As experimental results, mainly there were the average about 1% chroma gains totally; however there were up to 10% with several specific sequences. The decoding times were 110%/104% for YCbCr/CbCr respectively. Since they depend on the implementation, there will be some possibilities of improvement.

Finally, we would like to recommend that the colour space clipping filter should be more studied in the next CE14.

Reference

[1]	F. Galpin, P. Bordes, F. Le Léannec, Adaptive Clipping in JEM2.0, Joint Video Experts Team Document, JVET-C0040, July 2016.

[2]	Recommendation ITU-R BT.709-6 (06/2015), Parameter values for the HDTV standards for production and international programme exchange.

[3]	Y. Li,, H. Zhao, X. Jiang and G. Wang, A novel minimum distance clip algorithm for video coding, International Conference on Signal Processing, Communication and Computing (ICSPCC 2013), Aug. 2013.

[4]	L. Zhang, S. Ikonin, Description of Core Experiment 14 (CE14): Post-reconstruction filtering, Joint Video Experts Team Document, JVET-K1010, Ljubljana, July 2018.

[5]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-K1010, Ljubljana, July 2018.

[6]	A. Segall, E. François and D. Rusanovskyy, JVET common test conditions and evaluation procedures for HDR/WCG video, Joint Video Experts Team Document, JVET-K1011, Ljubljana, July 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix

Specification

Slice header syntax and sematics

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| colour_space_clipping_filter_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

colour_space_clipping_filter_enabled_flag equal to 1 specifies that a colour space clipping filter is enabled in the current slice. colour_space_clipping_filter_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual. When not present, the value of colour_space_clipping_filter_enabled_flag is inferred to be equal to 0.

Colour space clipping filter process

General

Inputs to this process are the reconstructed picture sample array prior to colour space clipping recPictureL and the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture sample array after colour space clipping cscPictureL and the arrays cscPictureCb and cscPictureCr.

This process is performed on a slice basis after the completion of the adaptive loop filter process for the decoded picture.

The sample values in the modified reconstructed picture sample array cscoPictureL and the arrays cscPictureCb and saoPictureCr are initially set equal to the sample values in the reconstructed picture sample array recPictureL and the arrays recPictureCb and recPictureCr, respectively.

When matrix_coeffs on VUI parameters of SPS is equal to 1, 4, 5, 6, 7, 9, 12, the constants Kb, Kr and Kg (=1.0-Kb-Kr) are specified. When not present, the value of matrix_coeffs is inferred to be equal to 2.

The integer variables for colour space transformation and inverse transformation are derived as follows:

BitDepth = Max(BitDepthY, BitDepthC)

shift= 14 - Max(0, BitDepth - 12)

r1 = Round(t1)

r2 = 0

r3 = Round(t2)

r4 = Round(-t1*(16 << (BitDepth - 8)) - t2*(1 << (BitDepth - 1)))

g1 = Round(t1)

g2 = Round(t3)

g3 = Round(t4)

g4 = Round(-t1*(16 << (BitDepth - 8)) - (t3 + t4)*(1 << (BitDepth - 1)))

b1 = Round(t1)

b2 = Round(t5)

b3 = 0

y1 = Round((Kr * 219)*(1 << shift) / 255)

y2 = Round((Kg * 219)*(1 << shift) / 255)

y3 = Round((Kb * 219)*(1 << shift) / 255)

y4 = 16 << (BitDepthY - 8)

c1 = Round(((-Kr * 112)*(1 << shift)) / ((1 - Kb) * 255))

c2 = Round(((-Kg * 112)*(1 << shift)) / ((1 - Kb) * 255))

c3 = Round((((1 - Kb) * 112)*(1 << shift)) / ((1 - Kb) * 255))

c4 = 1 << (BitDepthC - 1)

c5 = Round((((1 - Kr) * 112)*(1 << shift)) / ((1 - Kr) * 255))

c6 = Round(((-Kg * 112)*(1 << shift)) / ((1 - Kr) * 255))

c7 = Round(((-Kb * 112)*(1 << shift)) / ((1 - Kr) * 255))

c8 = 1 << (BitDepthC - 1)

Where the temporal real variables are derived as follows:

t1 = (255 << shift) / 219.0

t2 = (255 << shift)*(1 - Kr) / 112.0

t3 = - (255 << shift)*Kb*(1 - Kb) / (112 * Kg)

t4 = - (255 << shift)*Kr*(1 - Kr) / (112 * Kg)

t5 = (255 << shift)*(1 - Kb) / 112.0

Colour space clipping filter process for luma sample

Inputs to this process are:

–	The reconstructed picture sample array prior to colour space clipping recPictureL

–	The Rice parameter initialization states indexed by k.

–	The palette predictor variables, PredictorPaletteSize and PredictorPaletteEntries.

Outputs of this process are the modified reconstructed picture sample array after colour space clipping cscPictureL.

The variables PicWidthInSamplesY and PicHeightInSamplesY are set equal to pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

The array cscPictureL is derived as follows:

for (y = 0; y < PicHeightInSamplesY; y ++)

for (x = 0; x < PicWidthInSamplesY; x ++)

{

iY = recPictureY [x][y] << (BitDepth - BitDepthY)

iCb = recPictureCb [x/2][y/2] << (BitDepth - BitDepthC)

iCr = recPictureCr [[x/2][y/2] << (BitDepth - BitDepthC)

iR = (r1*iY + r2*iCb + r3*iCr + r4 + (1 << (shift - 1))) >> shift

iG = (g1*iY + g2*iCb + g3*iCr + g4 + (1 << (shift - 1))) >> shift

iB = (b1*iY + b2*iCb + b3*iCr + b4 + (1 << (shift - 1))) >> shift

if (iR<0 || iR >= 1 << BitDepth || iG<0 || iG >= 1 << BitDepth || iB<0 || iB >= 1 << BitDepth)

{

iR = Clip3(0, (1 << BitDepth) - 1, iR)

iG = Clip3(0, (1 << BitDepth) - 1, iG)

iB = Clip3(0, (1 << BitDepth) - 1, iB)

cscPictureY [x][y] = ((y1*iR + y2*iG + y3*iB + (1 << (shift+BitDepth-BitDepthY-1)))

>> (shift+BitDepth-BitDepthY)) + y4

}

else

cscPictureY [x][y] = recPictureY [x][y]

}

Colour space clipping filter process for chroma sample

Inputs to this process are the reconstructed picture sample arrays prior to colour space clipping recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture sample arrays after colour space clipping cscPictureCb and cscPictureCr.

The variables PicWidthInSamplesC and PicHeightInSamplesC are set equal to pic_width_in_luma_samples / SubWidthC and pic_height_in_luma_samples / SubHeightC, respectively.

The arrays cscPictureCb and cscPictureCr are derived as follows:

for (y = 0; y < PicHeightInSamplesC; y ++)

for (x = 0; x < PicWidthInSamplesC; x ++)

{

iY = (((recPictureY [2*x][2*y]+ recPictureY [2*x+1][2*y]+ recPictureY [2*x][2*y+1]

+ recPictureY [2*x+1][2*y+1])<<(BitDepth - BitDepthY))+2)>>2

iCb = recPictureCb [x][y] << (BitDepth - BitDepthC)

iCr = recPictureCr [x][y] << (BitDepth - BitDepthC)

iR = (r1*iY + r2*iCb + r3*iCr + r4 + (1 << (shift - 1))) >> shift

iG = (g1*iY + g2*iCb + g3*iCr + g4 + (1 << (shift - 1))) >> shift

iB = (b1*iY + b2*iCb + b3*iCr + b4 + (1 << (shift - 1))) >> shift

if (iR<0 || iR >= 1 << BitDepth || iG<0 || iG >= 1 << BitDepth || iB<0 || iB >= 1 << BitDepth)

{

iR = Clip3(0, (1 << BitDepth) - 1, iR)

iG = Clip3(0, (1 << BitDepth) - 1, iG)

iB = Clip3(0, (1 << BitDepth) - 1, iB)

cscPictureCb [x][y] = ((c1*iR + c2*iG + c3*iB + (1 << (shift+BitDepth-BitDepthC-1)))

>> (shift+BitDepth-BitDepthY)) + c4

cscPictureCr [x][y] = ((c5*iR + c6*iG + c7*iB + (1 << (shift+BitDepth-BitDepthY-1)))

>>( shift+BitDepth-BitDepthY)) + c8

}

else

{

cscPictureCb [x][y] = recPictureCb [x][y]

cscPictureCr [x][y] = recPictureCr [x][y]

}

}