| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0359_r1 |
| --- | --- |

| Title: | AHG12: Flexible Tile Splitting |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | +81-43-299-8530 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp |  |  |

_____________________________

- Abstract

This contribution is relevant to the tile functionality proposed in JVET-K0155. The proposed tile allows to split a picture into flexible partitioning tiles, where the width or height of the unit of tile can be multiplies of 4 (the minimum CU size), 8, 16, 32 and 64, i.e. smaller than CTU. By using partial CTUs on the right and bottom edges of each tile, a picture can be divided into tiles of more uniform size and this functionality helps better load balancing. Furthermore, it is also asserted that this feature is useful for 360 video sequences and frame packing sequences since the corresponding tile can fit the arbitrary rectangular face size. The authors implemented the proposed tile on VTM-2.0.1 with support of LFCrossTileBoundaryFlag in Adaptive loop filter (ALF) and evaluated the performance.

The experimental results with this feature (unit=32) reportedly show that luma BD-rate coding losses on average are 0.85% % for All Intra and 1.45% % for Random Access under the common test condition (CTC) for SDR sequences excluding padding data added by CABAC.

The experimental results without this feature (i.e. HEVC like tile) reportedly show that luma BD-rate coding losses on average are 0.82 % for All Intra and 1.37 % for Random Access under the common test condition (CTC) for SDR sequences excluding padding data added by CABAC.

Introduction

In the previous document (JVET-K0155[1]), we introduced a tile splitting method with fine granularity unit, i.e. the smaller unit than CTU. The conventional tile segmentation by CTU sometimes has difficulty to fit frame packing boundaries or to do good load balancing because the unit of tile splitting is coarse. This difficulty may be more severe in VVC’s 128x128 (potentially 256x256) CTU size. In this contribution, we tested the performance of the proposed method on VTM-2.0.1.

Technical description

We propose a syntax, tile_size_unit_idc, which controls the unit of tiles. If tile_size_unit_idc is 0, 1, 2, .. the tile height and width can be 4, 8, 16, 32….

This tile_size_unit_idc decides the unit of column_width_minus1 and column_height_minus1 if uniform_spacinf_flag is equal to 0), if uniform_spacinf_flag is equal to 1, the width and height is decided depending on picture size and tile_size_unit_idc.

Figure 1 shows an example of the conventional (HEVC) 2x2 tiles where equal size splitting cannot be possible. Figure 2 shows an example of the proposed tile segmentation where the picture can be split into tiles of the same size. In the right and bottom edges of picture, there’s no CTU  size  tile, which are effectively encoded and decoded as the same as in picture boundary with boundary splitting technique.

Figure 1: Example of 2x2 segmentation (conventional method)

Figure 2: Example of 2x2 segmentation (Proposal)

Syntax

Coding tree unit syntax

| xCtbAddr = CtbAddrInRs % PicWidthInCtbsY |  |
| --- | --- |
| yCtbAddr = CtbAddrInRs / PicWidthInCtbsY |  |
| tileId = TileId[ CtbAddrRsToTs[ CtbAddrInRs ] ] |  |
| col = tileId % ( num_tile_columns_minus1 + 1 ) |  |
| row = tileId / ( num_tile_columns_minus1 + 1 ) |  |
| xCtb = colX[ col ] + (xCtbAddr – colBd[col]) << CtbLog2SizeY |  |
| yCtb = rowY[ row ] + (yCtbAddr – rowBd[row]) << CtbLog2SizeY |  |
| … |  |

General Picture Parameter set RBSP syntax

| … |  |
| --- | --- |
| if ( tile_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| tile_size_unit_idc | ue(v) |
| if ( !uniform_spacing_flag ) { |  |
| for ( i=0; i < num_tile_columns_minus1: i++ ) |  |
| column_width_minus1[i] | ue(v) |
| for ( i=0; i < num_tile_rows_minus1: i++ ) |  |
| row_height_minus1[i] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| … |  |

Implementation

In order to implement the proposed tile partitioning, we made several modifications to support basic tile functionality as well as the flexible features on VTM-2.0.1.

Support of LFCrossTileBoundaryFlag by Aadaptive Loop Filter

VTM-2.0 unofficially supports the HEVC-style tiles when the macro HEVC_TILES_WPP is enabled in TypeDef.h. With HEVC_TILES_WPP enabled, a VTM-2.0 encoder has ‘—LFCrossTileBoundaryFlag’ option, which can control the behavior of loop filters on tile boundaries. However, Adaptive lop filter (ALF) has not supported the flag yet.

We therefore modified VTM’s ALF implementation to support LFCrossTileBoundaryFlag. Specifially, a temporary pixel buffer is employed to expand a tile area with padding pixels in order to avoid to refer pixels from other tiles for filtering.

Partitioning in tile boundaries

In  the proposed method, boundary partitioning is also performed in the right or bottom most tile. We visualized actual partitioning in tile boundary areas and confirmed that the boundary partitioning mechanism of VTM-2 appears to work well also on tile boundaries.

Figure 3 shows an example of partitioning in POC 8 of BQMall with RandomAccess, QP=37 and 2x2 uniform spacing tiles configuration. The red, white and black lines in Figure 3 depict tile boundaries, CTU boundaries and CU boundaries, respectively.

Figure 3: Example of partitioning from POC 8, BQMall (QP37, Random Access, 2x2 tiles)

Experimental results

The authors implemented the proposed method on VTM-2.0.1 with HEVC_TILES_WPP=1.

Other definitions for HEVC tools were set to zero.

The performance of the tile system for five cases with 2x2 splitting and the uniform spacng mode enabled  (--NumTileColumnsMinus1=1 --NumTileRowsMinus1=1 --UniformSpacingFlag=1) are tested. The other conditions are in accordance with the JVET CTC for SDR.

The tested five settings are as follows:

Test0: 128x128 block based tile (conventional)

Test1: 8x8 block based tile

Test2: 16x16 block based tile

Test3: 32x32 block based tile

The resulting size of 2x2 tiles for classA-C with each setting are described in Table 1.

Table 1: Resulting tile sizes in each test

|  | classA (3840x2160) | classB (1920x1080) | classC (832x480) |
| --- | --- | --- | --- |
| Test0 | (1920+1920) x (1024+1136) | (896+1024) x (512+568) | (384+448) x (256+224) |
| Test1 | (1920+1920) x (1080+1080) | (960+960) x (536+544) | (416+416) x (240+240) |
| Test2 | (1920+1920) x (1072+1088) | (960+960) x (544+536) | (416+416) x (240+240) |
| Test3 | (1920+1920) x (1088+1072) | (960+960) x (544+536) | (416+416) x (224+256) |

Table 2 shows the summary of results of Test0-3 for Intra only and Random Access conditions. The reference is VTM-2.0.1 and the proposed software is based on VTM-2.0.1 with HEVC_TILES_WPP macro is enabled.

Note that the tested bitstreams encoded by the proposed software does not include any padding data added by the CABAC engine, while the reference does. The most prominent example is DaylightRoad2 QP22 (AllIntra). More than 10% data in a bitstream are paddings. This is why ClassA2 shows a outstanding gain in Table2-5. We confirmed those paddings are not added because of a software bug when tiles are used on VTM-2.0.1 encoder built with HEVC_TILES_WPP enabled. Table 3 shows the performance without paddings.

Table 2: Summary of results of Test 0-3

Table 3: Summary of results of Test 0-3 (without padding in reference)

References

[1] Y. Yasugi, T. Ikai, “AHG12: Flexible Tile Partitioning” JVET-K0155, Jul. 2018, Ljubljana, SI.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).