INTERNATIONAL  STANDARD

ISO/IEC VVC

ITU-T Rec. H.VVC

ITU-T  RECOMMENDATION

Versatile video coding

Definitions

  - tile: A rectangular region of CTBs within a particular tile column and a particular tile row in a picture.

  - tile column: A rectangular region of CTBs having a height equal to the height of the picture and a width specified by syntax elements in the picture parameter set.

  - tile row: A rectangular region of CTBs having a height specified by syntax elements in the picture parameter set and a width equal to the width of the picture.

  - tile scan: A specific sequential ordering of CTBs partitioning a picture in which the CTBs are ordered consecutively in CTB raster scan in a tile whereas tiles in a picture are ordered consecutively in a raster scan of the tiles of the picture.

- Bitstream and picture formats, partitionings, scanning processes and neighbouring relationships

  - Partitioning of pictures, slices, tiles, CTUs

Partitioning of pictures into slices and tiles

This subclause specifies how a picture is partitioned into slices. Pictures are divided into slices and tiles. A slice is a a sequence of CTUs. Likewise, a tile is a sequence of CTUs.

For example, a picture may be divided into two slices as shown in Figure 6-4. In this example, the first slice contains 60 CTUs and the second slice contains the remaining 39 CTUs of the picture.

Unlike slices, tiles are always rectangular. A tile always contains an integer number of CTUs, and may consist of CTUs contained in more than one slice. Similarly, a slice may consist of CTUs contained in more than one tile.

When a picture is coded using three separate colour planes (separate_colour_plane_flag is equal to 1), a slice contains only CTUs of one colour component being identified by the corresponding value of colour_plane_id, and each colour component array of a picture consists of slices having the same colour_plane_id value. Coded slices with different values of colour_plane_id within a picture may be interleaved with each other under the constraint that for each value of colour_plane_id, the coded slice NAL units with that value of colour_plane_id shall be in the order of increasing CTU address in raster scan order for the first CTU of each coded slice NAL unit.

Availability processes

[Ed. (BB): Define appropriate availability checking process.]

Allowed binary split process

Input to this process is a binary split mode btSplit, a coding block width cbWidth, a coding block height cbHeight, a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture, a multi-type tree depth mttDepth, a MaxMttDepth offset depthOffset and a partition index partIdx.

Output of this process is the variable allowBtSplit.

Table 6-2 – Specification of parallelTtSplit, perpCbPos, perpCbSize, perpMaxPicSize and cbSize based on btSplit.

|  | btSplit = = SPLIT_BT_VER | btSplit = = SPLIT_BT_HOR |
| --- | --- | --- |
| parallelTtSplit | SPLIT_TT_VER | SPLIT_TT_HOR |
| cbSize | cbWidth | cbHeight |

The variables parallelTtSplit and cbSize are derived as specified in  Table 6-2.

The variable allowBtSplit is derived as follows:

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- cbSize is less than or equal to MinBtSizeY

- cbWidth is greater than MaxBtSizeY

- cbHeight is greater than MaxBtSizeY

- mttDepth is greater than or equal to MaxMttDepth + depthOffset

- Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_VER

- y0 + cbHeight is greater than TileY[ CurrTileId ] + TileHeight[ CurrTileId ]

- Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_HOR

- x0 + cbWidth is greater than TileX[ CurrTileId ] + TileWidth[ CurrTileId ]

- y0 + cbHeight is smaller than or equal to TileY[ CurrTileId ] + TileHeight[ CurrTileId ]

- Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE:

- mttDepth is greater than 0

- partIdx is equal to 1

- MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit

–	Otherwise, allowBtSplit is set equal to TRUE.

Allowed ternary split process

Input to this process is a ternary split mode ttSplit, a coding block width cbWidth, a coding block height cbHeight, a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture, a multi-type tree depth mttDepth and a MaxMttDepth offset depthOffset.

Output of this process is the variable allowTtSplit.

Table 6-3 – Specification of cbSize based on ttSplit.

|  | ttSplit = = SPLIT_TT_VER | ttSplit = = SPLIT_TT_HOR |
| --- | --- | --- |
| cbSize | cbWidth | cbHeight |

The variable cbSize is derived as specified in Table 6-3.

The variable allowTtSplit is derived as follows:

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- cbSize is less than or equal to 2 * MinTtSizeY

- cbWidth is greater than MaxTtSizeY

- cbHeight is greater than MaxTtSizeY

- mttDepth is greater than or equal to MaxMttDepth + depthOffset

- x0 + cbWidth is greater than TileX[ CurrTileId ] + TileWidth[ CurrTileId ]

- y0 + cbHeight is greater than TileY[ CurrTileId ] + TileHeight[ CurrTileId ]

–	Otherwise, allowTtSplit is set equal to TRUE.

Scanning processes

CTB raster and scanning process

[Ed. (BB): Define appropriate scanning process.]

The list colWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of tile unit blocks, is derived as follows:

if( uniform_spacing_flag )
	for( i = 0; i  <=  num_tile_columns_minus1; i++ )
		posR = ( ( i + 1 ) * PicWidthInTileUnitsY ) / ( num_tile_columns_minus1 + 1 )
		posL =  ( i     * PicWidthInTileUnitsY ) / ( num_tile_columns_minus1 + 1 )
		colWidth[ i ] = min(PicWidthInTileUnitsY – posL, posR – posL)
else {
	colWidth[ num_tile_columns_minus1 ] = PicWidhInTileUnitsY	(6-3)
	for( i = 0; i < num_tile_columns_minus1; i++ ) {
		colWidth[ i ] = column_width_minus1[ i ] + 1
		colWidth[ num_tile_columns_minus1 ]  −=  colWidth[ i ]
	}
}

The list rowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of tile unit blocks, is derived as follows:

if( uniform_spacing_flag )
	for( j = 0; j  <=  num_tile_rows_minus1; j++ )
		posB = ( ( j + 1 ) * PicHeightInTileUnitsY ) / ( num_tile_rows_minus1 + 1 )
		posT =  ( j     * PicHeightInTileUnitsY ) / ( num_tile_rows_minus1 + 1 )
		rowHeight [ j ] = min(PicHeightInTileUnitsY – posT, posB – posT )
else {
	rowHeight[ num_tile_rows_minus1 ] = PicHeightInTileUnitsY	(6-4)
	for( j = 0; j < num_tile_rows_minus1; j++ ) {
		rowHeight[ j ] = row_height_minus1[ j ] + 1
		rowHeight[ num_tile_rows_minus1 ]  −=  rowHeight[ j ]
	}
}

The list TileColX[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the X location of the top-left luma sample of the i-th tile column in units of luma samples, is derived as follows:

for ( TileColX[ 0 ] = 0, i = 0; i <= num_tile_columns_minus1; i++ )

TileColX[ i + 1 ] = TileColX[ i ] + colWidth[ i ] * TileUnitSizeY

The list TileRowY[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the Y location of the top-left luma sample of the j-th tile row in units of luma samples, is derived as follows:

for ( TileRowY[ 0 ] = 0, j = 0; j <= num_tile_rows_minus1; j++ )

TileRowY[ j + 1 ] = TileRowY[ j ] + rowHeight[ j ] * TileUnitSizeY

The list colBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th tile column left boundary in units of CTBs, is derived as follows:

for( colBd[ 0 ] = 0, i = 0; i  <=  num_tile_columns_minus1; i++ )
		colBd[ i + 1 ] = colBd[ i ] + Ceil( colWidth[ i ] * TileUnitSizeY ÷ CtbSizeY )		(6-5)

The list rowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th tile row top boundary in units of CTBs, is derived as follows:

for( rowBd[ 0 ] = 0, j = 0; j  <=  num_tile_rows_minus1; j++ )
	rowBd[ j + 1 ] = rowBd[ j ] + Ceil( rowHeight[ j ] * TileUnitSizeY ÷ CtbSizeY )		(6-6)

The list TileWidth, TileHeight, TileWidthInCtbsY, TileHeightInCtbsY, TileX, TileY, TileXInCtbsY and TileYInCtbsY for i ranging from 0 to NumTilesInPic – 1, inclusive, are derived as follows:

for(i = 0; i < NumTilesInPic; i++ ) {
	col = i % ( num_tile_columns_minus1 + 1 )
	row = i / ( num_tile_columns_minus1 + 1 )
	TileWidth [ i ] = colWidth[ col ] * TileUnitSizeY
	TileHeight [ i ] = rowHeight[ row ] * TileUnitSizeY
	TileWidthInCtbsY[ i ] = Ceil( TileWidth[ i ] ÷ CtbSizeY )
	TileHeightInCtbsY[ i ] = Ceil( TileHeight[ i ] ÷ CtbSizeY )
	TileX[ i ] = TileColX[ col ]
	TileY[ i ] = TileRowY[ row ]
	TileXInCtbsY[ i ] = colBd[ col ]
	TileYInCtbsY[ i ] = rowBd[ row ]
}

The list TileId[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID, is derived as follows:

for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1; j++ )
	for( i = 0; i  <=  num_tile_columns_minus1; i++, tileIdx++ )
		for( y = TileYInCtbsY[ j ]; y < TileYInCtbsY[ j ] + TileHeightInCtbsY[ i ]; y++ )	(6-9)
			for( x = TileXInCtbsY[ i ]; x < TileXInCtbsY[ i ] + TileWidthInCtbsY[ i ]; x++ )
				TileId[ y * PicWidthInCtbsY+ x ] = tileIdx

Ｔｈｅ list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbs; ctbAddrRs++ ) {
	tileId = TileId[ ctbAddrRs ]
	tbX = ctbAddrRs % PicWidthInCtbsY
	tbY = ctbAddrRs / PicWidthInCtbsY
	CtbAddrRsToTs[ ctbAddrRs ] = 0
	for( t = 0; t < tileId; t++ ) {
		CtbAddrRsToTs[ ctbAddrRs ] += TileWidthInCtbsY[ tileId ] * TileHeightInCtbsY[ tileId ]
	}
	CtbAddrRsToTs[ ctbAddrRs ]  +=  ( tbY − TileYInCtbsY[ tileId ] ) * TileWidthInCtbsY[ tileId ] + tbX − TileXInCtbsY[ tileX ]
}

The list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbs; ctbAddrRs++ )	(6-8)
	CtbAddrTsToRs[ CtbAddrRsToTs[ ctbAddrRs ] ] = ctbAddrRs

Syntax and semantics

  - Syntax in tabular form

    - Raw byte sequence payloads, trailing bits and byte alignment syntax

      - Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| tile_unit_size_idc | ue(v) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Slice header syntax

General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( tile_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

Slice data syntax

General slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| do { |  |
| coding_tree_unit( ) |  |
| end_of_slice_flag | ae(v) |
| CtbAddrInTs++ |  |
| CtbAddrInRs = CtbAddrTsToRs[ CtbAddrInTs ] |  |
| if( !end_of_slice_flag && tiles_enabled_flag && TileId[ CtbAddrInTs ] != TileId[ CtbAddrInTs − 1 ]) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| } |  |
| } while( !end_of_slice_flag ) |  |
| } |  |

Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| rx = CtbAddrInRs % PicWidthInCtbsY |  |
| ry = CtbAddrInRs / PicWidthInCtbsY |  |
| if ( tile_enabled_flag ){ |  |
| CurrTileId = TileId[ CtbAddrInRs ] |  |
| xCtb = TileX[ CurrTileId ] + (rx – TileXInCtbsY[ CurrTileId ]) << CtbLog2SizeY |  |
| yCtb = TileY[ CurrTileId ] + (ry – TileYInCtbsY[ CurrTileId ]) << CtbLog2SizeY |  |
| } else { |  |
| xCtb = rx << CtbLog2SizeY |  |
| yCtb = ry << CtbLog2SizeY |  |
| } |  |
| if( slice_alf_enable_flag ){ |  |
| alf_ctb_flag[ 0 ][ rx ][ ry ] | ae(v) |
| if( alf_chroma_ctb_present_flag ) { |  |
| if( alf_chroma_idc = = 1 ¦ ¦ alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 1 ][ rx ][ ry ] | ae(v) |
| if( alf_chroma_idc = = 2 ¦ ¦ alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 2 ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbLog2SizeY, 0 ) |  |
| else |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0, SINGLE_TREE ) |  |
| } |  |

| dual_tree_implicit_qt_split( x0, y0, log2CbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| if( log2CbSize > 6 ) { |  |
| x1 = x0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| dual_tree_implicit_qt_split( x0, y0, log2CbSize − 1, cqtDepth + 1 ) |  |
| if( x1 – TileX[ CurrTileId ] < TileWidth[ CurrTileId ]) |  |
| dual_tree_implicit_qt_split( x1, y0, log2CbSize − 1, cqtDepth + 1 ) |  |
| if( y1 – TileY[ CurrTileId ] < TileHeight[ CurrTileId ] ) |  |
| dual_tree_implicit_qt_split( x0, y1, log2CbSize − 1, cqtDepth + 1 ) |  |
| if( x1 – TileX[ CurrTileId ] < TileWidth[ CurrTileId ] && y1 – TileY[ CurrTileId ] < TileHeight[ CurrTileId ] ) |  |
| dual_tree_implicit_qt_split( x1, y1, log2CbSize − 1, cqtDepth + 1 ) |  |
| } else { |  |
| coding_quadtree( x0, y0, log2CbSize, cqtDepth, DUAL_TREE_LUMA ) |  |
| coding_quadtree( x0, y0, log2CbSize, cqtDepth, DUAL_TREE_CHROMA ) |  |
| } |  |
| } |  |

Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, cqtDepth, treeType ) { | Descriptor |
| --- | --- |
| if( ( ( ( x0 + ( 1 << log2CbSize ) – TileX[ CurrTileId ] <= TileWidth [ CurrTileId ] ) ? 1 : 0 ) + ( ( y0 + ( 1 << log2CbSize ) – TileY[ CurrTileId ] <= TileHeight [ CurrTileId ] ) ? 1 : 0 ) + ( ( ( 1 << log2CbSize ) <= MaxBtSizeY ) ? 1 : 0 ) ) >= 2 && log2CbSize > MinQtLog2SizeY ) |  |
| qt_split_cu_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( qt_split_cu_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| y1 = y0 + ( 1 << ( log2CbSize − 1 ) ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( x1– TileX[ CurrTileId ] < TileWidth [ CurrTileId ] ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( y1 – TileY[ CurrTileId ]< TileHeight[ CurrTileId ] ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| if( x1– TileX[ CurrTileId ] < TileWidth [ CurrTileId ] && y1– TileY[ CurrTileId ] < TileHeight[ CurrTileId ] ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, cqtDepth + 1, treeType ) |  |
| } else |  |
| multi_type_tree( x0, y0, 1 << log2CbSize, 1 << log2CbSize, 0, 0, 0, treeType ) |  |
| } |  |

Multi-type tree syntax

| multi_type_tree( x0, y0, cbWidth, cbHeight, mttDepth, depthOffset, partIdx, treeType ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && ( x0 + cbWidth – TileX[ CurrTileId ] <= TileWidth [ CurrTileId ] ) && (y0 + cbHeight – TileY[ CurrTileId ] <= TileHeight [ CurrTileId ] ) ) |  |
| mtt_split_cu_flag | ae(v) |
| if( mtt_split_cu_flag ) { |  |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  |
| mtt_split_cu_vertical_flag | ae(v) |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  |
| mtt_split_cu_binary_flag | ae(v) |
| if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_VER ) { |  |
| depthOffset += ( x0 + cbWidth – TileX[ CurrTileId ] > TileWidth[ CurrTileId ] ) ? 1 : 0 |  |
| x1 = x0 + ( cbWidth / 2 ) |  |
| multi_type_tree( x0, y0, cbWidth / 2, cbHeight, mttDepth + 1, depthOffset, 0, treeType ) |  |
| if( x1 – TileX[ CurrTileId ] < TileWidth[ CurrTileId ] ) |  |
| multi_type_tree( x1, y0, cbWidth / 2, cbHeightY, mttDepth + 1, depthOffset, 1, treeType ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_HOR ) { |  |
| depthOffset += ( y0 + cbHeight – TileY[ CurrTileId ] > TileHeight[ CurrTileId ] ) ? 1 : 0 |  |
| y1 = y0 + ( cbHeight / 2 ) |  |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 2, mttDepth + 1, depthOffset, 0, treeType ) |  |
| if( y1 – TileY[ CurrTileId ] < TileHeight[ CurrTileId ] ) |  |
| multi_type_tree( x0, y1, cbWidth, cbHeight / 2, mttDepth + 1, depthOffset, 1, treeType ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_TT_VER ) { |  |
| x1 = x0 + ( cbWidth / 4 ) |  |
| x2 = x0 + ( 3 * cbWidth / 4 ) |  |
| multi_type_tree( x0, y0, cbWidth / 4, cbHeight, mttDepth + 1, depthOffset, 0, treeType ) |  |
| multi_type_tree( x1, y0, cbWidth / 2, cbHeight, mttDepth + 1, depthOffset, 1, treeType ) |  |
| multi_type_tree( x2, y0, cbWidth / 4, cbHeight, mttDepth + 1, depthOffset, 2, treeType ) |  |
| } else { /* SPLIT_TT_HOR */ |  |
| y1 = y0 + ( cbHeight / 4 ) |  |
| y2 = y0 + ( 3 * cbHeight / 4 ) |  |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 4, mttDepth + 1, depthOffset, 0, treeType ) |  |
| multi_type_tree( x0, y1, cbWidth, cbHeight / 2, mttDepth + 1, depthOffset, 1, treeType ) |  |
| multi_type_tree( x0, y2, cbWidth, cbHeight / 4, mttDepth + 1, depthOffset, 2 , treeType) |  |
| } |  |
| } else |  |
| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

Semantics

    - Raw byte sequence payloads, trailing bits and byte alignment semantics

      - Picture parameter set RBSP semantics

tiles_enabled_flag equal to 1 specifies that there is more than one tile in each picture referring to the PPS. tiles_enabled_flag equal to 0 specifies that there is only one tile in each picture referring to the PPS.

It is a requirement of bitstream conformance that the value of tiles_enabled_flag shall be the same for all PPSs that are activated within a CVS.

tile_unit_size_idc specifies the size of a tile unit block in luma samples.

The variables TileUnitSizeY,  PicWidthInTileUnitsY and PicHeightInTileUnitsY are derived as follows;

TileUnitSizeY = Max (CtbSizeY >> ( tile_unit_size_idc ), 8 ) 	(7-14)

PicWidthInTileUnitsY = Ceil( pic_width_in_luma_samples ÷ TileUnitSizeY ) 	(7-14)

PicHeightInTileUnitsY = Ceil( pic_height_in_luma_samples ÷ TileUnitSizeY ) 	(7-14)

If tiles_enabled_flag is equal to 1, the variables PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY are modified as follows:

for(PicWidthInCtbsY = 0, i = 0; i <= num_tile_columns_minus1; i++ )

PicWidthInCtbsY += Ceil( colWidth[ i ] * TileUnitSizeY ÷ CtbSizeY )			(7-14)

for(PicHeightInCtbsY = 0, j = 0; j <= num_tile_rows_minus1; j++ )

PicHeightInCtbsY += Ceil( rowHeight[ j ] * TileUnitSizeY ÷ CtbSizeY ) 			(7-14)

PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY	(7-14)

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture in units of tile unit blocks. num_tile_columns_minus1 shall be in the range of 0 to PicWidthInTileUnitsY − 1, inclusive. When not present, the value of num_tile_columns_minus1 is inferred to be equal to 0.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture in units of tile unit blocks. num_tile_rows_minus1 shall be in the range of 0 to PicHeightInTileUnitsY − 1, inclusive. When not present, the value of num_tile_rows_minus1 is inferred to be equal to 0.

The variable NumTilesInPic is derived as follows:

NumTilesInPic = ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1)

When tiles_enabled_flag is equal to 1, num_tile_columns_minus1 and num_tile_rows_minus1 shall not be both equal to 0.

uniform_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture. uniform_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements column_width_minus1[ i ] and row_height_minus1[ i ]. When not present, the value of uniform_spacing_flag is inferred to be equal to 1.

column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs.

row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs.

The following variables are derived by invoking the CTB raster and tile scanning conversion process as specified in clause 6.5.1:

–	The list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the CTB raster scan of a picture to a CTB address in the tile scan,

–	the list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in the tile scan to a CTB address in the CTB raster scan of a picture,

–	the list TileId[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID,

Slice header semantics

General slice header semantics

slice_address specifies the address of the first CTB in the slice, in CTB raster scan of a picture. The length of the slice_address syntax element is Ceil( Log2( PicSizeInCtbsY ) ) bits. The value of slice_address shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive, and the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

The variable CtbAddrInRs, specifying a CTB address in CTB raster scan of a picture, is set equal to slice_address.

The variable CtbAddrInTs, specifying a CTB address in tile scan, is set equal to CtbAddrRsToTs[ CtbAddrInRs ].

The variable CurrTileId, specifying an ID of a tile, is set equal to TileId[ CtbAddrInRs ].

