| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0360-v2 |
| --- | --- |

| Title: | CE7: Adaptive residual scaling (CE7.2.1, CE7.2.2, CE7.2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the coding performance of adaptive residual scaling (ARS). Simulation results reportedly show that -3.74%, -3.56%, and -3.13% MSSSIM-Y BD-rates are achieved for AI, RA and LB configurations, respectively. Compared to CU-level delta QP signaling with MaxCuDeltaQpDepth = 2, ARS reportedly leads to around -0.58%, -1.10%, -1.73% in AI, RA and LB, respectively.

Proposed Method

Visual distortion of a block is influenced by the background of the block. Luminance masking and contrast masking effect of the background are considered in the quantizer design, in order to make a more perceptually uniform quantizer. The supporting background are two lines around the current CU.

A luminance factor fl(Ba) is derived based on the average luminance of the background Ba (normalized to 8-bit),

flBa=1+60-Ba150,       Ba∈0,60   1,                      Ba∈60,170       1+Ba-170425,    Ba∈170,255.

Ba = nLnN

where L(n) denotes a luminance pixel in the background, and N is the number of available background pixels. The division by N is handled by multiplication with a value from an aspect-ratio-related look-up table, followed by a right shift.

A contrast masking factor fcBc is derived based on the Mean Absolute Difference (MAD) between the background luminance and the average luminance (i.e., a way to measure the contrast Bc).

fcBc=1,            Bc≤5Bc+510,     Bc>5.

Bc = n|Ln-Ba|N

The scaler is based on the product of the two factors and is constrained in a range of [2/2,2]. The scaler is then applied to scale the quantization step size. For the flexibility of the quantizer, a slice-header parameter s is signaled to further adjust the scaler. At encoder, the parameter s is derived from the geometric mean of masking factors calculated from original luma samples of 16x16 blocks inside the slice, and is written into the slice header.

qs'= qsQP×scaler=qsQP×clipflBa×fcBc×s2, 0.5, 2.0

To avoid floating-point calculation, the factors and scaler are all integerized into a 9-bit integers, and the squared root operation is realized by a look-up table with 49 entries.

Scaling quantization step size is equivalent to scaling residual if quantization processing precision is high enough. The latter is friendlier to hardware pipeline, and thus the adaptive quantiztion is implemented as adaptive residual scaling. The residual scaler is derived based on the reconstructed luma samples around the current CU, and thus is the same in encoder and decoder.

In encoder, the residual scaling performs on the transform coefficients before quantization to maintain higher precision.

Further improvements based on comments from Ljubljana meeting include:

Support user specified masking curves in SPS

The luminance masking and contrast masking curves are supported to be signaled in SPS. The default luminance masking and contrast masking are set as predictors, and a delta value can be signaled to modify the sampling points on the masking curve (delta = idx * step, and the idx is signaled). A new masking curve is recalculated based on the modified sampling points (i.e., values on a curve between two sampling points are generated by linear interpolation). Figure 1 illustrates the default luminance/contrast masking curve, the sampling points on the curves, and an example of modified luminance masking curve.

Figure 1 Illustration of masking curve derivation

Enable ARS for chroma

An SPS flag is used to signal the ARS also applied to chroma residual.

ARS strength adjustment

An SPS index is used to signal the scaling of the ARS factors by different strength, i.e., magnifying or reducing the ARS influence.

ARS factors is adjusted by

f’ = (f - 1) * (1 + index * 0.25) + 1

where f the original ARS factor and f’ is the adjusted ARS factor; index value can be 0, 1 or 2.

Harmonize with QP in deblocking

The ARS factors are treated as fractional delta QP, and are added with the CU QP values in deblocking process.

In addition, to study the impact of granularity of the ARS factors, a test is conducted (CE7.2.2) in which a CU smaller than 256 pixels uses the ARS factor of its parent node that is closest to but no smaller than 256 pixels. This scheme is like using quantization group of 256 pixels for ARS.

An encoder QP signaling scheme has been implemented to show the gain from encoder only processing (CE7.2.3). For each CU width split depth <= MaxDQpCuDepth, a residual scaling factor is derived based on the original pixels of the CU and mapped into a delta QP. The delta QP is then signaled into the bitstream. For CE7.2.3, the MaxDQpCuDepth is set as 0, i.e., CTU level delta QP is allowed. In another test, MaxDQpCuDepth = 2 is used. More results can be found in JVET-L0362.

When signaling delta QP, the CU-level lambda is changed according to the QP of a CU, and the RD-cost comparison between a CU and sub-CUs are based on slice lambda. To make a fair comparison with CU delta QP scheme, in CE7.2.1 test, CU-level lambda is changed according to the derived ARS factors (by treating ARS as fractional QP, and changing lambda accordingly). In an additional test, the results of ARS without CU-level lambda change is provided.

Simulation results

The proposed method was integrated into the VTM 2.0.1 reference software, and are tested in VTM CTC configurations. Table 1 summarizes the simulation results of the tests. Detailed results can be found in the attached excel sheets. MsSSIM are used as quality metric in calculation of BD-rate.

Table 1 Simulation results of the two tests

| Tests |  | MsSSIM -Y | MsSSIM -U | MsSSIM -V | PSNR -Y | PSNR -U | PSNR -V | EncT | DecT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ARS w/o lambda Change (additional results, cross-checked by KDDI) | AI | -3.74% | 0.17% | 0.14% | 2.02% | 0.43% | 0.41% | 102% |  | 104% |
|  | RA | -3.56% | -0.30% | -0.34% | 2.17% | -0.07% | -0.10% | 100% |  | 102% |
|  | LB | -3.13% | -1.78% | -2.42% | 2.72% | -1.94% | -2.47% | 95% |  | 102% |
| - ARS w/ lambda Change (CE7.2.1) | AI | -5.24% | 0.23% | 0.12% | 2.54% | 0.55% | 0.47% | 103% |  | 103% |
|  | RA | -5.81% | 1.37% | 0.97% | 2.72% | 2.38% | 2.12% | 103% |  | 102% |
|  | LB | -5.99% | 4.87% | 3.90% | 3.48% | 6.13% | 5.22% | 99% |  | 102% |
| - ARS w/ lambda Change, for large blocks containing >= 256 pixels (CE7.2.2) | AI | -4.14% | -0.04% | -0.12% | 2.65% | 0.11% | 0.09% | 102% |  | 103% |
|  | RA | -4.77% | 1.23% | 0.97% | 2.85% | 2.23% | 1.96% | 103% |  | 102% |
|  | LB | -4.68% | 4.43% | 3.37% | 3.71% | 5.47% | 4.49% | 98% |  | 100% |
| - delta QP, MaxDQPCuDepth = 0 (CE7.2.3) | AI | -2.90% | -1.22% | -1.25% | 0.98% | -1.26% | -1.25% | 101% |  | 100% |
|  | RA | -2.91% | -0.21% | 0.14% | 1.01% | 0.51% | 0.97% | 104% |  | 100% |
|  | LB | -2.41% | 1.34% | 0.99% | 1.59% | 2.27% | 1.95% | 101% |  | 99% |
| - delta QP, MaxDQPCuDepth = 2 (additional results, cross-checked by KDDI) | AI | -4.66% | -1.97% | -2.09% | 3.01% | -1.94% | -1.98% | 100% |  | 103% |
|  | RA | -4.70% | 0.54% | 0.81% | 3.20% | 1.99% | 2.49% | 105% |  | 101% |
|  | LB | -4.25% | 4.17% | 3.90% | 4.29% | 6.12% | 5.69% | 101% |  | 101% |

It is shown that -3.74%, -3.56%, and -3.13% MsSSIM-Y BD-rates are achieved for AI, RA and LB configurations by ARS without lambda change, respectively. Compared to CU-level delta QP signaling with MaxCuDeltaQpDepth = 2 (Test 5 vs. Test 2), ARS reportedly leads to around -0.58%, -1.10%, -1.73% in AI, RA and LB, respectively.

In general, increasing MaxDQpCuDepth from 0 to 2 can strengthen the QP adaptation with small amount of bits, thus improving BD-rate. However, further enlarging the MaxDQpCuDepth (e.g., to 4, results provided in JVET-L0362) does not improve the MS-SSIM Y BD-rate, as the additional signaling cost eats up the gain of more accurate QP adaptation. With decoder side ARS derivation, additional gain is achieved beyond “quantization group” of 256 pixels, by comparing Test 2 and Test 3.

Conclusions

This contribution reports the performance of ARS and an encoder only counterpart, and -3.74%, -3.56%, and -3.13% MsSSIM-Y BD-rates are achieved for AI, RA and LB configurations by ARS.

It is recommended to adopt ARS into VTM and continue the study on perceptual quantization.

Acknowledgement

We would like to thank KDDI for cross-checking this proposal.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).