| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0368-v2 |
| --- | --- |

| Title: | CE4: Affine merge enhancement with simplification (Test 4.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes the following simplifications for the affine merge mode in JVET-L0366:

- The pruning process for inherited affine candidates is simplified by comparing the coding units covering the neighboring positions, instead of comparing the derived affine candidates in VTM-2.0.1. Up to 2 inherited affine candidates are inserted into affine merge list. The pruning process for constructed affine candidates is totally removed.

- The MV scaling operation in constructed affine candidate is removed. If the reference indices of control points are different, the constructed motion model is discarded.

- The number of constructed affine candidates is reduced from 10 to 6.

The proposed changes reduce the complexity of affine merge list construction without comprising coding efficiency. For all simplifications (test 4.2.2.e), simulation results reportedly show on average 0.66%/0.27% luma BD-rate gain with 7%/7% encoding time and 6%/3% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

When combined with line buffer reduction of affine inherited candidates (test 4.2.2.f), simulation results reportedly show on average 0.57%/0.25% luma BD-rate gain with 6%/6% encoding time and 6%/3% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

The combination of CE4.2.2.e and CE4.4.7.a (HPMV, History based motion vector prediction) is conducted, donated as test 4.2.2.h. Simulation results reportedly show on average 1.31%/0.62% luma BD-rate gain with 6%/7% encoding time and 6%/4% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

Introduction

In JVET-L0366 [1], the original affine merge mode in VTM-2.0.1 is enhanced by constructing an affine merge candidate list with 5 candidates. The affine merge candidate list is constructed by inserting 2 kinds of candidates: inherited affine candidates and constructed affine candidates.

In this contribution, following simplifications for the common based of affine merge mode are applied:

- The pruning process for inherited affine candidates is simplified by comparing the coding units covering the neighboring positions, instead of comparing the derived affine candidates in VTM-2.0.1. Up to 2 inherited affine candidates are inserted into affine merge list. The pruning process for constructed affine candidates is totally removed.

- The MV scaling operation in constructed affine candidate is removed. If the reference indices of control points are different, the constructed motion model is discarded.

- The number of constructed affine candidates is reduced from 10 to 6. The combinations of constructed affine candidates are inserted into to candidate list as following order:

{CP1, CP2, CP3}, {CP1, CP2, CP4}, {CP1, CP3, CP4}, {CP2, CP3, CP4}, {CP1, CP2}, {CP1, CP3}

Test specification

The tests for this contribution are descripted in Table 1.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| 4.2.2.a | Simplify affine merge list construction by reducing pruning operation |
| 4.2.2.b | Simplify affine merge list construction by reducing MV scaling operation |
| 4.2.2.c | Simplify affine merge list construction by reducing maximum number of candidates |
| 4.2.2.d | Reduce line buffer in affine merge mode |
| 4.2.2.e | 4.2.2.a + 4.2.2.b + 4.2.2.c |
| 4.2.2.f | 4.2.2.a + 4.2.2.b + 4.2.2.c + 4.2.2.d |
| 4.2.2.g | 4.2.2.a + 4.2.2.b + 4.2.2.c + 4.2.2.d + 4.1.3.c |
| 4.2.2.h | 4.2.2.e + 4.4.7.a (History-based Motion Vector Prediction) |

Simulation results

The contribution has been implemented on top of VTM-2.0.1. The test condition descripted in JVET-K1024 [2] is used for verifying the contribution.

The results are summarized in Table 2. The simulation results of test 4.2.1 are also provided as anchor. More detailed results are provided from Table 3 toTable 10.

Table 2 Summary of overall performance in RA/LB configuration (over VTM2.0.1)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.2.1 | -0.75% | -0.62% | -0.78% | 107% | 107% | -0.44% | -0.40% | -0.40% | 108% | 105% |
| 4.2.2.a | -0.73% | -0.66% | -0.75% | 107% | 107% | -0.45% | -0.51% | -0.44% | 108% | 105% |
| 4.2.2.b | -0.73% | -0.59% | -0.78% | 106% | 107% | -0.36% | -0.25% | -0.41% | 107% | 104% |
| 4.2.2.c | -0.70% | -0.54% | -0.76% | 106% | 107% | -0.42% | -0.39% | -0.39% | 108% | 105% |
| 4.2.2.d | -0.65% | -0.50% | -0.69% | 107% | 106% | -0.45% | -0.29% | -0.50% | 109% | 105% |
| 4.2.2.e | -0.66% | -0.52% | -0.69% | 107% | 106% | -0.27% | -0.30% | -0.22% | 107% | 103% |
| 4.2.2.f | -0.57% | -0.46% | -0.60% | 106% | 106% | -0.25% | -0.36% | -0.33% | 106% | 103% |
| 4.2.2.g | -0.56% | -0.38% | -0.58% | 106% | 106% | -0.24% | -0.19% | -0.44% | 107% | 103% |
| 4.2.2.h | -1.31% | -1.35% | -1.56% | 106% | 106% | -0.62% | -0.67% | -0.93% | 107% | 104% |

Test a

Table 3 Coding performance of test 4.2.2.a over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.72% | -0.91% | 106% | 109% |
| Class A2 | -0.96% | -1.00% | -1.14% | 107% | 108% |
| Class B | -0.66% | -0.57% | -0.64% | 107% | 106% |
| Class C | -0.46% | -0.47% | -0.47% | 107% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.66% | -0.75% | 107% | 107% |
| Class D | -0.64% | -0.62% | -0.56% | 108% | 111% |
| Class F (optional) | -0.12% | -0.14% | -0.14% | 105% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -0.36% | -0.31% | 109% | 104% |
| Class C | -0.33% | -0.37% | -0.40% | 108% | 104% |
| Class E | -0.74% | -0.93% | -0.72% | 109% | 106% |
| Overall | -0.45% | -0.51% | -0.44% | 108% | 105% |
| Class D | -0.58% | -0.30% | -0.55% | 107% | 107% |
| Class F (optional) | -0.13% | 0.39% | -0.26% | 106% | 101% |

Test b

Table 4 Coding performance of test 4.2.2.b over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.65% | -0.87% | 105% | 108% |
| Class A2 | -0.99% | -1.01% | -1.16% | 107% | 108% |
| Class B | -0.66% | -0.53% | -0.74% | 106% | 106% |
| Class C | -0.46% | -0.32% | -0.46% | 107% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.59% | -0.78% | 106% | 107% |
| Class D | -0.58% | -0.58% | -0.65% | 108% | 111% |
| Class F (optional) | -0.13% | -0.16% | -0.12% | 105% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.27% | -0.39% | 108% | 104% |
| Class C | -0.24% | -0.23% | -0.42% | 106% | 103% |
| Class E | -0.54% | -0.23% | -0.42% | 108% | 105% |
| Overall | -0.36% | -0.25% | -0.41% | 107% | 104% |
| Class D | -0.29% | -0.03% | -0.55% | 106% | 107% |
| Class F (optional) | -0.09% | 0.12% | 0.23% | 105% | 100% |

Test c

Table 5 Coding performance of test 4.2.2.c over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | -0.51% | -0.86% | 105% | 108% |
| Class A2 | -0.94% | -0.94% | -1.16% | 107% | 108% |
| Class B | -0.63% | -0.45% | -0.66% | 106% | 107% |
| Class C | -0.44% | -0.38% | -0.50% | 106% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.54% | -0.76% | 106% | 107% |
| Class D | -0.59% | -0.48% | -0.71% | 107% | 112% |
| Class F (optional) | -0.12% | -0.16% | -0.14% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.37% | -0.13% | 109% | 105% |
| Class C | -0.34% | -0.27% | -0.37% | 107% | 105% |
| Class E | -0.65% | -0.58% | -0.86% | 109% | 106% |
| Overall | -0.42% | -0.39% | -0.39% | 108% | 105% |
| Class D | -0.58% | 0.12% | -0.54% | 107% | 107% |
| Class F (optional) | -0.06% | 0.11% | 0.18% | 106% | 102% |

Test d

Table 6 Coding performance of test 4.2.2.d over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.89% | -0.53% | -0.85% | 106% | 107% |
| Class A2 | -0.74% | -0.71% | -0.92% | 108% | 107% |
| Class B | -0.58% | -0.50% | -0.66% | 107% | 106% |
| Class C | -0.48% | -0.31% | -0.43% | 107% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.65% | -0.50% | -0.69% | 107% | 106% |
| Class D | -0.65% | -0.61% | -0.78% | 108% | 111% |
| Class F (optional) | -0.08% | -0.10% | -0.12% | 105% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.46% | -0.60% | 110% | 104% |
| Class C | -0.33% | -0.06% | -0.16% | 107% | 105% |
| Class E | -0.75% | -0.33% | -0.81% | 109% | 106% |
| Overall | -0.45% | -0.29% | -0.50% | 109% | 105% |
| Class D | -0.53% | 0.35% | -0.52% | 107% | 107% |
| Class F (optional) | -0.05% | 0.14% | -0.08% | 106% | 101% |

Test e

Table 7 Coding performance of test 4.2.2.e over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.90% | -0.48% | -0.75% | 106% | 107% |
| Class A2 | -0.90% | -0.94% | -1.06% | 108% | 107% |
| Class B | -0.59% | -0.43% | -0.57% | 107% | 105% |
| Class C | -0.38% | -0.36% | -0.51% | 107% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.66% | -0.52% | -0.69% | 107% | 106% |
| Class D | -0.48% | -0.50% | -0.57% | 107% | 109% |
| Class F (optional) | -0.11% | -0.06% | -0.12% | 105% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.33% | -0.27% | 107% | 103% |
| Class C | -0.16% | -0.29% | -0.06% | 106% | 103% |
| Class E | -0.42% | -0.28% | -0.34% | 108% | 104% |
| Overall | -0.27% | -0.30% | -0.22% | 107% | 103% |
| Class D | -0.21% | -0.08% | -0.71% | 106% | 106% |
| Class F (optional) | -0.05% | 0.31% | 0.21% | 105% | 100% |

Test f

Table 8 Coding performance of test 4.2.2.f over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.48% | -0.74% | 106% | 107% |
| Class A2 | -0.66% | -0.73% | -0.84% | 107% | 107% |
| Class B | -0.54% | -0.44% | -0.48% | 106% | 105% |
| Class C | -0.36% | -0.26% | -0.45% | 106% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -0.46% | -0.60% | 106% | 106% |
| Class D | -0.46% | -0.32% | -0.61% | 107% | 110% |
| Class F (optional) | -0.05% | -0.10% | -0.13% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | -0.25% | -0.22% | 107% | 103% |
| Class C | -0.13% | -0.34% | -0.50% | 105% | 103% |
| Class E | -0.40% | -0.57% | -0.29% | 107% | 104% |
| Overall | -0.25% | -0.36% | -0.33% | 106% | 103% |
| Class D | -0.20% | 0.49% | -0.04% | 105% | 106% |
| Class F (optional) | 0.09% | 0.33% | 0.43% | 104% | 100% |

Test g

Table 9 Coding performance of test 4.2.2.g over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.38% | -0.72% | 105% | 107% |
| Class A2 | -0.68% | -0.66% | -0.81% | 106% | 107% |
| Class B | -0.49% | -0.42% | -0.56% | 106% | 105% |
| Class C | -0.37% | -0.11% | -0.35% | 106% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.56% | -0.38% | -0.58% | 106% | 106% |
| Class D | -0.45% | -0.30% | -0.48% | 107% | 110% |
| Class F (optional) | -0.02% | -0.10% | -0.06% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | -0.21% | -0.55% | 107% | 103% |
| Class C | -0.12% | -0.19% | -0.28% | 106% | 102% |
| Class E | -0.39% | -0.17% | -0.48% | 107% | 103% |
| Overall | -0.24% | -0.19% | -0.44% | 107% | 103% |
| Class D | -0.17% | 0.40% | 0.27% | 105% | 106% |
| Class F (optional) | 0.12% | 0.39% | 0.17% | 104% | 100% |

Test h

Table 10 Coding performance of test 4.2.2.h over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.31% | -1.09% | -1.57% | 106% | 108% |
| Class A2 | -1.56% | -1.92% | -2.01% | 107% | 107% |
| Class B | -1.52% | -1.50% | -1.60% | 106% | 106% |
| Class C | -0.88% | -0.91% | -1.16% | 106% | 105% |
| Class E |  |  |  |  |  |
| Overall | -1.31% | -1.35% | -1.56% | 106% | 106% |
| Class D | -0.74% | -0.96% | -0.94% | 108% | 109% |
| Class F (optional) | -0.63% | -0.77% | -0.78% | 104% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.95% | -0.97% | -1.24% | 107% | 104% |
| Class C | -0.40% | -0.61% | -0.66% | 107% | 103% |
| Class E | -0.35% | -0.27% | -0.77% | 108% | 103% |
| Overall | -0.62% | -0.67% | -0.93% | 107% | 104% |
| Class D | -0.30% | -0.09% | -0.46% | 106% | 106% |
| Class F (optional) | -0.73% | -1.00% | -0.54% | 105% | 102% |

Conclusions

This contribution proposes the several simplifications for the affine merge mode in JVET-L0366. The proposed changes reduce the complexity of affine merge list construction without comprising coding efficiency. For all simplifications (test 4.2.2.e), simulation results reportedly show on average 0.66%/0.27% luma BD-rate gain with 7%/7% encoding time and 6%/3% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

When combined with line buffer reduction of affine inherited candidates (test 4.2.2.f), simulation results reportedly show on average 0.57%/0.25% luma BD-rate gain with 6%/6% encoding time and 6%/3% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

The combination of CE4.2.2.e and CE4.4.7.a (HPMV, History based motion vector prediction) is conducted, donated as test 4.2.2.h. Simulation results reportedly show on average 1.31%/0.62% luma BD-rate gain with 6%/7% encoding time and 6%/4% decoding time increase in RA/LB configurations, compared to VTM-2.0.1.

References

- H. Chen, H. Yang, J. Chen, “CE4: Common base for affine merge mode (Test 4.2.1)”, JVET-L0366, the 12th JVET meeting, Oct. 2018, Macao, CN.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).