| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0370-v1 |
| --- | --- |

| Title: | CE4: Symmetrical MVD mode (Test 4.5.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of integrating symmetrical mode for bi-prediction over VTM-2.0.1. The proposed mode derives the MVD for the list 1 from list 0 based on the assumption of linear motion. Simulation results reportedly shows on average 0.58% BD-rate saving with 4% encoding time for RA configuration over VTM-2.0.1. When combined with MVD representation in JVET-K0115, results shows on average 0.46% BD-rate saving with 2% encoding time for RA configuration over VTM-2.0.1.

Introduction

In this contribution, a mode is proposed for motion information coding in bi-prediction.

Firstly, in slice level, variables BiDirPredFlag, RefIdxSymL0 and RefIdxSymL1 are derived as follows:

- The forward reference picture in reference picture list 0 which is nearest to current picture is searched. If found, RefIdxSymL0 is set equal to the reference index of the forward picture.

- The backward reference picture in reference picture list 1 which is nearest to current picture is searched. If found, RefIdxSymL1 is set equal to the reference index of the backward picture.

- If both forward and backward picture are found, BiDirPredFlag is set equal to 1.

- Otherwise, following applies:

- The backward reference picture picture in reference picture list 0 which is nearest to current is searched. If found, RefIdxSymL0 is set equal to the reference index of the backward picture.

- The forward reference picture in reference picture list 1 which is nearest to current picture is searched. If found, RefIdxSymL1 is set equal to the reference index of the forward picture.

- If both backward and forward picture are found, BiDirPredFlag is set equal to 1. Otherwise, BiDirPredFlag is set equal to 0.

Secondly, in CU level, a symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction for the CU is bi-prediction and BiDirPredFlag is equal to 1.

When the flag is true, only mvp_l0_flag, mvp_l1_flag and MVD0 are explicitly signaled. The reference indices are set equal to RefIdxSymL0, RefIdxSymL1 for list 0 and list 1, respectively. MVD1 is just set equal to –MVD0. The final motion vectors are shown in below formula.

mvx0, mvy0=(mvpx0+mvdx0,mvpy0+mvdy0) mvx1, mvy1=(mvpx1-mvdx0,mvpy1-mvdy0)

Figure 1 Illustration for symmetrical mode

The modifications in coding unit syntax are shown in Table 1.

Table 1 Modifications in coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_BI && BiDirPredFlag && inter_affine_flag[ x0 ][ y0 ] == 0 ) |  |
| symmetric_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 && !symmetric_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 && !symmetric_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| … |  |
| } else { |  |
| if( !symmetric_mvd_flag[ x0 ][ y0 ] ) |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) |  |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

Test specification

The tests for this contribution are descripted in Table 2.

Table 2 test specification for this contribution

| Test # | Test description |
| --- | --- |
| 4.5.1.a | Symmetrical MVD mode |
| 4.5.2.b | Symmetrical MVD combined with MVD representation in JVET-K0115 |

Simulation results

The contribution has been implemented on top of VTM-2.0.1. The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The results are summarized in Table 3. More detailed results are provided from Table 4 to Table 5.

For test 4.5.1.a, results reportedly shows on average 0.58% BD-rate saving with 4% encoding time for RA configuration over VTM-2.0.1. When combined with MVD representation in JVET-K0115 [2], results shows on average 0.46% BD-rate saving with 2% encoding time for RA configuration over VTM-2.0.1.

Table 3 Summary of overall performance in RA configuration (over VTM-2.0.1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 4.5.1.a | -0.58% | -0.42% | -0.44% | 104% | 99% |
| 4.5.1.b | -0.46% | -0.30% | -0.33% | 102% | 99% |

Test a

Table 4 Coding performance of test 4.5.1.a over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | -0.53% | -0.57% | 103% | 99% |
| Class A2 | -0.74% | -0.67% | -0.62% | 104% | 99% |
| Class B | -0.58% | -0.38% | -0.39% | 104% | 99% |
| Class C | -0.37% | -0.19% | -0.28% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -0.42% | -0.44% | 104% | 99% |
| Class D | -0.37% | -0.25% | -0.26% | 105% | 99% |
| Class F (optional) | -0.26% | -0.23% | -0.31% | 103% | 99% |

Test b

Table 5 Coding performance of test 4.5.1.b over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.37% | -0.42% | 101% | 100% |
| Class A2 | -0.65% | -0.54% | -0.56% | 103% | 99% |
| Class B | -0.47% | -0.28% | -0.31% | 102% | 99% |
| Class C | -0.27% | -0.09% | -0.12% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -0.30% | -0.33% | 102% | 99% |
| Class D | -0.31% | -0.27% | -0.24% | 103% | 100% |
| Class F (optional) | -0.18% | -0.19% | -0.21% | 102% | 100% |

Conclusions

This contribution reports the results of integrating symmetrical mode for bi-prediction over VTM-2.0.1. The proposed mode derives the MVD for the list 1 from list 0 based on the assumption of linear motion. Simulation results reportedly shows on average 0.58% BD-rate saving with 4% encoding time for RA configuration over VTM-2.0.1. When combined with MVD representation in JVET-K0115, results shows on average 0.46% BD-rate saving with 2% encoding time for RA configuration over VTM-2.0.1.

References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

- S. Jeong, M. W. Park, C. Kim, “CE4 Ultimate motion vector expression in JVET-J0024 (Test 4.2.9)”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).