| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0375-v1 |
| --- | --- |

| Title: | CE10-related: inter prediction sample filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiwei Xu Haitao Yang Yin Zhao Jianle Chen | Tel: Email: | +8613758193753 xuweiwei3@huawei.com haitao.yang@huawei.com yin.zhao@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of inter prediction samples filtering. A spatial filter is applied on the prediction samples from merge/skip mode. A CU-level flag is added to indicate whether using inter prediction samples filtering. Compared to VTM2.0.1 anchor, the average BD-rate is -0.29%~-0.48% in RA with different filter.

Introduction

After the inter prediction samples is obtained in MC, the prediction samples is filtered using the spatial neighboring samples.

Filter with 2 spatial neighboring samples

Inter prediction samples filtering can use spatial neighboring samples only in the left and top neighbor as follows.

P(x,y) = (W1*L(-1,y) + W2*A(x,-1) + W3*pred(x,y) + 4) >> 3

W1, W2 and W3 denote the weighting factors, and W1, W2 and W3 are set as 1, 1, and 6, respectively. (x,y) is the coordinates of current prediction samples relative to the above left corner sample.is the filtered prediction sample, L is the hor neighboring samples, A is the ver neighboring samples and Pred(x,y) is the original prediction samples. A CU-level flag is added to indicate whether using inter prediction samples filtering.

Figure 1. Inter prediction sample filtering

Filter with 2 spatial neighboring samples couples

Inter prediction samples filtering can also be performed by adding additional neighboring pixels

P(x,y) = (W1*L(-1,y) + W2*L(-2,y) + W3*A(x,-1) + W4*A(x,-2) + W5*pred(x,y) + 4) >> 3

W1, W2, W3, W4 and W5 denote the weighting factors, and W1, W2, W3, W4 and W5 are set as 1, 1, 1, 1, and 12, respectively. (x,y) is the coordinates of current prediction samples relative to the above left corner sample.is the filtered prediction sample, L is the hor neighboring samples, A is the ver neighboring samples and Pred(x,y) is the original prediction samples. A CU-level flag is added to indicate whether using inter prediction samples filtering.

Filter with PLANAR pattern

Inter prediction samples filtering can also be performed by using intra planar pattern weighting procedure to generate the hor and ver samples. A CU-level flag is added to indicate whether using inter prediction samples filtering.

Simulation results

This testing results is obtained by running BMS2.0.1 with VTM cfg file, including anchor of VTM and test. The contribution has been implemented on top of BMS2.0.1. The simulations were performed following the JVET common test conditions (CTC) [1].

Filter with 2 spatial neighboring samples

The summary results are provided in Table 1. On average, the BD-rate differences are -0.30% in RA when W1, W2 and W3 are set as 1, 1, and 6, respectively.

Table 1 Coding performance over VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | 0.03% | -0.19% | 105% | 100% |
| Class A2 | -0.28% | -0.22% | -0.39% | 106% | 100% |
| Class B | -0.34% | -0.19% | -0.19% | 105% | 100% |
| Class C | -0.27% | -0.07% | -0.23% | 106% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.12% | -0.24% | 105% | 100% |
| Class D | -0.12% | -0.19% | -0.37% | 106% | 100% |
| Class F (optional) | -0.32% | -0.25% | -0.27% | 104% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.26% | -0.34% | -0.55% | 107% | 101% |
| Class E | -0.30% | -0.61% | -0.73% | 105% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.21% | -0.49% | -1.06% | 107% | 104% |
| Class F (optional) | -0.60% | -0.29% | -0.65% | 106% | 101% |

Filter with 2 spatial neighboring samples couples

When using four neighboring candidates

P(x,y) = (W1*L(-1,y) + W2*L(-2,y) + W3*A(x,-1) + W4*A(x,-2) + W5*pred(x,y) + 4) >> 3

, the summary results are provided in Table 2. On average, the BD-rate differences are -0.29% in RA when W1, W2, W3, W4 and W5 are set as 1, 1, 1, 1, and 12, respectively. Which means

Table 2 Coding performance over VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.10% | -0.17% | 105% | 100% |
| Class A2 | -0.25% | -0.19% | -0.38% | 106% | 100% |
| Class B | -0.35% | -0.23% | -0.17% | 105% | 100% |
| Class C | -0.27% | -0.09% | -0.32% | 106% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.16% | -0.25% | 106% | 100% |
| Class D | -0.17% | -0.19% | -0.29% | 106% | 100% |
| Class F (optional) | -0.33% | -0.23% | -0.28% | 105% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.56% | -0.67% | 106% | 100% |
| Class C | -0.26% | -0.38% | -0.56% | 107% | 99% |
| Class E | -0.27% | -0.12% | -0.49% | 105% | 100% |
| Overall | -0.31% | -0.39% | -0.59% | 106% | 100% |
| Class D | -0.19% | -0.54% | -0.99% | 107% | 101% |
| Class F (optional) | -0.71% | -0.70% | -0.45% | 106% | 99% |

Filter with PLANAR pattern

When using Planar pattern, the performance can be improved. The summary results are provided in Table 3. On average, the BD-rate differences are -0.48% in RA.

Table 3 Coding performance over VTM2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.02% | -0.30% | 106% | 102% |
| Class A2 | -0.41% | -0.25% | -0.54% | 107% | 101% |
| Class B | -0.54% | -0.29% | -0.23% | 107% | 102% |
| Class C | -0.44% | -0.16% | -0.41% | 107% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.48% | -0.19% | -0.35% | 107% | 102% |
| Class D | -0.27% | -0.16% | -0.14% | 106% | 101% |
| Class F (optional) | -0.27% | -0.15% | -0.16% | 106% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |

Conclusions

This contribution reports the results of inter prediction sample filtering. Compared to VTM anchor on BMS 2.0.1 with VTM cfg, the average BD-rate differences are-0.29% in RA when W1, W2 and W3 are set as 1, 1, and 6, respectively. When using Planar pattern weighting, the average BD-rate differences can be -0.48% in RA.

We recommend to study the proposed method in CE.

References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).