| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0386-v4 |
| --- | --- |

| Title: | CE6.1.6: Efficient Implementations of AMT with Transform Adjustment Filters (TAF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of CE6.1.6. A unified transformation approach is proposed to approximate the DCT-8 and DST-7 used as primary transforms in VTM-2.0.1, where each transform is approximated with an adjustment filter as pre-processing and inverse of DCT-2. The results show approximately 2% and 5% reduction in encoding and decoding time against VTM-2.0.1 anchor and less than 0.05% loss in BD rate.

An additional test is presented where the adjustment filtering is applied on 32x32 DCT-4 resulted from a butterfly-based factorization of 64x64 DCT-2. Less than 0.05% loss in BD rate is observed in the experimental results against VTM-2.0.1 anchor.

# Introduction

In VTM 2.0.1, pairs of DCT-2, DST-7 or DCT-8 are used for separable transformation of residual block, where DCT-2 is used up to size 64x64 and DST-7 and DCT-8 are allowed up to size 32x32. Supporting multiple transforms has several consequences related to complexity and computational costs. For instance, not all transforms can be implemented efficiently for all dimensions, and they may require different types of hardware designs. Specifically, DCT-2 has efficient hardware implementations (partial-butterfly structures, etc.), so that it is feasible to use DCT-2 for large blocks. However, DST-7 and DCT-8 do not have such efficient implementations and supporting DCT-8 and DST-7 for large sizes (e.g., 64x64) may be quite costly. This proposal addresses these problems by using a unified (i.e., a single type of) transform implementation and additional processing filters, called transform adjustment filters (TAFs) of very low complexity, with practically no loss in coding performance.

An additional test applying transform adjustment filters in sub part of 64x64 DCT-2 is presented. In partial-butterfly structure, an 64x64 DCT-2 can be factorized into one 32x32 DCT-4, which is implemented with matrix multiplication and one 32x32 DCT-2, which can be further factorized into smaller size DCT-2 and DCT-4. The large size multiplier such as for 32x32 matrix multiplication can be costly in hardware implementation.  To address this issue, we apply adjustment filter on the 32x32 DCT-4, which approximate DCT-4 with 32x32 DCT2 and an additional processing filter of low complexity. The method results in almost no loss in coding performance. Along with the adjustment filters in DCT-8 and DST-7, the maximum size multiplier needed in the hardware implementation is reduced to size 16.

# CE Description: Proposed method

In this core experiment, the contribution proposed in JVET-J0066 is implemented and tested. The proposed transform method approximates different types of DCT and DSTs in VTM (DST-7, DCT-8) by applying adjustment filters to a transform in the DCT-2 family (DCT-2, DCT-3, DST-2, and DST-3), as shown in Fig. 1.

The adjustment filters are defined by sparse block-band orthogonal matrices. In this contribution, “6-taps” sparse block-band matrices are considered, i.e., there are only 6 non-zero elements in each row of the adjustment matrix. The adjustment is a pre-processing filter in the forward AMT transform (as shown in Fig. 1), and its inverse is a post-processing filter in the inverse AMT transform.

Figure 1: Adjustment filters to DCT-2 family

The proposed technique only employs DCT-2, which is by far the most optimized transform among different DST/DCT types, and sparse block-band matrices have very little computational and memory complexity over DCT-2. So, the overall complexity is significantly lower than using full DCT-8  and DST-7 transforms as in VTM-2.0.1. Moreover, the transform implementation is unified by means of using only DCT-2 with simple pre- and post-processing operations using sparse matrices.

The DCT-2 transform in HEVC is implemented with the partial-butterfly structure, which is also employed in the implementation of DCT-2 family in this contribution. Besides, in the submitted software, only the elements needed for partial butterfly computation are stored. The memory needed is approximately 1/3 of the memory size needed to store full DCT-2 matrix.

In partial-butterfly implementation, an NxN DCT-2 can be factorized into one N/2 x N/2 DCT-2 (which can be further factorized with butterfly structure) and one N/2 x N/2 DCT-4, which can be implemented with matrix multiplication. In this proposal, an additional test on approximating DCT-4 with transform adjustment filters is presented. The method approximates 32x32 DCT-4 from 64x64 DCT-2 factorization with an additional processing filter and a 32x32 DCT-2, which can be further factorized using butterfly structure. Along with DCT-8 and DST-7 approximation, it eliminates the need in implementing large multiplier such as of size 32 and the storage of 32x32 transform coefficients for DCT-4.

# Complexity Metrics

## Operation counts for individual transforms

The operation counts for 1D forward/inverse transforms in this CE are shown in Table 1.

The operation counts include the number of operations needed for implementing DCT-2 family and adjustment filters (multiplication of sparse band matrices).

Table 1: Operation counts for 1D forward/inverse transforms

|  | Number of Multiplications |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 8 | 64 | 182 | 534 | 1750 |
| DCT8 | 8 | 64 | 182 | 534 | 1750 |
| DCT2 | 8 | 24 | 86 | 342 | 683 |

|  | Number of Additions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 15 | 65 | 210 | 594 | 1842 |
| DCT8 | 15 | 65 | 210 | 594 | 1842 |
| DCT2 | 13 | 37 | 113 | 401 | 807 |

|  | Number of Shifts |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 5 | 9 | 34 | 66 | 98 |
| DCT8 | 5 | 9 | 34 | 66 | 98 |
| DCT2 | 5 | 9 | 17 | 33 | 33 |

## Memory requirements for storing transform cores

The number of elements and bits needed for transform are shown in Table 2.

Table 2: Number of elements and bits for each transform matrix

|  | Number of elements in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 16 | 64 | 96 | 192 | 384 | 752 |
| DCT8 | 16 | 64 | 0 | 0 | 0 | 80 |
| DCT2 | 16 | 64 | 86 | 342 | 1366 | 1874 |

|  | Number of bits in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 144 | 576 | 864 | 1728 | 3456 | 6768 |
| DCT8 | 144 | 576 | 0 | 0 | 0 | 720 |
| DCT2 | 160 | 640 | 860 | 3420 | 13660 | 18740 |

Note that the band matrix associated with DCT-8 is a reflection of the band matrix of DCT-7, and therefore no additional memory is needed to store its band matrices.

## Additional Complexity Related Information

## The bit-precision required by the transforms: The same precision in the current BMS/VTM is used.

## Specify if a transform requires multiple iterations: No additional iteration is required from BMS.

## Encoder and decoder times: Listed in the experimental results in Section 3.

## List of all combinations of transform and transforms block sizes used for primary transformation: In this CE, two transform size configurations are tested:

- The same transform sizes allowed in VTM-2.0.1 are implemented and tested. In other words, proposed method is used to implement DCT-8 and DST-7 with sizes 16x16 and 32x32.

- In addition, DCT-8 and DST-7 of size 64x64 are implemented and tested using the proposed method.

## Analysis of implementation and arithmetic commonalities of proposed transforms

The pre-processing and post processing with adjustment filters in encoder and decoder are implemented with transform multiplication (only non-zero elements are involved)

For DCT-2, the transform is implemented with partial butterflies as in BMS.

For other types of DCT/DSTs, partial butterflies are used for DCT-2 family applied for block size 16x16, 32x32, and 64x64, while for block size 8x8, matrix multiplication is used. Same as for BMS, 4x4 transforms for DST-1, DST-7, and DCT-8 are implemented with partial butterflies and 4x4 DCT-5 is implemented with matrix multiplication. In Table 3, the implementation types described for each transform are summarized.

Table 3: Analysis of implementation (PB: partial butterflies, MM: matrix multiplication)

| Size | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- |
| DST7 |  | PB | MM | PB | PB | PB |
| DCT8 |  | PB | MM | PB | PB | PB |
| DCT2 | PB | PB | PB | PB | PB | PB |

# Experimental Results

The proposed method is implemented based on BMS-2.0.1 software and tested under CTC [3] (i.e., VTM 2.0.1 configurations). In addition, it is tested by enabling inter MTS (Inter AMT) via EMT=3 and EMTFast =3 on top of CTC (VTM 2.0.1 + EMT=3). Table 4 presents results of proposed method used to implement DCT-8 and DST-7 for 16x16 and 32x32. In addition, Table 5 presents results for allowing 64x64 DCT-8 and DST-7 in addition to 16x16 and 32x32.

Table 4: Results of the proposed method used to implement DST-7 and DCT-8 for sizes 16x16 and 32x32.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test: CE6.1.6-a Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6-a + EMT=3 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.06% | 97% | 87% | 0.02% | 0.05% | 0.06% | 98% | 86% |
| Class A2 | 0.04% | 0.04% | -0.03% | 98% | 91% | 0.04% | 0.04% | -0.03% | 99% | 90% |
| Class B | 0.05% | 0.08% | 0.07% | 98% | 90% | 0.05% | 0.08% | 0.07% | 98% | 90% |
| Class C | 0.01% | -0.03% | 0.15% | 99% | 94% | 0.01% | -0.03% | 0.15% | 99% | 95% |
| Class E | 0.04% | -0.09% | 0.02% | 98% | 92% | 0.04% | -0.09% | 0.02% | 98% | 90% |
| Overall | 0.03% | 0.01% | 0.06% | 98% | 91% | 0.03% | 0.01% | 0.06% | 98% | 90% |
| Class D | -0.01% | 0.42% | -0.07% | 99% | 94% | -0.01% | 0.42% | -0.07% | 99% | 95% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: CE6.1.6-a Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6-a + EMT=3 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.05% | 0.11% | 99% | 97% | -0.01% | -0.01% | -0.03% | 97% | 96% |
| Class A2 | 0.04% | -0.02% | 0.08% | 99% | 99% | 0.02% | -0.02% | 0.07% | 98% | 98% |
| Class B | 0.04% | -0.01% | 0.07% | 100% | 99% | 0.02% | -0.14% | -0.02% | 96% | 95% |
| Class C | 0.02% | 0.08% | 0.11% | 98% | 97% | 0.03% | 0.02% | 0.11% | 97% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.09% | 99% | 98% | 0.01% | -0.05% | 0.03% | 97% | 96% |
| Class D | 0.02% | -0.17% | 0.07% | 99% | 97% | -0.01% | -0.13% | 0.15% | 97% | 96% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: CE6.1.6-a Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6-a + EMT=3 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.02% | 0.19% | 100% | 102% | -0.02% | -0.27% | 0.37% | 96% | 93% |
| Class C | 0.02% | 0.00% | -0.05% | 99% | 97% | 0.02% | -0.08% | 0.12% | 97% | 95% |
| Class E | 0.02% | -0.25% | 0.08% | 100% | 98% | -0.03% | -0.27% | 1.08% | 97% | 96% |
| Overall | 0.02% | -0.07% | 0.08% | 100% | 99% | -0.01% | -0.21% | 0.46% | 97% | 95% |
| Class D | 0.06% | 0.61% | -0.29% | 99% | 98% | -0.08% | 0.25% | 0.07% | 98% | 99% |

Simulation results in Table 4 show less than 0.05% loss in luma BD-rate with around 2% and 5% reduction for encoding and decoding time in VTM-2.0.1 configuration.

Table 5: Results of the proposed method used to implement DST-7 and DCT-8 for sizes 16x16, 32x32 and 64x64.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test: CE6.1.6-c Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6-c + EMT=3 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.27% | -0.28% | 99% | 88% | -0.14% | -0.27% | -0.28% | 98% | 86% |
| Class A2 | -0.04% | -0.09% | -0.12% | 98% | 90% | -0.04% | -0.09% | -0.12% | 98% | 89% |
| Class B | 0.02% | 0.02% | -0.01% | 98% | 89% | 0.02% | 0.02% | -0.01% | 98% | 88% |
| Class C | 0.00% | -0.04% | 0.16% | 99% | 95% | 0.00% | -0.04% | 0.16% | 99% | 96% |
| Class E | 0.03% | 0.02% | -0.15% | 99% | 92% | 0.03% | 0.02% | -0.15% | 98% | 92% |
| Overall | -0.02% | -0.06% | -0.06% | 98% | 91% | -0.02% | -0.06% | -0.06% | 98% | 90% |
| Class D | -0.02% | 0.36% | -0.05% | 98% | 95% | -0.02% | 0.36% | -0.05% | 99% | 94% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: CE6.1.6-c Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6-c + EMT=3 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.85% | -0.62% | 103% | 98% | -0.39% | -0.67% | -0.39% | 108% | 96% |
| Class A2 | -0.04% | -0.20% | -0.18% | 101% | 98% | -0.13% | -0.15% | -0.05% | 105% | 97% |
| Class B | -0.02% | -0.21% | -0.21% | 102% | 100% | -0.12% | -0.19% | -0.34% | 104% | 95% |
| Class C | 0.00% | 0.08% | 0.01% | 99% | 97% | 0.00% | 0.14% | 0.06% | 103% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.26% | -0.23% | 101% | 98% | -0.14% | -0.19% | -0.19% | 104% | 96% |
| Class D | 0.03% | -0.34% | 0.03% | 100% | 100% | -0.01% | -0.10% | 0.09% | 101% | 97% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: CE6.1.6-c Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6-c + EMT=3 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.03% | -0.31% | -0.33% | 102% | 102% | -0.21% | -0.16% | -0.34% | 105% | 92% |
| Class C | 0.01% | -0.07% | -0.02% | 101% | 100% | -0.07% | 0.24% | 0.04% | 102% | 98% |
| Class E | -0.16% | -0.32% | -1.29% | 100% | 101% | -0.28% | -0.15% | -0.89% | 106% | 97% |
| Overall | -0.05% | -0.23% | -0.47% | 101% | 101% | -0.18% | -0.03% | -0.35% | 104% | 95% |
| Class D | 0.06% | -0.03% | -0.27% | 99% | 97% | -0.11% | 0.05% | -0.34% | 101% | 98% |

# Additional CE Results

The results combining TAF approximation of DCT-8 and DST-7 up to size 32x32 and TAF approximation of 32x32 DCT-4 in DCT-2 implementation are presented in Table 6.

Table 6: Results of the proposed method used to implement DST-7 and DCT-8 for sizes 16x16, 32x32 and 32x32 DCT-4 in 64x64 DCT-2 implementation.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test: CE6.1.6 + TAF in DCT-4 Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6 + EMT=3 + TAF in DCT-4 Over VTM-2.0.1 + EMT=3 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.09% | 98% | 87% | 0.02% | 0.00% | 0.09% | 98% | 86% |
| Class A2 | 0.05% | 0.10% | 0.01% | 98% | 92% | 0.05% | 0.10% | 0.01% | 98% | 90% |
| Class B | 0.04% | 0.04% | 0.11% | 98% | 89% | 0.04% | 0.04% | 0.11% | 98% | 90% |
| Class C | 0.00% | 0.04% | 0.19% | 99% | 93% | 0.00% | 0.04% | 0.19% | 99% | 94% |
| Class E | 0.06% | 0.02% | -0.03% | 98% | 91% | 0.06% | 0.02% | -0.03% | 98% | 89% |
| Overall | 0.03% | 0.04% | 0.09% | 98% | 90% | 0.03% | 0.04% | 0.09% | 98% | 90% |
| Class D | -0.02% | 0.33% | 0.00% | 99% | 95% | -0.02% | 0.33% | 0.00% | 99% | 95% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test: CE6.1.6 + TAF in DCT-4 Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6 + EMT=3 + TAF in DCT-4 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.06% | 0.00% | 100% | 98% | 0.01% | -0.03% | -0.05% | 97% | 96% |
| Class A2 | 0.06% | 0.09% | 0.04% | 99% | 98% | 0.04% | 0.07% | 0.01% | 97% | 96% |
| Class B | 0.03% | 0.07% | -0.01% | 100% | 99% | 0.04% | -0.12% | 0.01% | 96% | 94% |
| Class C | 0.04% | 0.14% | 0.07% | 98% | 96% | 0.01% | 0.01% | 0.15% | 97% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.04% | 0.09% | 0.02% | 99% | 98% | 0.03% | -0.03% | 0.04% | 97% | 96% |
| Class D | 0.03% | -0.17% | 0.04% | 100% | 99% | 0.01% | -0.10% | 0.06% | 98% | 98% |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test: CE6.1.6 + TAF in DCT-4 Over VTM-2.0.1 |  |  |  |  | Test: CE6.1.6 + EMT=3 + TAF in DCT-4 Over VTM-2.0.1 + EMT=3 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.05% | -0.17% | 100% | 99% | -0.01% | -0.23% | 0.18% | 96% | 92% |
| Class C | 0.00% | -0.09% | 0.05% | 99% | 98% | -0.03% | 0.04% | 0.19% | 97% | 93% |
| Class E | -0.04% | 0.35% | -0.06% | 99% | 98% | 0.00% | 0.54% | 0.60% | 96% | 96% |
| Overall | 0.00% | 0.04% | -0.07% | 99% | 99% | -0.01% | 0.05% | 0.29% | 96% | 93% |
| Class D | -0.01% | 0.44% | -0.35% | 99% | 97% | -0.07% | 0.63% | 0.76% | 98% | 97% |

## Operation counts and Memory Requirements for the additional test

Table 7 and 8 present the number of operations and the amount of memory required for TAF with the proposed DCT-2 implementation.

Table 7: Operation counts for 1D forward/inverse transforms including TAF for DCT-4

|  | Number of Multiplications |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 8 | 64 | 182 | 534 | 1750 |
| DCT8 | 8 | 64 | 182 | 534 | 1750 |
| DCT2 with TAF | 8 | 24 | 86 | 342 | 705 |

|  | Number of Additions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 15 | 65 | 210 | 594 | 1842 |
| DCT8 | 15 | 65 | 210 | 594 | 1842 |
| DCT2 with TAF | 13 | 37 | 113 | 401 | 857 |

|  | Number of Shifts |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 5 | 9 | 34 | 66 | 98 |
| DCT8 | 5 | 9 | 34 | 66 | 98 |
| DCT2 with TAF | 5 | 9 | 17 | 33 | 33 |

Table 8: Number of elements and bits for each transform matrix

|  | Number of elements in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 16 | 64 | 96 | 192 | 384 | 752 |
| DCT8 | 16 | 64 | 0 | 0 | 0 | 80 |
| DCT2 with TAF | 16 | 64 | 86 | 342 | 534 | 1042 |

|  | Number of bits in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 144 | 576 | 864 | 1728 | 3456 | 6768 |
| DCT8 | 144 | 576 | 0 | 0 | 0 | 720 |
| DCT2 with TAF | 160 | 640 | 860 | 3420 | 5340 | 10420 |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, Slovenia, July 2018.

- A. Said, H. Egilmez, V. Seregin, and M. Karczewicz, “Complexity Reduction for Adaptive Multiple Transforms (AMT) using Adjustment Stages” 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).