    - Deblocking filter process

      - General

Inputs to this process are the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

The vertical edges in a picture are filtered first. Then the horizontal edges in a picture are filtered with samples modified by the vertical edge filtering process as input. The vertical and horizontal edges in the CTBs of each CTU are processed separately on a coding unit basis. The vertical edges of the coding blocks in a coding unit are filtered starting with the edge on the left-hand side of the coding blocks proceeding through the edges towards the right-hand side of the coding blocks in their geometrical order. The horizontal edges of the coding blocks in a coding unit are filtered starting with the edge on the top of the coding blocks proceeding through the edges towards the bottom of the coding blocks in their geometrical order.

NOTE – Although the filtering process is specified on a picture basis in this Specification, the filtering process can be implemented on a coding unit basis with an equivalent result, provided the decoder properly accounts for the processing dependency order so as to produce the same output values.

The deblocking filter process is applied to all prediction block edges and transform block edges of a picture, except the following types of edges:

- Edges that are at the boundary of the picture,

- Edges that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0,

- Edges that coincide with upper or left boundaries of slices with slice_loop_filter_across_slices_enabled_flag equal to 0 or slice_deblocking_filter_disabled_flag equal to 1,

- Edges within slices with slice_deblocking_filter_disabled_flag equal to 1,

- Edges that do not correspond to 8x8 sample grid boundaries of the considered component,

- Edges within chroma components for which both sides of the edge use inter prediction,

- Edges of chroma transform blocks that are not edges of the associated transform unit.

The edge type, vertical or horizontal, is represented by the variable edgeType as specified in Table 8-11.

Table 8-11 – Name of association to edgeType

| edgeType | Name of edgeType |
| --- | --- |
| 0 (vertical edge) | EDGE_VER |
| 1 (horizontal edge) | EDGE_HOR |

When slice_deblocking_filter_disabled_flag of the current slice is equal to 0, for each coding unit with luma coding block size log2CbSize and location of top-left sample of the luma coding block ( xCb, yCb ), the vertical edges are filtered by the following ordered steps:

        - The luma coding block size nCbS is set equal to 1 << log2CbSize.

        - The variable filterLeftCbEdgeFlag is derived as follows:

          - If one or more of the following conditions are true, filterLeftCbEdgeFlag is set equal to 0:

            - The left boundary of the current luma coding block is the left boundary of the picture.

            - The left boundary of the current luma coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

            - The left boundary of the current luma coding block is the left boundary of the slice and slice_loop_filter_across_slices_enabled_flag is equal to 0.

          - Otherwise, filterLeftCbEdgeFlag is set equal to 1.

        - All elements of the two-dimensional (nCbS)x(nCbS) array verEdgeFlags are initialized to be equal to zero.

        - The derivation process of transform block boundary specified in clause 8.7.2.2 is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the transform block size log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable filterLeftCbEdgeFlag, the array verEdgeFlags and the variable edgeType set equal to EDGE_VER as inputs, and the modified array verEdgeFlags as output.

        - The derivation process of prediction block boundary specified in clause 8.7.2.3 is invoked with the luma coding block size log2CbSize, the prediction partition mode PartMode, the array verEdgeFlags and the variable edgeType set equal to EDGE_VER as inputs, and the modified array verEdgeFlags as output.

        - The derivation process of the boundary filtering strength specified in clause 8.7.2.4 is invoked with the reconstructed luma picture sample array prior to deblocking recPictureL, the luma location ( xCb, yCb ), the luma coding block size log2CbSize, the variable edgeType set equal to EDGE_VER and the array verEdgeFlags as inputs, and an (nCbS)x(nCbS) array verBs as output.

        - The vertical edge filtering process for a coding unit as specified in clause 8.7.2.5.1 is invoked with the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when  ChromaArrayType is  not equal to 0, the arrays recPictureCb and recPictureCr, the luma location ( xCb, yCb ), the luma coding block size log2CbSize and the array verBs as inputs, and the modified reconstructed picture, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr, as output.

When slice_deblocking_filter_disabled_flag of the current slice is equal to 0, for each coding unit with luma coding block size log2CbSize and location of top-left sample of the luma coding block ( xCb, yCb ), the horizontal edges are filtered by the following ordered steps:

- The luma coding block size nCbS is set equal to 1 << log2CbSize.

- The variable filterTopCbEdgeFlag is derived as follows:

  - If one or more of the following conditions are true, the variable filterTopCbEdgeFlag is set equal to 0:

    - The top boundary of the current luma coding block is the top boundary of the picture.

    - The top boundary of the current luma coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

    - The top boundary of the current luma coding block is the top boundary of the slice and slice_loop_filter_across_slices_enabled_flag is equal to 0.

  - Otherwise, the variable filterTopCbEdgeFlag is set equal to 1.

- All elements of the two-dimensional (nCbS)x(nCbS) array horEdgeFlags are initialized to zero.

- The derivation process of transform block boundary specified in clause 8.7.2.2 is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the transform block size log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable filterTopCbEdgeFlag, the array horEdgeFlags and the variable edgeType set equal to EDGE_HOR as inputs, and the modified array horEdgeFlags as output.

- The derivation process of prediction block boundary specified in clause 8.7.2.3 is invoked with the luma coding block size log2CbSize, the prediction partition mode PartMode, the array horEdgeFlags and the variable edgeType set equal to EDGE_HOR as inputs, and the modified array horEdgeFlags as output.

- The derivation process of the boundary filtering strength specified in clause 8.7.2.4 is invoked with the reconstructed luma picture sample array prior to deblocking recPictureL, the luma location ( xCb, yCb ), the luma coding block size log2CbSize, the variable edgeType set equal to EDGE_HOR and the array horEdgeFlags as inputs, and an (nCbS)x(nCbS) array horBs as output.

- The horizontal edge filtering process for a coding unit as specified in clause 8.7.2.5.2 is invoked with the modified reconstructed picture, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays

recPictureCb and recPictureCr, the luma location ( xCb, yCb ), the luma coding block size log2CbSize, and the array horBs as inputs and the modified reconstructed picture, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr, as output.

      - Derivation process of transform block boundary

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xB0, yB0 ) specifying the top-left sample of the current luma block relative to the top-left sample of the current luma coding block,

- a variable log2TrafoSize specifying the size of the current block,

- a variable trafoDepth,

- a variable filterEdgeFlag,

- a two-dimensional (nCbS)x(nCbS) array edgeFlags,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered. Output of this process is the modified two-dimensional (nCbS)x(nCbS) array edgeFlags.

Depending on the value of split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ], the following applies:

- If split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 1, the following ordered steps apply:

        - The variables xB1 and yB1 are derived as follows:

- The variable xB1 is set equal to xB0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

- The variable yB1 is set equal to yB0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

        - The derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable filterEdgeFlag, the array edgeFlags and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

        - The derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xCb, yCb ), the luma location ( xB1, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable filterEdgeFlag, the array edgeFlags and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

        - The derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable filterEdgeFlag, the array edgeFlags and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

        - The derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xCb, yCb ), the luma location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable filterEdgeFlag, the array edgeFlags and the variable edgeType as inputs, and the output is the modified version of array edgeFlags.

- Otherwise (split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 0), the following applies:

  - If     edgeType     is     equal      to      EDGE_VER,     the      value     of      edgeFlags[ xB0 ][ yB0 + k ]      for k = 0..( 1 << log2TrafoSize ) − 1 is derived as follows:

    - If xB0 is equal to 0, edgeFlags[ xB0 ][ yB0 + k ] is set equal to filterEdgeFlag.

    - Otherwise, edgeFlags[ xB0 ][ yB0 + k ] is set equal to 1.

  - Otherwise   (edgeType   is    equal    to    EDGE_HOR),    the    value    of    edgeFlags[ xB0 + k ][ yB0 ]    for k = 0..( 1 << log2TrafoSize ) − 1 is derived as follows:

    - If yB0 is equal to 0, edgeFlags[ xB0 + k ][ yB0 ] is set equal to filterEdgeFlag.

    - Otherwise, edgeFlags[ xB0 + k ][ yB0 ] is set equal to 1.

      - Derivation process of prediction block boundary

Inputs to this process are:

- a variable log2CbSize specifying the luma coding block size,

- a prediction partition mode PartMode,

- a two-dimensional (nCbS)x(nCbS) array edgeFlags,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered. Output of this process is the modified two-dimensional (nCbS)x(nCbS) array edgeFlags.

Depending on the values of edgeType and PartMode, the following applies for k = 0..( 1 << log2CbSize ) − 1:

- If edgeType is equal to EDGE_VER, the following applies:

  - When PartMode is equal to PART_Nx2N or PART_NxN, edgeFlags[ 1 << ( log2CbSize − 1 ) ][ k ] is set equal to 1.

  - When PartMode is equal to PART_nLx2N, edgeFlags[ 1 << ( log2CbSize − 2 ) ][ k ] is set equal to 1.

  - When PartMode is equal to PART_nRx2N, edgeFlags[ 3 * ( 1 << ( log2CbSize − 2 ) ) ][ k ] is set equal to 1.

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

  - When PartMode is equal to PART_2NxN or PART_NxN, edgeFlags[ k ][ 1 << ( log2CbSize − 1 ) ] is set equal to 1.

  - When PartMode is equal to PART_2NxnU, edgeFlags[ k ][ 1 << ( log2CbSize − 2 ) ] is set equal to 1.

  - When PartMode is equal to PART_2NxnD, edgeFlags[ k ][ 3 * ( 1 << ( log2CbSize − 2 ) ) ] is set equal to 1.

      - Derivation process of boundary filtering strength

Inputs to this process are:

- a luma picture sample array recPictureL,

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a two-dimensional (nCbS)x(nCbS) array edgeFlags.

Output of this process is a two-dimensional (nCbS)x(nCbS) array bS specifying the boundary filtering strength. The variables xDi, yDj, xN and yN are derived as follows:

- If edgeType is equal to EDGE_VER, xDi is set equal to ( i <<  3 ), yDj is set equal to ( j  <<  2 ), xN is set equal to  ( 1 << ( log2CbSize − 3 ) ) − 1 and yN is set equal to ( 1 << ( log2CbSize − 2 ) ) − 1.

- Otherwise (edgeType is equal to EDGE_HOR), xDi is set equal to ( i << 2 ), yDj is set equal to ( j << 3 ), xN is set equal to ( 1 << ( log2CbSize − 2 ) ) − 1 and yN is set equal to ( 1 << ( log2CbSize − 3 ) ) − 1.

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

  - If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

  - Otherwise (edgeFlags[ xDi ][ yDj ] is equal to 1), the following applies:

    - The sample values p0 and q0 are derived as follows:

      - If edgeType is equal to EDGE_VER, p0 is set equal to recPictureL[ xCb + xDi − 1 ][ yCb + yDj ] and q0 is set equal to recPictureL[ xCb + xDi ][ yCb + yDj ].

      - Otherwise	(edgeType	is	equal	to	EDGE_HOR),	p0	is	set	equal	to recPictureL[ xCb + xDi ][ yCb + yDj − 1 ] and q0 is set equal to recPictureL[ xCb + xDi ][ yCb + yDj ].

    - The variable bS[ xDi ][ yDj ] is derived as follows:

      - If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

      - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

      - Otherwise, if one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

        - For the prediction of the luma prediction block containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the luma prediction block containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two luma prediction blocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a luma prediction block with top-left luma sample covering ( xPb, yPb ), is equal to PredFlagL0[ xPb ][ yPb ]  +  PredFlagL1[ xPb ][ yPb ].

        - One motion vector is used to predict the luma prediction block containing the sample p0 and one motion vector is used to predict the luma prediction block containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 4 in units of quarter luma samples.

        - Two motion vectors and two different reference pictures are used to predict the luma prediction block containing the sample p0, two motion vectors for the same two reference pictures are used to predict the luma prediction block containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two luma prediction blocks for the same reference picture is greater than or equal to 4 in units of quarter luma samples.

        - Two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample p0, two motion vectors for the same reference picture are used to predict the luma prediction block containing the sample q0 and both of the following conditions are true:

          - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two luma prediction blocks is greater than or equal to 4 in quarter luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two luma prediction blocks is greater than or equal to 4 in units of quarter luma samples.

          - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the luma prediction block containing the sample p0 and the list 1 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the luma prediction block containing the sample p0 and list 0 motion vector used in the prediction of the luma prediction block containing the sample q0 is greater than or equal to 4 in units of quarter luma samples.

      - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.

      - Edge filtering process

        - Vertical edge filtering process

Inputs to this process are:

- the picture sample array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr,

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block,

- an array bS specifying the boundary filtering strength.

Outputs of this process are the modified picture sample array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

The filtering process for edges in the luma coding block of the current coding unit consists of the following ordered steps:

          - The variable nD is set equal to 1 << ( log2CbSize − 3 ).

          - For xDk equal to k << 3 with k = 0..nD − 1 and yDm equal to m << 2 with m = 0..nD * 2 − 1, the following applies:

–	When bS[ xDk ][ yDm ] is greater than 0, the following ordered steps apply:

            - The decision process for luma block edges as specified in clause 8.7.2.5.3 is invoked with the luma picture sample array recPictureL, the location of the luma coding block ( xCb, yCb ), the luma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_VER and the boundary filtering strength bS[ xDk ][ yDm ] as inputs, and the decisions dE, dEp and dEq, and the variables β and tC as outputs.

            - The filtering process for luma block edges as specified in clause 8.7.2.5.4 is invoked with the luma picture sample array recPictureL, the location of the luma coding block ( xCb, yCb ), the luma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_VER, the decisions dE, dEp and dEq, and the variables β and tC as inputs, and the modified luma picture sample array recPictureL as output.

When ChromaArrayType is not equal to 0, the following applies.

The filtering process for edges in the chroma coding blocks of current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1  <<  ( log2CbSize − 3 ).

- The variable edgeSpacing is set equal to 8 / SubWidthC.

- The variable edgeSections is set equal to nD * ( 2 / SubHeightC ).

- For xDk equal to k * edgeSpacing with k = 0..nD − 1 and yDm equal to m << 2 with m = 0..edgeSections − 1, the following applies:

–     When	bS[ xDk * SubWidthC ][ yDm * SubHeightC ]	is	equal	to	1	and ( ( ( xCb / SubWidthC + xDk ) >> 3 ) << 3 ) is equal to xCb / SubWidthC + xDk, the following ordered  steps apply:

  - The filtering process for chroma block edges as specified in clause 8.7.2.5.5 is invoked with the chroma picture     sample     array     recPictureCb,     the     location     of     the     chroma     coding     block      ( xCb / SubWidthC, yCb / SubHeightC ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_VER and a variable cQpPicOffset set equal to pps_cb_qp_offset as inputs, and the modified chroma picture sample array recPictureCb as output.

  - The filtering process for chroma block edges as specified in clause 8.7.2.5.5 is invoked with the chroma picture     sample     array     recPictureCr,     the     location     of     the     chroma     coding     block       ( xCb / SubWidthC, yCb / SubHeightC ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_VER and a variable cQpPicOffset set equal to pps_cr_qp_offset as inputs, and the modified chroma picture sample array recPictureCr as output.

        - Horizontal edge filtering process

Inputs to this process are:

- the picture sample array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr,

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block,

- an array bS specifying the boundary filtering strength.

Outputs of this process are the modified picture sample array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

The filtering process for edges in the luma coding block of the current coding unit consists of the following ordered steps:

          - The variable nD is set equal to 1 << ( log2CbSize − 3 ).

          - For yDm equal to m << 3 with m = 0..nD − 1 and xDk equal to k << 2 with k = 0..nD * 2 − 1, the following applies:

–	When bS[ xDk ][ yDm ] is greater than 0, the following ordered steps apply:

            - The decision process for luma block edges as specified in clause 8.7.2.5.3 is invoked with the luma picture sample array recPictureL, the location of the luma coding block ( xCb, yCb ), the luma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_HOR and the boundary filtering strength bS[ xDk ][ yDm ] as inputs, and the decisions dE, dEp and dEq, and the variables β and tC as outputs.

            - The filtering process for luma block edges as specified in clause 8.7.2.5.4 is invoked with the luma picture sample array recPictureL, the location of the luma coding block ( xCb, yCb ), the luma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_HOR, the decisions dEp, dEp and dEq, and the variables β and tC as inputs, and the modified luma picture sample array recPictureL as output.

When ChromaArrayType is not equal to 0, the following applies.

The filtering process for edges in the chroma coding blocks of current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1  <<  ( log2CbSize − 3 ).

- The variable edgeSpacing is set equal to 8 / SubHeightC.

- The variable edgeSections is set equal to nD * ( 2 / SubWidthC ).

- For yDm equal to m * edgeSpacing with m = 0..nD − 1 and xDk equal to k << 2 with k = 0..edgeSections − 1, the following applies:

–     When	bS[ xDk * SubWidthC ][ yDm * SubHeightC ]	is	equal	to	1	and ( ( ( yCb / SubHeightC + yDm ) >> 3 ) << 3 ) is equal to yCb / SubHeightC + yDm, the following ordered steps apply:

  - The filtering process for chroma block edges as specified in clause 8.7.2.5.5 is invoked with the chroma picture     sample     array     recPictureCb,     the     location     of     the     chroma     coding     block      ( xCb / SubWidthC, yCb / SubHeightC ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_HOR and a variable cQpPicOffset set equal to pps_cb_qp_offset as inputs, and the modified chroma picture sample array recPictureCb as output.

  - The filtering process for chroma block edges as specified in clause 8.7.2.5.5 is invoked with the chroma picture     sample     array     recPictureCr,     the     location     of     the     chroma     coding     block       ( xCb / SubWidthC, yCb / SubHeightC ), the chroma location of the block ( xDk, yDm ), a variable edgeType set equal to EDGE_HOR and a variable cQpPicOffset set equal to pps_cr_qp_offset as inputs, and the modified chroma picture sample array recPictureCr as output.

        - Decision process for luma block edges

Inputs to this process are:

- a luma picture sample array recPictureL,

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xBl, yBl ) specifying the top-left sample of the current luma block relative to the top-left sample of the current luma coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable bS specifying the boundary filtering strength.

Outputs of this process are:

- the variables dE, dEp and dEq containing decisions,

- the variables β and tC.

If edgeType is equal to EDGE_VER, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived as follows: qi,k = recPictureL[ xCb + xBl + i ][ yCb + yBl + k ]	(8-345)

pi,k = recPictureL[ xCb + xBl − i − 1 ][ yCb + yBl + k ]	(8-346)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived as follows:

qi,k = recPictureL[ xCb + xBl + k ][ yCb + yBl + i ]	(8-347)

pi,k = recPictureL[ xCb + xBl + k ][ yCb + yBl − i − 1 ]	(8-348)

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable qPL is derived as follows:

qPL = ( ( QpQ + QpP + 1 )  >> 1 )	(8-349)

The value of the variable β′ is determined as specified in Table 8-12 based on the luma quantization parameter Q derived as follows:

Q = Clip3( 0, 51, qPL + ( slice_beta_offset_div2  <<  1 ) )	(8-350)

where slice_beta_offset_div2 is the value of the syntax element slice_beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as follows:

β = β′ * ( 1  <<  ( BitDepthY − 8 ) )	(8-351)

The value of the variable tC′ is determined as specified in Table 8-12 based on the luma quantization parameter Q derived as follows:

Q = Clip3( 0, 53, qPL + 2 * ( bS − 1 ) + ( slice_tc_offset_div2  <<  1 ) )	(8-352)

where slice_tc_offset_div2 is the value of the syntax element slice_tc_offset_div2 for the slice that contains sample q0,0. The variable tC is derived as follows:

tC = tC′ * ( 1  <<  ( BitDepthY − 8 ) )	(8-353)

Depending on the value of edgeType, the following applies:

- If edgeType is equal to EDGE_VER, the following ordered steps apply:

          - The variables dpq0, dpq3, dp, dq and d are derived as follows:

dp0 = Abs( p2,0 − 2 * p1,0 + p0,0 )	(8-354)

dp3 = Abs( p2,3 − 2 * p1,3 + p0,3 )	(8-355)

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )	(8-356)

dq3 = Abs( q2,3 − 2 * q1,3 + q0,3 )	(8-357)

dpq0 = dp0 + dq0	(8-358)

dpq3 = dp3 + dq3	(8-359)

dp = dp0 + dp3	(8-360)

dq = dq0 + dq3	(8-361)

d = dpq0 + dpq3	(8-362)

          - The variables dE, dEp and dEq are set equal to 0.

          - When d is less than β, the following ordered steps apply:

            - The variable dpq is set equal to 2 * dpq0.

            - For the sample location ( xCb + xBl, yCb + yBl ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,0, p3,0, q0,0, and q3,0, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam0.

            - The variable dpq is set equal to 2 * dpq3.

            - For the sample location ( xCb + xBl, yCb + yBl + 3 ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,3, p3,3, q0,3, and q3,3, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam3.

            - The variable dE is set equal to 1.

            - When dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

            - When dp is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEp is set equal to 1.

            - When dq is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEq is set equal to 1.

          - The variable CbSize_P and CbSize_Q are derived as follows:

            - The variable CbSize_P is set to coding block width that p0,0 belongs to and CbSize_Q is set to coding block width that q0,0 belongs to.

          - When dE is equal to 2, the following ordered steps apply:

            - The variable CbSize_Th is derived as follows:

- If bS is equal to 2, CbSize_Th is set equal to 16,

- Otherwise if bS is equal to 1, CbSize_Th is set equal to 32.

            - When CbSize_P is equal to or greater than CbSize_Th, and CbSize_Q is equal to or greater than CbSize_Th, the variable d0_1 and d3_1 are derived as follows:

dp0_1 = Abs( p7,0 −  p0,0 )	(8-xxx)

dp3_1 = Abs( p7,3 −  p0,3 )	(8-xxx)

dq0_1 = Abs( q7,0 – q0,0 )	(8-xxx)

dq3_1 = Abs( q7,3 – q0,3 )	(8-xxx)

d0_1 = dp0_1 + dq0_1	(8-xxx)

d3_1 = dp3_1 + dq3_1	(8-xxx)

            - When d0_1 is less than (β >> 3), and d3_1 is less than (β >> 3), the variable dp_2 and dq_2 are derived as follows:

dp0_2 = Abs( p7,0 −  p4,0 )	(8-xxx)

dp3_2 = Abs( p7,3 −  p4,3 )	(8-xxx)

dq0_2 = Abs( q7,0 – q4,0 )	(8-xxx)

dq3_2 = Abs( q7,3 – q4,3 )	(8-xxx)

dp_2 = dp0_2 + dp3_2	(8-xxx)

dq_2 = dq0_2 + dq3_2	(8-xxx)

            - When dp_2 is less than (β >> 3), add 4 to dE.

            - When dq_2 is less than (β >> 3), add 8 to dE.

- Otherwise (edgeType is equal to EDGE_HOR), the following ordered steps apply:

- The variables dpq0, dpq3, dp, dq and d are derived as follows:

dp0 = Abs( p2,0 − 2 * p1,0 + p0,0 )	(8-363)

dp3 = Abs( p2,3 − 2 * p1,3 + p0,3 )	(8-364)

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )	(8-365)

dq3 = Abs( q2,3 − 2 * q1,3 + q0,3 )	(8-366)

dpq0 = dp0 + dq0	(8-367)

dpq3 = dp3 + dq3	(8-368)

dp = dp0 + dp3	(8-369)

dq = dq0 + dq3	(8-370)

d = dpq0 + dpq3	(8-371)

- The variables dE, dEp and dEq are set equal to 0.

- When d is less than β, the following ordered steps apply:

  - The variable dpq is set equal to 2 * dpq0.

  - For the sample location ( xCb + xBl, yCb + yBl ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,0, p3,0, q0,0 and q3,0, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam0.

  - The variable dpq is set equal to 2 * dpq3.

  - For the sample location ( xCb + xBl + 3, yCb + yBl ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,3, p3,3, q0,3 and q3,3, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam3.

  - The variable dE is set equal to 1.

  - When dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

  - When dp is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEp is set equal to 1.

  - When dq is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEq is set equal to 1.

- The variable CbSize_P and CbSize_Q are derived as follows:

  - The variable CbSize_P is set to coding block width that p0,0 belongs to and CbSize_Q is set to coding block width that q0,0 belongs to.

- When dE is equal to 2, the following ordered steps apply:

  - The variable CbSize_Th is derived as follows:

- If bS is equal to 2, CbSize_Th is set equal to 16,

- Otherwise if bS is equal to 1, CbSize_Th is set equal to 32.

  - When CbSize_P is equal to or greater than CbSize_Th, and CbSize_Q is equal to or greater than CbSize_Th, the variable d0_1 and d3_1 are derived as follows:

dp0_1 = Abs( p7,0 −  p0,0 )	(8-xxx)

dp3_1 = Abs( p7,3 −  p0,3 )	(8-xxx)

dq0_1 = Abs( q7,0 – q0,0 )	(8-xxx)

dq3_1 = Abs( q7,3 – q0,3 )	(8-xxx)

d0_1 = dp0_1 + dq0_1	(8-xxx)

d3_1 = dp3_1 + dq3_1	(8-xxx)

            - When d0_1 is less than (β >> 3), and d3_1 is less than (β >> 3), the variable dp_2 and dq_2 are derived as follows:

dp0_2 = Abs( p7,0 −  p4,0 )	(8-xxx)

dp3_2 = Abs( p7,3 −  p4,3 )	(8-xxx)

dq0_2 = Abs( q7,0 – q4,0 )	(8-xxx)

dq3_2 = Abs( q7,3 – q4,3 )	(8-xxx)

dp_2 = dp0_2 + dp3_2	(8-xxx)

dq_2 = dq0_2 + dq3_2	(8-xxx)

            - When dp_2 is less than (β >> 3), add 4 to dE.

            - When dq_2 is less than (β >> 3), add 8 to dE.

Table 8-12 – Derivation of threshold variables β′ and tC′ from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β′ | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC′ | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |  |  |  |
| β′ | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | - | - |  |  |  |
| tC′ | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 22 | 24 |  |  |  |

        - Filtering process for luma block edges

Inputs to this process are:

- a luma picture sample array recPictureL,

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a luma location ( xBl, yBl ) specifying the top-left sample of the current luma block relative to the top-left sample of the current luma coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- the variables dE, dEp and dEq containing decisions,

- the variables β and tC.

Output of this process is the modified luma picture sample array recPictureL. Depending on the value of edgeType, the following applies:

- If edgeType is equal to EDGE_VER, the following ordered steps apply:

          - The sample values pi,k and qi,k with i = 0..7 and k = 0..3 are derived as follows:

qi,k = recPictureL[ xCb + xBl + i ][ yCb + yBl + k ]	(8-372)

pi,k = recPictureL[ xCb + xBl − i − 1 ][ yCb + yBl + k ]	(8-373)

          - When dE is not equal to 0, for each sample location ( xCb + xBl, yCb + yBl + k ), k = 0..3, the following ordered steps apply:

            - The filtering process for a luma sample as specified in clause 8.7.2.5.7 is invoked with the sample values pi,k, qi,k with i = 0..7, the locations ( xPi, yPi ) set equal to ( xCb + xBl − i − 1, yCb + yBl + k ) and ( xQi, yQi ) set equal to ( xCb + xBl + i, yCb + yBl + k ) with i = 0..6, the decision dE, the variables dEp and dEq and the variable tC as inputs, and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi' and qj' as outputs.

            - When nDp is greater than 0, the filtered sample values pi' with i = 0..nDp − 1 replace the corresponding samples inside the sample array recPictureL as follows:

recPictureL[ xCb + xBl − i − 1 ][ yCb + yBl + k ] = pi'	(8-374)

            - When nDq is greater than 0, the filtered sample values qj' with j = 0..nDq − 1 replace the corresponding samples inside the sample array recPictureL as follows:

recPictureL[ xCb + xBl + j ][ yCb + yBl + k ] = qj'	(8-375)

- Otherwise (edgeType is equal to EDGE_HOR), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..7 and k = 0..3 are derived as follows:

qi,k = recPictureL[ xCb + xBl + k ][ yCb + yBl + i ]	(8-376)

pi,k = recPictureL[ xCb + xBl + k ][ yCb + yBl − i − 1 ]	(8-377)

- When dE is not equal to 0, for each sample location ( xCb + xBl + k, yCb + yBl ), k = 0..3, the following ordered steps apply:

  - The filtering process for a luma sample as specified in clause 8.7.2.5.7 is invoked with the sample values pi,k, qi,k with i = 0..7, the locations ( xPi, yPi ) set equal to ( xCb + xBl + k, yCb + yBl − i − 1 ) and ( xQi, yQi ) set equal to ( xCb + xBl + k, yCb + yBl + i ) with i = 0..6, the decision dE, the variables dEp and dEq, and the variable tC as inputs, and the number of filtered samples nDp and nDq from each side of the block boundary and the filtered sample values pi' and qj' as outputs.

  - When nDp is greater than 0, the filtered sample values pi' with i = 0..nDp − 1 replace the corresponding samples inside the sample array recPictureL as follows:

recPictureL[ xCb + xBl + k ][ yCb + yBl − i − 1 ] = pi'	(8-378)

  - When nDq is greater than 0, the filtered sample values qj' with j = 0..nDq − 1 replace the corresponding samples inside the sample array recPictureL as follows:

recPictureL[ xCb + xBl + k ][ yCb + yBl + j ] = qj'	(8-379)

        - Filtering process for chroma block edges

This process is only invoked when ChromaArrayType is not equal to 0. Inputs to this process are:

- a chroma picture sample array s′,

- a chroma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top- left chroma sample of the current picture,

- a chroma location ( xBl, yBl ) specifying the top-left sample of the current chroma block relative to the top-left sample of the current chroma coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cQpPicOffset specifying the picture-level chroma quantization parameter offset. Output of this process is the modified chroma picture sample array s′.

If edgeType is equal to EDGE_VER, the values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s′[ xCb + xBl + i ][ yCb + yBl + k ]	(8-380)

pi,k = s′[ xCb + xBl − i − 1 ][ yCb + yBl + k ]	(8-381)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi and qi with i = 0..1 and k = 0..3 are derived as follows: qi,k = s′[ xCb + xBl + k ][ yCb + yBl + i ]	(8-382)

pi,k = s′[ xCb + xBl + k ][ yCb + yBl − i − 1 ]	(8-383)

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

If ChromaArrayType is equal to 1, the variable QpC is determined as specified in Table 8-10 based on the index qPi derived as follows:

qPi = ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset	(8-384) Otherwise (ChromaArrayType is greater than 1), the variable QpC is set equal to Min( qPi, 51 ).

NOTE – The variable cQpPicOffset provides an adjustment for the value of pps_cb_qp_offset or pps_cr_qp_offset, according to whether the filtered chroma component is the Cb or Cr component. However, to avoid the need to vary the amount of the adjustment within the picture, the filtering process does not include an adjustment for the value of slice_cb_qp_offset or slice_cr_qp_offset, nor (when chroma_qp_offset_list_enabled_flag is equal to 1) for the value of CuQpOffsetCb or CuQpOffsetCr.

The value of the variable tC′ is determined as specified in Table 8-12 based on the chroma quantization parameter Q derived as follows:

Q = Clip3( 0, 53, QpC + 2 + ( slice_tc_offset_div2  <<  1 ) )	(8-385)

where slice_tc_offset_div2 is the value of the syntax element slice_tc_offset_div2 for the slice that contains sample q0,0. The variable tC is derived as follows:

tC = tC′ * ( 1  <<  ( BitDepthC − 8 ) )	(8-386)

The value of the variable β′ is determined as specified in Table 8-12 based on the chroma quantization parameter Q derived as follows:

Q = Clip3( 0, 51, QpC + ( slice_beta_offset_div2  <<  1 ) )	(8-xxx)

where slice_beta_offset_div2 is the value of the syntax element slice_beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as follows:

β = β′ * ( 1  <<  ( BitDepthC − 8 ) )	(8-xxx)

Depending on the value of edgeType, the following applies:

- If edgeType is equal to EDGE_VER, for each sample location ( xCb + xBl, yCb + yBl + k ), k = 0..3, the following ordered steps apply:

          - The variable CbSize_P is set to coding block width that p0,0 belongs to and CbSize_Q is set to coding block width that q0,0 belongs to, and the variable dE is set equal to 0.

          - When bS is greater than 1, the variable dE is set equal to 1.

          - The variable CbSize_Th is derived as follows:

- If bS is equal to 2, CbSize_Th is set equal to 8,

- Otherwise if bS is equal to 1, CbSize_Th is set equal to 32.

          - When CbSize_P is equal to or greater than CbSize_Th, and CbSize_Q is equal to or greater than CbSize_Th, the variable dE is set equal to 1, and the decision process for a chroma block edges as specified in clause 8.7.2.5.9 is invoked with the sample values pi,k, qi,k, with i = 0..3, the locations ( xCb + xBl − 1, yCb + yBl + k ) and ( xCb + xBl, yCb + yBl + k ) and the variable tC , β as inputs, the variable dE as outputs.

          - The filtering process for a chroma sample as specified in clause 8.7.2.5.8 is invoked with the sample values pi,k, qi,k, with i = 0..3, the locations ( xCb + xBl − 1, yCb + yBl + k ) and ( xCb + xBl, yCb + yBl + k ), the variable tC and the variable dE as inputs, and the number of filtered samples nD, the filtered sample values pi′ and qi′, with i = 0..nD as outputs.

          - The filtered sample values pi′ and qi′ with i = 0..nD replace the corresponding samples inside the sample array s′ as follows:

s′[ xCb + xBl + i][ yCb + yBl + k ] = qi′	(8-387)

s′[ xCb + xBl – i – 1  ][ yCb + yBl + k ] = pi′	(8-388)

- Otherwise (edgeType is equal to EDGE_HOR), for each sample location ( xCb + xBl + k, yCb + yBl ), k = 0..3, the following ordered steps apply:

- The variable CbSize_P is set to coding block width that p0,0 belongs to and CbSize_Q is set to coding block width that q0,0 belongs to, and the variable dE is set equal to 0.

- When bS is greater than 1, the variable dE is set equal to 1.

- The variable CbSize_Th is derived as follows:

- If bS is equal to 2, CbSize_Th is set equal to 8,

- Otherwise if bS is equal to 1, CbSize_Th is set equal to 32.

- When CbSize_P is equal to or greater than CbSize_Th, and CbSize_Q is equal to or greater than CbSize_Th, the variable dE is set equal to 1, and the decision process for a chroma block edges as specified in clause 8.7.2.5.9 is invoked with the sample values pi,k, qi,k, with i = 0..3, the locations ( xCb + xBl − 1, yCb + yBl + k ) and ( xCb + xBl, yCb + yBl + k ) and the variable tC , β as inputs, the variable dE as outputs.

- The filtering process for a chroma sample as specified in clause 8.7.2.5.8 is invoked with the sample values pi,k, qi,k, with i = 0..3, the locations ( xCb + xBl − 1, yCb + yBl + k ) and ( xCb + xBl, yCb + yBl + k ), the variable tC and the variable dE as inputs, and the number of filtered samples nD, the filtered sample values pi′ and qi′, with i = 0..nD as outputs.

- The filtered sample values pi′ and qi′ with i = 0..nD replace the corresponding samples inside the sample array s′ as follows:

s′[ xCb + xBl + k ][ yCb + yBl + i ] = qi′	(8-389)

s′[ xCb + xBl + k ][ yCb + yBl – i − 1 ] = pi′	(8-390)

        - Decision process for a luma sample

Inputs to this process are:

- the sample values p0, p3, q0 and q3,

- the variables dpq, β and tC.

Output of this process is the variable dSam containing a decision. The variable dSam is specified as follows:

- If dpq is less than ( β  >>  2 ),  Abs( p3 − p0 ) + Abs( q0 − q3 )  is  less than ( β  >>  3 )  and  Abs( p0 − q0 )  is less than ( 5 * tC + 1 ) >> 1, dSam is set equal to 1.

- Otherwise, dSam is set equal to 0.

        - Filtering process for a luma sample

Inputs to this process are:

- the luma sample values pi and qi with i = 0..7,

- the luma locations of pi and qi, ( xPi, yPi ) and ( xQi, yQi ) with i = 0..6,

- a variable dE,

- the variables dEp and dEq containing decisions to filter samples p1 and q1, respectively,

- a variable tC.

Outputs of this process are:

- the number of filtered samples nDp and nDq,

- the filtered sample values pi′ and qj′ with i = 0..nDp − 1, j = 0..nDq − 1. Depending on the value of dE, the following applies:

- If the variable dE is greater than 2, the following orderd steps apply:

  - If the variable (dE & 0x4) is true, nDp is set equal to 7 and the following extended strong filtering applies:

p0′ = Clip3( p0 − 2 * tC, p0 + 2 * tC, ( p4 + 2 * p3 + 2 * p2 + 2 * p1 + 2 * p0 + 2 * q0 + 2 * q1  + 2 * q2 + q3 + 8 )  >> 4 )	(8-xxx)

p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p5 + p4 +2 * p3 + 2 * p2 + 2 * p1 + 2 * p0 + 2 * q0 + 2 * q1  +  q2 + q3 + 8 )  >> 4 )	(8-xxx)

p2′ = Clip3( p2 − 2 * tC, p2 + 2*tC, ( p5 + 2 * p4 +2 * p3 + 2 * p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1  +  q2 + q3 + 8 )  >> 4 )	(8-xxx)

p3′ = Clip3( p3 − 2 * tC, p3 + 2 * tC, ( p6 + 2 * p5 + 2 * p4 +2 * p3 + 2 * p2 + 2 * p1 + 2 * p0 + q0 + q1  +  q2 + q3 + 8 )  >> 4  )	(8-xxx)

p4′ = Clip3( p4 − 2 * tC, p4 + 2 * tC, ( p6 + 3 * p5 + 3 * p4 +3 * p3 + p2 + p1 + p0 + q0 + q1  +  q2 + 8 )  >> 4 )	(8-xxx)

p5′ = Clip3( p5 − 2 * tC, p5 + 2*tC, ( p7 + 2 * p6 + 5 * p5 + 2 * p4 + p3 + p2 + p1 + p0 + q0 + q1 + 8 )  >> 4  )	(8-xxx)

p6′ = Clip3( p6 − 2 * tC, p6 + 2*tC, ( 3 * p7 + 5 * p6 + 2 * p5 + p4 + p3 + p2 + p1 + p0 + q0 + 8 )  >> 4 )	(8-xxx)

  - Otherwise, nDp is set equal to 3 and the following strong filtering applies:

p0′ = Clip3( p0 − 2 * tC, p0 + 2 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >> 3 )	(8-xxx)

p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 )  >> 2 )	(8-xxx)

p2′ = Clip3( p2 − 2 * tC, p2 + 2*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >> 3 )	(8-xxx)

  - If the variable (dE & 0x8) is true, nDq is set equal to 7 and the following extended strong filtering applies:

q0′ = Clip3( q0 − 2 * tC, q0 + 2 * tC, ( q4 + 2 * q3 + 2 * q2 + 2 * q1 + 2 * q0 + 2 * p0 + 2 * p1  + 2 * p2 + p3 + 8 )  >> 4 )	(8-xxx)

q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( q5 + q4 +2 * q3 + 2 * q2 + 2 * q1 + 2 * q0 + 2 * p0 + 2 * p1  +  p2 + p3 + 8 )  >> 4 )	(8-xxx)

q2′ = Clip3( q2 − 2 * tC, q2 + 2*tC, ( q5 + 2 * q4 +2 * q3 + 2 * q2 + 2 * q1 + 2 * q0 + 2 * p0 + p1  +  p2 + p3 + 8 )  >> 4 )	(8-xxx)

q3′ = Clip3( q3 − 2 * tC, q3 + 2 * tC, ( q6 + 2 * q5 + 2 * q4 +2 * q3 + 2 * q2 + 2 * q1 + 2 * q0 + p0 + p1  +  p2 + p3 + 8 )  >> 4  )	(8-xxx)

q4′ = Clip3( q4 − 2 * tC, q4 + 2 * tC, ( q6 + 3 * q5 + 3 * q4 +3 * q3 + q2 + q1 + q0 + p0 + p1  +  p2 + 8 )  >> 4 )	(8-xxx)

q5′ = Clip3( q5 − 2 * tC, q5 + 2*tC, ( q7 + 2 * q6 + 5 * q5 + 2 * q4 + q3 + q2 + q1 + q0 + p0 + p1 + 8 )  >> 4  )	(8-xxx)

q6′ = Clip3( q6 − 2 * tC, q5 + 2*tC, ( 3 * q7 + 5 * q6 + 2 * q5 + q4 + q3 + q2 + q1 + q0 + p0 + 8 )  >> 4 )	(8-xxx)

  - Otherwise, nDq is set equal to 3 and the following strong filtering applies:

q0′ = Clip3( q0 − 2 * tC, q0 + 2 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >> 3 )	(8-xxx)

q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 )  >> 2 )	(8-xxx)

q2′= Clip3( q2 − 2 * tC, q2 + 2 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >> 3 )	(8-xxx)

- Otherwise if the variable dE is equal to 2, nDp and nDq are both set equal to 3 and the following strong filtering applies:

p0′ = Clip3( p0 − 2 * tC, p0 + 2 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >> 3 )	(8-391)

p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 )  >> 2 )	(8-392)

p2′ = Clip3( p2 − 2 * tC, p2 + 2*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >> 3 )	(8-393)

q0′ = Clip3( q0 − 2 * tC, q0 + 2 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >> 3 )	(8-394)

q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 )  >> 2 )	(8-395)

q2′= Clip3( q2 − 2 * tC, q2 + 2 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >> 3 )	(8-396)

- Otherwise, nDp and nDq are set both equal to 0 and the following weak filtering applies:

  - The following applies:

 = ( 9 * ( q0 −  p0 ) − 3 * ( q1 − p1 ) + 8 ) >> 4	(8-397)

  - When Abs() is less than tC * 10, the following ordered steps apply:

          - The filtered sample values p0′ and q0′ are specified as follows:

 = Clip3( −tC, tC,  )	(8-398)

p0′ = Clip1Y( p0 +  )	(8-399)

q0′ = Clip1Y( q0 −  )	(8-400)

          - When dEp is equal to 1, the filtered sample value p1′ is specified as follows:

p = Clip3( −( tC  >>  1 ), tC  >>  1, ( ( ( p2 + p0 + 1 )  >>  1 ) − p1 +  )  >> 1 )	(8-401)

p1′ = Clip1Y( p1 + p )	(8-402)

          - When dEq is equal to 1, the filtered sample value q1′ is specified as follows:

q = Clip3( −( tC  >>  1 ), tC  >>  1, ( ( ( q2 + q0 + 1 )  >>  1 ) − q1 −  )  >> 1 )	(8-403)

q1′ = Clip1Y( q1 + q )	(8-404)

          - nDp is set equal to dEp + 1 and nDq is set equal to dEq + 1.

When nDp is greater than 0 and one or more of the following conditions are true, nDp is set equal to 0:

- pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xP0 ][ yP0 ] is equal to 1.

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

- palette_mode_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1. When nDq is greater than 0 and one or more of the following conditions are true, nDq is set equal to 0:

- pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQ0 ][ yQ0 ] is equal to 1.

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

- palette_mode_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

        - Filtering process for a chroma sample

This process is only invoked when ChromaArrayType is not equal to 0. Inputs to this process are:

- the chroma sample values pi and qi with i = 0..1,

- the chroma locations of p0 and q0, ( xP0, yP0 ) and ( xQ0, yQ0 ),

- a variable tC.

Outputs of this process are:

- the number of filtered samples nD,

- the filtered sample values p0′ and q0′.

The filtered sample values are derived as follows:

- If the variable dE is equal to 2, nD is set equal to 3 and the following strong filtering applies:

p0′ = Clip3( p0 − 2 * tC, p0 + 2 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >> 3 )	(8-xxx)

p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 )  >> 2 )	(8-xxx)

p2′ = Clip3( p2 − 2 * tC, p2 + 2*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >> 3 )	(8-xxx)

q0′ = Clip3( q0 − 2 * tC, q0 + 2 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >> 3 )	(8-xxx)

q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 )  >> 2 )	(8-xxx)

q2′= Clip3( q2 − 2 * tC, q2 + 2 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >> 3 )	(8-xxx)

- If the variable dE is equal to 1, nD is set equal to 1 and the following strong filtering applies:

 = Clip3( −tC, tC, ( ( ( ( q0 − p0 )  <<  2 ) + p1 − q1 + 4 )  >>  3 ) )	(8-405)

p0′ = Clip1C( p0 +  )	(8-406)

q0′ = Clip1C( q0 −  )	(8-407)

When one or more of the following conditions are true, the filtered sample value, p0′ is substituted by the corresponding input sample value p0:

  - pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xP0 * SubWidthC ][ yP0 * SubHeightC ] is equal to 1.

  - cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

  - palette_mode_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

When one or more of the following conditions are true, the filtered sample value, q0′ is substituted by the corresponding input sample value q0:

  - pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQ0 * SubWidthC ][ yQ0 * SubHeightC ] is equal to 1.

  - cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

  - palette_mode_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

        - Decision process for a chroma block edges

This process is only invoked when ChromaArrayType is not equal to 0. Inputs to this process are:

- the chroma sample values pi,k and qi,k with i = 0..3, k=0..3,

- the variable tc, β

Outputs of this process are:

- the variables dE, containing decisions,

          - The variables dpq0, dpq3, dp, dq and d are derived as follows:

dp0 = Abs( p2,0 − 2 * p1,0 + p0,0 )	(8-xxx)

dp3 = Abs( p2,3 − 2 * p1,3 + p0,3 )	(8-xxx)

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )	(8-xxx)

dq3 = Abs( q2,3 − 2 * q1,3 + q0,3 )	(8-xxx)

dpq0 = dp0 + dq0	(8-xxx)

dpq3 = dp3 + dq3	(8-xxx)

dp = dp0 + dp3	(8-xxx)

dq = dq0 + dq3	(8-xxx)

d = dpq0 + dpq3	(8-xxx)

          - When d is less than β, the following ordered steps apply:

            - The variable dpq is set equal to 2 * dpq0.

            - The decision process for a chroma sample as specified in clause 8.7.2.5.10 is invoked with sample values p0,0, p3,0, q0,0, and q3,0, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam0.

            - The variable dpq is set equal to 2 * dpq3.

            - The decision process for a luma sample as specified in clause 8.7.2.5.10 is invoked with sample values p0,3, p3,3, q0,3, and q3,3, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam3.

            - When dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

        - Decision process for a chroma sample

Inputs to this process are:

- the sample values p0, p3, q0 and q3,

- the variables dpq, β and tC.

Output of this process is the variable dSam containing a decision. The variable dSam is specified as follows:

- If dpq is less than ( β  >>  2 ),  Abs( p3 − p0 ) + Abs( q0 − q3 )  is  less than ( β  >>  3 )  and  Abs( p0 − q0 )  is less than ( 5 * tC + 1 ) >> 1, dSam is set equal to 1.

- Otherwise, dSam is set equal to 0.