| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0401 |
| --- | --- |

| Title: | CE4-related: Modification on History-based Mode Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Yu Han, Han Huang, Chun-Chi Chen, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1795 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents 2 changes on History-based Motion Vector Prediction (HMVP) mode [1] which reduces the complexity in both encoder side and decoder side. For the “VTM2.0.1 as reference” tests in RA case with the size of merge list equal to 8, the presented method with HMVP buffer size equal to 16 achieved objective gain of -0.56% in terms of average luma BD-rate improvement. The presented method with HMVP buffer size equal to 20 achieved -0.58% coding gain.

# Introduction

The HMVP method for merge mode [1] is firstly presented in 11th JVET meeting and studied in CE4. In this method, a table with multiple HMVP candidates is generated during encoder/decoding process. A First-In-First-Out (FIFO) rule is applied to updating the table, and redundancy check is also use when inserting a new HMVP candidate to the table. The table must be updated at every time encoding/decoding an inter CU. This contribution presents simplification methods to reduce the complexity of HMVP mode and achieves comparable coding gain.

# Technical description

## Update HMVP table without pruning

In CE4.4.7, the size of HMVP table is 6. In the worst case, 6 pruning operations are need for updating the HMVP table at every time of decoding an inter CU in the decoder side. The pruning operations include reference direction, reference POC, and motion vector comparisons between two motion vector predictors. According to our simulation results based on the CTC in RA case with size of merge list equal to 8, the percentage of CU coded in HMVP mode is only 17.14% in inter coded CU as shown in Table 1. (CUs in affine mode aren’t added. The motion vector of CU in affine mode is not used to update HMVP table). In Table 2, the percentage of pruning operation in merge list generation is presented. It shows that only 35.71% of pruning is used for merge list generation in the decoder side, and 64.29% pruning is used for HMVP table update. So, lots of inefficient motion vector scaling, comparison operations consume limited encoder/decoder resource.

Table 1. Percentage of HMVP mode in inter CU

Percentage=num_HMVPnum_Inter-num_Affine

Table 2. Percentage of pruning for merge list generation

Percentage=num_Prun_for_Merge_List_Generationnum_Prun_for_Merge_List_Generation+num_Prun_for_HMVP_Table_Update

Since the HMVP table is used only when the CU is coded in merge mode and refer to a HMVP candidate, it is not necessary to update the HMVP table at every time of encoding/decoding an inter CU. In this conurbation, redundancy check is removed from HMVP table update.

## Subsampling HMVP table when adding HMVP candidates in merger list

In this contribution, the size of HMVP table is increased to 20 or 16, and one out of every 4 is picked to add in the merge list.

In order to reduce the number of pruning, restrict the maximum number of pruning to be 6 when adding a new HMVP candidate in merge list. In the case of merge list size equal to 8, the maximum number of pruning is 30 which is the same as HMVP method in CE4.4.7 in the worst case. The details of pruning number comparation is shown in the table below:

Table 2. Number of pruning

# Simulation results

As descripted in CE4 [2], the simulation is based on VTM2.0.1 with the size of merge list equal to 8.

## The test results of HMVP table equal to 20

Simulation results as below, the anchor is VTM2.0.1 with default setting and HMVP mode in CE4.4.7 with default setting:

## The test results of HMVP table equal to 16

Simulation results as below, the anchor is VTM2.0.1 with default setting and HMVP mode in CE4.4.7 with default setting:

# Conclusion

This contribution presented simplification methods to reduce the complexity of HMVP mode. For the “VTM2.0.1 as reference” tests in RA case with the size of merge list equal to 8, the presented method with HMVP buffer size equal to 16 achieved objective gain of -0.56% in terms of average luma BD-rate improvement. The presented method with HMVP buffer size equal to 20 achieved -0.58% coding gain.

# References

- L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, JVET-K0104, “CE4-related: History-based Motion Vector Prediction”, Ljubljana, July 2018.

- H. Yang, S. Liu, K. Zhang, JVET-K1024, “Description of CE4: Inter prediction and motion vector coding”, Ljubljana, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).