| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0402-v1 |
| --- | --- |

| Title: | CE7-related: Complexity reduction of significance map coding and bypass of greater than 4 flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung Jianle Chen | Tel: Email: | 1-408-330-4690 cheung.auyeung@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution describes an approach which combines complexity reduction in the context modelling of significance map with “Reduction of the number of context-coded bins” from CE7.1.3. In particular, this document reports an approach which reduces the number of operations to determine the context index for the coding of sig_coeff_flag and/or limits the number of rem_abs_gt2_flag to improve the throughtput of CABAC. Partial results of luman BDR is reported.

# Introduction

This contribution describes an approach which reduces the number of operations to determine the context index for the coding of sig_coeff_flag and/or limits the number of rem_abs_gt2_flag to improve the throughtput of CABAC.

# Complexity reduction in sginificance map coding

## Selection of context index of significance map in VVC2

In VVC2 [2], the selection of the context index of the probability model for the coding of the syntax element sig_coeff_flag by CABAC is determined by a sum of a context index offset and a context index base. The context index base is determined by the location of the coefficient. The context index offset is determined by the values of the absolute levels or partially reconstructed absolute levels in a local neighbourhood known as template. The template used in VVC2 [2] is illustrated in Figure 1. This contribution is intended to reduce the computation of the context index offset.

Figure 1: Illustration of the template used for selecting probability models. The black square specifies the current scan position and the blue squares represent the local neighbourhood used.

Let AbsLevel[ xC ][ yC ] be the absolute value of a transform coefficient located at ( xC , yC ) of a block of tranform coefficient. If ( xC , yC ) is outside the block, AbsLevel[ xC ][ yC ] is considered to have a value of zero.

Let ctxIdOffsetSig[ xC ][ yC ] be the context index offset of sig_coeff_flag[ xC ][ yC ]. VVC2 set ctxIdOffsetSig[ xC ][ yC ] has the following value:

sumAbs = absLevelClip2(AbsLevel[ xC +1][ yC ]) + absLevelClip2(AbsLevel[ xC +2][ yC ]) +

absLevelClip2(AbsLevel[ xC ][ yC +1]) + absLevelClip2(AbsLevel[ xC +1][ yC+1 ]) +

absLevelClip2(AbsLevel[ xC ][ yC +2])

ctxIdOffsetSig[ xC ][ yC ] = min(sumAbs, 5)

where the function absLevelClip2(a) is defined as

absLevelClip2(a) = max(4 – a&1, a)

and the operator & is the bitwise and operator.

The function absLevelClip2(a) depends on the parity of the absolute value a. The function absLevelClip2(a) clips the value to two values. If a is even, it is clipped to a maximum value of 4. If a is odd. It is clipped to a maximum value of 3. If a coefficient inside the template has a value 5, it would result in a smaller value on ctxIdOffsetSig[ xC ][ yC ]  than a coefficient in the template with a value of 4.

## Proposed operation reduction in the selection of context index of significance map

To reduce the computation of ctxIdOffsetSig[ xC ][ yC ], this contribution propose to determine ctxIdOffsetSig[ xC ][ yC ] as in the followings:

sumAbs = absLevelClip(AbsLevel[ xC +1][ yC ]) + absLevelClip(AbsLevel[ xC +2][ yC ]) +

absLevelClip(AbsLevel[ xC ][ yC +1]) + absLevelClip(AbsLevel[ xC +1][ yC+1 ]) +

absLevelClip(AbsLevel[ xC ][ yC +2])

ctxIdOffsetSig[ xC ][ yC ] = min(sumAbs, 5)

where the function absLevelClip(a) reduced the number of operations in absLevelClip2(a). It is defined as

absLevelClip(a) = max(3, a)

The function absLevelClip(a) does not depend on the parity of the absolute value a. It is a monotonically non-decreasing function of a.

# Reduction of the number of context-coded rem_abs_gt2_flag bins

This contribution reports an approach to limit the number of context-coded rem_abs_gt2_flag bins to be encoded in a CG; and increase the number of abs_remainder syntax elements that are encoded in bypass mode.

Let N <16 be a positive integer which indicates the maximum number of rem_abs_gt2_flag that can be encoded in a CG. The syntax element for encoding the coefficients with at least one nonzero transform coefficient contains the following syntax elements coded in four passes.

# First pass is encoded in regular mode. For each coefficient in the CG, encode the following syntax elements as in JVET-K1001-v4 [2],

# When sig_coeff_flag is not default to 1, encode the syntax element sig_coeff_flag which indicates whether the current coefficient is nonzero.

# When sig_coeff_flag indicates that the current coefficient is nonzero, signal the syntax elements par_level_flag and rem_abs_gt1_flag.

# The par_level_flag indicates whether the coefficient is odd or even (0: odd, 1: even).

# The rem_abs_gt1_flag indicates whether the absolute value of the coefficient is greater than 2.

# Second pass is encoded in regular mode. For the first N coefficient in the CG with the absolute value of the coefficient greater than 2, encode the syntax element rem_abs_gt2_flag which indicates whether the absolute value of a coefficient is greater than 4.

# Third pass is encoded in bypass mode. For each coefficient in the CG with an absolute value which is not completely signaled in the previous passes, encode the syntax element abs_remainder which indicates the remaining value of the absolute value of the coefficient level that is not signaled in the previous passes.

# Fourth pass is encoded in bypass mode. For each nonzero coefficient in the CG, encode the syntax element coeff_sign_flag which indicates the sign of a significant coefficient (0: positive, 1: negative).

## Residual coding syntax with limits on the number of rem_abs_gt2_flag

Let MaxNumGt2Flag be the maximum number of context-coded rem_abs_gt2_flag. The following syntax table describe the decoding process with the number of rem_abs_gt2_flag limited to MaxNumGt2Flag:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( transform_skip_enabled_flag && ( cIdx ! = 0 ¦ ¦ cu_mts_flag[ x0 ][ y0 ] = = 0 ) && ( log2TbWidth <= 2 ) && ( log2TbHeight <= 2 ) ) |  |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numSigCoeff = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| numSigCoeff++ |  |  |
| par_level_flag[ n ] | ae(v) |
| rem_abs_gt1_flag[ n ] | ae(v) |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + 2 * rem_abs_gt1_flag[ n ] |  |
| if( dep_quant_enabled_flag ) |  |
| QState = QStateTransTable[ QState ][ par_level_flag[ n ] ] |  |
| } |  |
| idxGt2Flag = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| if( rem_abs_gt1_flag[ n ] ) { |  |
| if(idxGt2Flag < MaxNumGt2Flag) |  |
| rem_abs_gt2_flag[ n ] | ae(v) |
| } |  |
| idxGt2Flag++ |  |
| } |  |
| } |  |
| idxGt2Flag = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * rem_abs_gt2_flag[ n ] |  |
| if( idxGt2Flag < MaxNumGt2Flag ) { |  |
| if (AbsLevel[ xC ][ yC ] > 4 ) { |  |
| abs_remainder[ n ] |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| } |  |
| if ( AbsLevel[ xC ][ yC ] > 2) { |  |
| idxGt2Flag++ |  |
| } |  |
| } |  |
| else if ( AbsLevel[ xC ][ yC ] > 2) { |  |
| abs_remainder[ n ] |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| } |  |
| } |  |
| if( dep_quant_enabled_flag ¦ ¦ !sign_data_hiding_enabled_flag ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !signHidden ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( dep_quant_enabled_flag ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ par_level_flag[ n ] ] |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHidden ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( ( n = = firstSigScanPosSb ) && ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( cu_mts_flag[ x0 ][ y0 ] && ( cIdx = = 0 ) && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && numSigCoeff > 2 ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) ) ) { |  |
| mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |  |

# Results

## VTM2 standard configuration results

The following results are generated with the CTC configuration files.

### Computation reduction of significance map context index

### Limiting the number of rem_abs_gt2_flag (greater than 4 flags) to 2

### Computation reduction of significance map context index and limiting the number of rem_abs_gt2_flag (greater than 4 flags) to 2

### Computation reduction of significance map context index and limiting the number of rem_abs_gt2_flag (greater than 4 flags) to 3

# Reference

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “Non-CE7: Alternative Entropy Coding for Dependent Quantization”, JVET-K0072-v2, 11th JVET Meeting, Ljubljana, SI, 10–18 July, 2018.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2)”, JVET-K1001-v4, 11th JVET Meeting, Ljubljana, SI, 10–18 July, 2018.

- J. Chen, Y. Ye, S.H. Kim, “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2)”, JVET-K1002-v1, 11th JVET Meeting, Ljubljana, SI, 10–18 July, 2018.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

# Patent rights declaration(s)

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).