| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0406 |
| --- | --- |

| Title: | CE14: Test on in-loop bilateral filter from JVET-J0021/JVET-K0384 with parametrization (CE14.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Nikolay Shlyakhov Marta Karczewicz | Tel: Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution report results of CE14.2 test on in-loop Bilateral Filtering (BIF) method that was proposed in the JVET-J0021 and latter tested in JVET-K0384. In this contribution, the simplified, parametrized version of the original BIF [2], [3] is tested. It is proposed to represent the weighting functions of the BIF with a piece-wise linear model. This drastically reduces memory requirements (size of the stored LUT) from 30KB in [2] to 120 Bytes in the current implementation and can enable a low complexity LUT-free implementation of the BIF. It is also proposed to skip processing of the 4x4 blocks, thus further reducing computation complexity of the proposed BIF.

Proposed BIF was integrated in the VTM2.01 and tested in two configurations: CE14.2.a tests BIF with 10-ranges PWL model and CE14.2.b tests the reduced complexity of the PWL to 2 non-zero ranges, with its parameters being adapted to QP value and block partitioning. Filtering of 4x4 blocks was omitted in both tests. The following BD rate results comparing to the VTM2.0.1 CTC are reported in this document:

CE14.2.a: AI : -0.43%	0.20%	0.18%; RA: -0.60%	-0.12%	-0.07%; LDB: -0.77%	0.31%	0.22%

CE14.2.b: AI : -0.42%	0.13%	0.16%; RA: -0.59%	-0.13%	-0.11%; LDB: -0.72%	0.23%	0.15%

# Introduction

The bilateral filter is an edge-preserving filter that takes into account the geometric distances between pixels as well as the intensity differences between them. By considering intensity differences between pixels, the bilateral filter prevents blurring of edges in an image during filtering.

This contribution reports results of CE14.2 test on in-loop Bilateral Filtering method that was proposed in the JVET-J0021 and integrated to the VTM2.0.1 code base with simplifications. This method was earlier tested in VTM1.0 and BMS1.0 code bases and those results were provided in document JVET-K00384.

Proposed bilateral filter is a 5-tap bilateral filter which is applied on the reconstructed block when the transform has non-zero coefficients and QP is larger than 17. The strength of the filter is controlled by QP and the center weight of the filter.

# Proposed technology

The proposed bilateral filter is always applied to luma blocks with non-zero transform coefficients and slice quantization parameter larger than 17. Therefore, there is no need to signal the usage of the bilateral filter. Bilateral filter, if applied, is performed on decoded samples right after the inverse transform. In addition, the filter parameters, i.e., weights are explicitly derived from the coded information.

The filtering process is defined as:

P0,  0'=P0, 0+k=1KWkPk, 0-P0, 0×Pk, 0-P0, 0,			(1)

where P0,  0 is the intensity of the current sample and P0, 0' is the modified intensity of the current sample, Pk,  0 and Wk are the intensity and weighting parameter for the k-th neighboring sample, respectively. An example of one current sample and its four neighboring samples (i.e., K=4) is depicted in Figure 1.

Figure 1: Neighboring samples utilized in bilateral filter.

More specifically, the weight Wk(x) associated with the k-th neighboring sample is defined as follows:

Wk(x)=Distancek× Rangek(x) 					(2)

wherein

Distancek=e- 100002σd21+4*e- 100002σd2,				(3)

Rangekx=e– x28*QP – 17*(QP – 17) 						(4)

and σd is dependent on the coded mode and coding block sizes. The described filtering process is applied to intra-coded blocks, and inter-coded blocks when TU is further split, to enable parallel processing.

To better capture statistical properties of video signal, and improve performance of the filter, weights function resulted from Equation (2) are being adjusted by the σd parameter, tabulated in Table 1 as being dependent on coding mode and parameters of block partitioning (minimal size).

Table 1 Value of σd for different block sizes and coding modes

| Min(block width, block height) | Inter mode | Intra mode |
| --- | --- | --- |
| 4 | 62 | 82 |
| 8 | 52 | 72 |
| Other | 32 | 52 |

To further improve the coding performance, for inter-coded blocks when TU is not split, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference between two windows covering current sample and the neighboring sample. Therefore, the equation of filtering process is revised to:

P0,  0'=P0, 0+k=1NWk1Mm=-M/2M/2abs(Pk, m-P0, m)×(Pk, 0-P0, 0)		(4)

wherein Pk, m and P0, m represent the m-th sample value within the windows centered at  Pk,  0 and P0,  0, respectively. In this proposal, the window size is set to 3×3. An example of two windows covering P2,  0 and P0,  0 are depicted in Figure 2.

Figure 2: Windows covering two samples utilized in weight calculation.

## Parametrization of the weighting functions:

Enabling weights construction as specified in Eq. 2-4, produces a family of weighted functions consisting of 270 functions or variable length. Figure 3 visualize resulting functions for a set of QP values:

Figure 3. Example of Wk(x) as function of x (shown on horizontal axis) and QP values

This family of weighting functions Wkx, QP, mode produced from Eq. 2-4 can be parameterized by a piece-wise linear model from a basis function W0(x), which is coarsely represented with a small number of pivot points W0 and derivation scaler for QP Sqp and mode Sm.

Below are provided an example of PWL model parameters being tested in CE14.2.a, with full family of W(x) being compactly represented with 120 bytes:

W0 = {42950, 37903, 26050, 13944, 5813, 1887, 477, 94, 14, 2}

Sm = { 30195, 19316, 26068, 19316, 1470 };

Sqp = { 65535, 43690, 32768, 26214, 21845, 18724, 16384, 14563, 13107, 11915, 10922, 10082, 9362, 8738, 8192, 7710, 7281, 6898, 6553, 6241, 5957, 5698, 5461, 5242, 5041, 4854, 4681, 4519, 4369, 4228, 4096, 3971, 3855, 3744, 3640, 3542, 3449, 3360, 3276, 3196, 3120, 3048, 2978, 2912, 2849};

The process of W(x, Qp, mode) derivation utilize a piece-wise linear interpolation between j pivot points specified by W0 as following:

W[j] = W0[j]*Sm >> 15

d = (W[j+1] - W[j]))*Sqp >> 17

W(x) = max(W[j+1], W[j] +(x - x[j+1]) )*d);

This approach allows to achieve a high accuracy approximation of the weight function W(x,Qp, mode), as shown in Figure 4 below:

Figure 4. Results of approximation of a weight function W(x,Qp,mode) with proposed 10-ranges PWL model

In the test CE14.2.b, a further reduced complexity PWL is utilized to reconstruct the family functions W(x,Qp, mode). Total number of two ranges are defied for PWL, with x value separating two ranges are selected to be power of 2, thus can be represented with 1<<topShift. Under these constrains a reduced complexity derivation process can be employed:

With W(Qp, mode, x) being represented with 2 ranges, thus 3 pivot points W[3]:

d[0] = (W[1] - W[0]))*Sqp >> 17

d[1] = (W[2] - W[1]))*Sqp >> 17

For each x value, with x > 0 representing absolute difference of intensity values, the w(x) can be derived as following:

idx1 = x>>topShift;

idx2 = x - idx1;

W(x)= std::max(0, W0[idx1] + d[idx2] * idx2 );

The follow low complexity implementation can be utilized in a LUT-free implementation of the proposed BIF.

Proposed 2 ranges PWL with adaptive range sizes allows to achieve more coarse, however still sufficiently accurate approximation of the weighting function W(x), as shown in Figure 4 below:

Figure 4. Results of approximation of a weight function W(x,Qp,mode) with proposed 2-ranges PWL model

## Memory requirements and computational complexity:

Memory requirements for proposed parametrized BIF are estimated in 60 16 bits LUT entries, resulting in 120 Bytes of the storage. This estimate is compared to the earlier proposals in Table 3:

Table 2 Number of entries in LUTs used in proposed and JEM/BMS-1.0 bilateral filter

| Method | LUT for weights | LUTs for division |
| --- | --- | --- |
| JEM/BMS-1.0 | 3468 | 325 x 2 |
| BIF in K0384 [3] | 28795 | - |
| Proposed Parametrized | 60 | - |

Generation of set of weights for every QP, mode and block size is performed by piece-wise linear interpolation on 10 intervals, which may be implemented before the filtering process for CU. It may be implemented as generation of LUT of length 1024 and referencing to generated LUT as in previous proposals for weight derivation.

Weight derivation by piecewise linear process may be expressed as

d = ((pivot[j+1] – pivot[j])*k(QP)) >> 17;   //(negative slope is derived from tables by approximating reciprocal)

W(QP, mode, block size) = max(pivot[j+1], pivot[j] + pivotj- (Pk, 0-P0, 0) *d); // (linear interpolation).

Weight derivation may be simplified to piecewise linear interpolation on less number of intervals, in particular, on 1 or 2 intervals where weighting function is greater than zero.

The reduction in the total number of entries in the LUTs used in JEM/BMS-1.0 bilateral filter comes at the cost of increased computation. The number of addition, multiplication and bit-shit operations used in the filtering of one pixel in proposed bilateral filter (not considering the 3x3 window based bilateral filter used to filter inter blocks) and the bilateral filter in JEM/BMS-1.0 are summarized in the first column of the Table 3. (The numbers in Table 3 assume a sliding window implementation explained earlier). In this estimate, the LUT implementation is assumed. In the case of LUT-free implementation, the weight W(x, Qp, mode) need to be derived for each pixel, thus additional complexity for weighting function approximation is required. This additional complexity for LUT-free implementation will depend on the complexity of the model, Table 3 below provide complexity estimate in form of number of operations for two examples of PWL model: 2 ranges (as tested in CE12.2) and hypothetical single range approximation.

Table 3 Number of operations for weight derivation

|  | Proposed parametrized |  |  |  |
| --- | --- | --- | --- | --- |
|  | Filtering | Additional complexity in 2 ranges LUT-Free | Additional complexity in 1 ranges LUT-Free | JEM/BMS-1.0 |
| #additions | 7 | 2 | 2 | 8 |
| #multiplications | 2 | 2 | 1 | 3 |
| #bitshifts | 1 | 2 | 1 | 2 |
| #max |  | 1 | 1 | 2 |

# Experimental Results

The CE14.2 test was implemented on top of BMS2.01 and tested under VTM CTC. Two configurations of the proposed method was tested: CE14.2.a – proposed BIF is implemented with PWL employing 10 ranges (pieces), and CE14.a.b – proposed BIF is implemented with reduced complexity PWL employing 2 ranges. Filtering of 4x4 blocks was omitted in both tests. Simulation results for tested method are provided in Tables 3– 4. Note: more accurate time estimates will be provided in an updated version of the document.

## Sub test CE14.2.a

Table 2. CE14.2.a simulation results comparing to VTM2.01 CTC

## Sub test CE14.2.b

Table 2. CE14.2.b simulation results comparing to VTM2.01 CTC

# Conclusion

This contribution report results of CE14.2 test on parametrized in-loop Bilateral Filtering (BIF). It is proposed to represent the weighting functions of the BIF with a piece-wise linear model. This drastically reduces memory requirements (size of the stored LUT) from 30KB in [2] to around 120 Bytes in the current implementation and can enable a low complexity LUT-free implementation of the BIF. It is also proposed to skip processing of the 4x4 blocks, thus further reducing computation complexity of the proposed BIF.

Proposed BIF was integrated in the VTM2.01 and tested in two configurations: CE14.2.a tests BIF with 10-ranges PWL model and CE14.2.b tests the reduced complexity of the PWL to 2 non-zero ranges, with its parameters being adapted to QP value and block partitioning. Filtering of 4x4 blocks was omitted in both tests. The following BD rate results comparing to the VTM2.0.1 CTC are reported in this document:

It is proposed to adopt the proposed parametrized BIF method into the next working draft.

# Patent rights declaration(s)

Qualcomm Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Description of Core Experiment 11 (CE14): Deblocking”, JVET-K1034.

- “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, 10th JVET Meeting, San Diego, California, USA, April 2018, JVET-J0021.

- “CE2.1.3: In-loop bilateral filter”, 11th JVET Meeting, Ljubljana, Slovenia, July 2018, JVET-K0384.

- “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010.