| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0409-v1 |
| --- | --- |

| Title: | Non-CE2: Filter Coefficients simplification for filtering complexity reduction in ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Sethuraman Consulate-1, #1 Richmond Road Bangalore-560025 India | Tel: Email: | +91-80-66601000 sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_______`______________________

# Abstract

In this proposal, a method for normatively constraining the maximum number of ones in the binary representation of the absolute value of all the filter coefficients except the central coefficient in ALF is proposed with the aim of simplifying the complexity associated with the multiplications required for the non-separable 2-D filtering. Specifically, the number is constrained to a low value such as 2 or 3 so that the multiplication can be realized with just 2 or 3 shifts and 1 or 2 additions respectively. The proposal also presents results harmonized with an earlier proposal that constrained the absolute value of certain filter coefficients to be a power of 2. The results indicate that the approximation results in -0.14% luma coding gain drop for the 3-shift option and slightly higher drop for the 2-shift option.

# Introduction

At the July meeting, a complexity analysis of the ALF method adopted in VTM-2.0 was performed. It was concluded that the 13 multiplications per output sample was a critical complexity aspect for further study. JVET-K0215 had proposed a method for constraining the absolute value of the filter coefficients at a distance of more than 2 from the central coefficient to a power of 2 and had reported very minimal drop in BDRATE. With the max filter size of 7x7 adopted in VTM-2.0, less than half the multiplications only are replaced with shifts.

In this proposal, other methods of constraining the filter coefficient values are studied with the aim of replacing all but the multiplication by the central coefficient with a low number of shifts, while evaluating the impact of solving the set of equations only once.

In addition, the proposed method is harmonized with the method of JVET-K0215 so that the absolute value of 6 coefficients (out of 13) are constrained to a power of 2, while the absolute value of another 6 coefficients are constrained using the proposed method. The results with and without solving the set of equations more than once are presented.

# Proposed algorithms

The proposed algorithm normatively constrains the maximum number of ones in the binary representation of the absolute value of the filter coefficient to a low value such as 3 or 2 for all filter coefficients except the central coefficient (which is derived at the decoder). Whenever, the binary representation of the absolute value of a coefficient has more than the specified number of ones, a correction equal to half of the value that would have been lost, if only the first 3 ones from the most-significant bit position are retained, is added to the absolute value. After this correction, only the first 3 non-zero bit positions are retained and the remaining bit positions are set to zero. The filter coefficient value is obtained by attaching the sign of the original coefficient to the modified absolute value.

From a signaling point of view, no changes are made. The modified filter values are coded in the bit-stream using the existing method adopted in VTM-2.0. Since the signaling overheads with temporal prediction are already quite a small percentage of the total bit-rate, the scope for signaling overhead reduction by leveraging the constraint appears minimal.

Since the central coefficient is derived on the decoding side, all the errors incurred on the other coefficients get applied to the central position.

To further reduce the multiplication complexity, the proposed method is harmonized with the JVET-K0215 method so that absolute values of 6 coefficients (out of 13) are constrained to be a power of 2, while 6 other coefficients which are at a distance of less than 6 from the central coefficient are constrained using the proposed method.

# Complexity analysis

The currently adopted filter coefficients are 9-bits in precision. Hence, each output sample (with the half-symmetry) requires 13 10-bit x 9-bit multiplications for the 7x7 diamond filter shape.

With the proposed constraint, the multiplications for all but the central coefficient can be realized using just 2 or 3 shifts (and 1 or 2 additions). When compared to the 10-bit x 9-bit multiplication which requires 9 shifts and 8 additions, the proposed constraint offers a substantial reduction in the hardware requirements for the multiplier.

It should be noted that each coefficient can use completely different set of shift values. Many CPU architectures (e.g. ARM and x86-AVX3) have SIMD instructions that support different shift values on the different SIMD lanes. Hence, having different shift values across the coefficients does not pose any SIMD limitation.

Since number of combinations out of 9-bits for 2 or 3 positions only to be 1 is limited, a look-up table can be designed to get the 2 or 3 shifts from the absolute value of the coefficient.

With the harmonized method with K0215, 6 multiplications get replaced by a single shift, while 6 other multiplications get replaced by 2 or 3 shifts (and 1 or 2 additions).

# Test Results

The test results are presented by comparing the ALF with the proposed changes against VTM-2.0. Fixed filters are disabled during the runs.

The following table shows the BDRATE results for the different cases tested. (Runs against VTM-2.0 are underway. Results shown here are with VTM-1.0 with ALF adopted in VTM-2.0)

| Test-cases | ALF Tool-ON BD-RATE in VTM-1.0 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 3-shifts for 12-coeffs | -4.84% | -3.08% | -2.50% |
| 2-shifts for 12 coeffs | -4.72% | -2.96% | -2.42% |
| 1 shift for 12 coeffs | -4.27% | -2.70% | -2.26% |
| 6 with 1-shift + 6 with 3-shifts (without re-computation between groups) | -4.59% | -2.83% | -2.16% |
| 6 with 1-shift + 6 with 3-shifts (with re-computation between groups) | -4.83% | -3.05% | -2.50% |

Other than the last row in the table, all other results are without solving the set of equations for the filter coefficients more than once.

# Summary

The proposed normative constraint on the filter coefficients results in a substantial reduction in the multiplication complexity in ALF. The drop in BD-RATE for the 3-shifts constraint is <0.1% when compared to the ALF adopted in VTM-2.0 (with fixed filters disabled). It is requested that the proposed method be considered for further study in ALF core experiments.

# References

- S. Esenlik, Z. Zhao, J. Chen, “CE2: ALF with Multiplication Replaced by Bit-Shifting (Test 4.1.3)”, JVET-K0215, July 2018.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).