| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0410 |
| --- | --- |

| Title: | CE11-related: On deblocking tC table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin 100 Winchester Cir Los Gatos, CA-95032, USA | Tel: Email: | anorkin@netflix.com |
| Source: | Netflix |  |  |

_____________________________

# Abstract

The document argues that the values chosen during the last meeting to extend the deblocking tC table (as a consequence of extending the maximum QP value to 63) do not reflect the intent of the tC table design. The document proposes new values to extend the tC table that keep linear relation of tC with the quantization step size which is asserted to be the design intent for the deblocking tC table.

# Introduction

During the Lubljana meeting, contribution K0251 [1] has been adopted, which has proposed extension of the QP range in VVC video codec. After discussion at the meeting, it was decided to extend the QP range even further, to QP=64. Extension of the QP range also required extension of the deblocking tables (which derive values of deblocking parameters tC and beta based on the block QP) so that deblocking continued to be adaptive for this larger QP values. This extension of deblocking tables was done at the meeting, and supposedly linear extension of the values in HEVC tables was used.

This contribution argues that the extension of the deblocking tC table has not been done according to the design of this table and proposes different values for the extensions of the tC table. The proposed changes do not have any effect on the common test conditions [2].

# Proposed changes to tC table

The tC table is used for clipping (restricting) values of possible modifications done by the deblocking filtering. Parameters tC also used in certain cases to decide whether the strong deblocking filtering should be applied. The strongest block artifacts originate from the quantization effects introduced by the video codec. Therefore, the tC table is designed to have linear dependency with (being proportional to) the quantization step (Qs). As known, quantization step doubles every 6 QP values, thus being proportional to 2QP/6. Figure 1 shows the plot of the tC table values against the QP, including the extended valued proposed at the last meeting. Figure 2 shows the tC table vs 2QP/6 which is proportional to the quantization step size. You can see that the extended values proposed last meeting as part of [1] do not follow linear relationship with the quantization step size. It is proposed to modify the values in the tC table so that the linear relationship with the quantization step size is kept, as in Figure 2. The plot of proposed tC values vs QP is shown in Figure 3. You can see that proposed values of tC have exponential relationships with QP values, which was the intention when designing the tC table.

Figure 1. tC values (including extension from K0251) relative to QP

Figure 2. tC values relative to 2QP/6. Note that the proposed values have linear relationship
 between tC and the quantization step.

Figure 3. tC values relative QP. The proposed values have exponential relationship between the tC and QP (as intended).

The proposed values to the tC table are thus as follows:

tC = [ 0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  0  1  1  1  1  1  1  1  1  1  2  2  2  2  3  3  3  3  4  4  4  5  5  6  6  7  8  9 10 11 13 14 16 18 20 22 25 28 31 35 39 44 50 56 63 70 79 88 99]

Replaced values are highlighted with yellow.

The effect of proposed modifications on the subjective quality is hard to evaluate since video quality at the extended range of QPs is outside of the useful range for a significant part of content (according to author’s opinion).

# Proposed changes to the draft text

The deblocking filter is not part of the draft specification text yet, therefore, the contribution does not provide the specification changes since there is nothing to change in the specification.

# Conclusions

It is proposed to adopt the proposed changes to the deblocking tC table to the VTM software and to the draft specification text, when the deblocking filtering specification text is available.

# References

- S.-T. Hsiang, S.-M. Lei, CE7.3.2: Extension of quantization parameter value range, JVET-K0251, Ljubljana, July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video", JVET-K1010, Ljubljana, July 2018.

# Patent rights declaration(s)

Netflix does not have any current or pending patent rights relating to the technology described in this contribution.