|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | H.265 |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2018) |
|  | SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS Infrastructure of audiovisual services – Coding of moving video |  |  |  |
|  | High efficiency video coding |  |  |  |
|  | Recommendation ITU-T H.265 |  |  |  |

# Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply:

  - decoded picture buffer (DPB): A buffer holding decoded pictures for reference, output reordering, or output delay

  - long-term reference picture: A picture that is marked as "used for long-term reference".

  - long-term reference picture set: The two reference picture set (RPS) lists that may contain long-term reference pictures.

  - reference picture: A picture that is a short-term reference picture or a long-term reference picture.

NOTE – A reference picture contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

  - reference picture set (RPS): A set of reference pictures associated with a picture, consisting of all reference pictures that are prior to the associated picture in decoding order, that may be used for inter prediction of the associated picture or any picture following the associated picture in decoding order.

NOTE – The RPS of a picture consists of five RPS lists, three of which are to contain short-term reference pictures and the other two are to contain long-term reference pictures.

  - short-term reference picture set: The three RPS lists that may contain short-term reference pictures.

#### Sequence parameter set RBSP syntax

##### General sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| st_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| ... |  |
| } |  |

    - Slice segment header syntax

#### General slice segment header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nal_unit_type != IRAP_NUT) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| st_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_msb_lt[ i ] | ue(v) |
| delta_poc_lsb_lt_minus1[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

### Short-term reference picture set syntax

| st_ref_pic_set( stRpsIdx ) { | Descriptor |
| --- | --- |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

#### Sequence parameter set RBSP semantics

##### General sequence parameter set RBSP semantics

num_short_term_ref_pic_sets specifies the number of st_ref_pic_set( ) syntax structures included in the SPS. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE 5 – A decoder should allocate memory for a total number of num_short_term_ref_pic_sets + 1 st_ref_pic_set( ) syntax structures since there may be a st_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture. A st_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture has an index equal to num_short_term_ref_pic_sets.

long_term_ref_pics_present_flag equal to 0 specifies that no long-term reference picture is used for inter prediction of any coded picture in the CVS. long_term_ref_pics_present_flag equal to 1 specifies that long-term reference pictures may be used for inter prediction of one or more coded pictures in the CVS.

### Slice segment header semantics

#### General slice segment header semantics

When present, the value of the slice segment header syntax elements slice_pic_parameter_set_id, slice_pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx and num_long_term_pics shall be the same in all slice segment headers of a coded picture. When present, the value of the slice segment header syntax elements delta_poc_msb_lt[ i ], delta_poc_lsb_lt_minus1[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice segment headers of a coded picture for each possible value of i.

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term RPS of the current picture is derived based on one of the st_ref_pic_set( ) syntax structures in the active SPS that is identified by the syntax element short_term_ref_pic_set_idx in the slice header. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term RPS of the current picture is derived based on the st_ref_pic_set( ) syntax structure that is directly included in the slice headers of the current picture. When num_short_term_ref_pic_sets is equal to 0, the value of short_term_ref_pic_set_sps_flag shall be equal to 0.

short_term_ref_pic_set_idx specifies the index, into the list of the st_ref_pic_set( ) syntax structures included in the active SPS, of the st_ref_pic_set( ) syntax structure that is used for derivation of the short-term RPS of the current picture. The syntax element short_term_ref_pic_set_idx is represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. When not present, the value of short_term_ref_pic_set_idx is inferred to be equal to 0. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive.

The variable CurrRpsIdx is derived as follows:

–	If short_term_ref_pic_set_sps_flag is equal to 1, CurrRpsIdx is set equal to short_term_ref_pic_set_idx.

–	Otherwise, CurrRpsIdx is set equal to num_short_term_ref_pic_sets.

num_long_term_pics specifies the number of entries in the long-term RPS of the current picture that are directly signalled in the slice header. When not present, the value of num_long_term_pics is inferred to be equal to 0.

delta_poc_msb_lt[ i ] specifies the difference between the picture order count values of the current picture and i-th entry in the long-term RPS of the current picture divided by MaxPicOrderCntLsb.

delta_poc_lsb_lt_minus1[ i ] plus 1 specifies the difference between the picture order count values of the current picture and i-th entry in the the long-term RPS of the current picture modulo MaxPicOrderCntLsb. The length of the delta_poc_lsb_lt_minus1[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th entry in the long-term RPS of the current picture is not used for reference by the current picture.

The variables DeltaPocLt[ i ] and UsedByCurrPicLt[ i ] are derived as follows:

–	DeltaPocLt[ i ] is set equal to delta_poc_msb_lt[ i ] * MaxPicOrderCntLsb + delta_poc_lsb_lt_minus1[ i ] + 1 and UsedByCurrPicLt[ i ] is set equal to used_by_curr_pic_lt_flag[ i ].

It is a requirement of bitstream conformance that DeltaPocLt[  i ] is not equal to DeltaPocLt[  j ] for any i and j in the range of 0 to num_long_term_pics – 1, inclusive, unless i is equal to j.

-

-

### Short-term reference picture set semantics

The st_ref_pic_set( stRpsIdx ) syntax structure may be present in an SPS or in a slice header. Depending on whether the syntax structure is included in a slice header or an SPS, the following applies:

–	If present in a slice header, the st_ref_pic_set( stRpsIdx ) syntax structure specifies the short-term RPS of the current picture (the picture containing the slice), and the following applies:

–	The content of the st_ref_pic_set( stRpsIdx ) syntax structure shall be the same in all slice headers of the current picture.

–	The value of stRpsIdx shall be equal to the syntax element num_short_term_ref_pic_sets in the active SPS.

–	The short-term RPS of the current picture is also referred to as the num_short_term_ref_pic_sets-th candidate short-term RPS in the semantics specified in the remainder of this clause.

–	Otherwise (present in an SPS), the st_ref_pic_set( stRpsIdx ) syntax structure specifies a candidate short-term RPS, and the term "the current picture" in the semantics specified in the remainder of this clause refers to each picture that has short_term_ref_pic_set_idx equal to stRpsIdx in a CVS that has the SPS as the active SPS.

-

-

-

-

-

num_negative_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture.

num_positive_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values greater than the picture order count value of the current picture.

delta_poc_s0_minus1[ i ] plus 1, when i is equal to 0, specifies the difference between the picture order count values of the current picture and i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value less than that of the current picture, or, when i is greater than 0, specifies the difference between the picture order count values of the ( i − 1 )-th entry and the i-th entry in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture. The value of delta_poc_s0_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

used_by_curr_pic_s0_flag[ i ] equal to 0 specifies that the i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value less than that of the current picture is not used for reference by the current picture.

delta_poc_s1_minus1[ i ] plus 1, when i is equal to 0, specifies the difference between the picture order count values of the current picture and the i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value greater than that of the current picture, or, when i is greater than 0, specifies the difference between the picture order count values of the i-th entry and the ( i − 1 )-th entry in the current candidate short-term RPS that have picture order count values greater than the picture order count value of the current picture. The value of delta_poc_s1_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

used_by_curr_pic_s1_flag[ i ] equal to 0 specifies that the i-th entry in the current candidate short-term RPS that has picture order count value greater than that of the current picture is not used for reference by the current picture.

The variables NumNegativePics[ stRpsIdx ], NumPositivePics[ stRpsIdx ], UsedByCurrPicS0[ stRpsIdx ][ i ], UsedByCurrPicS1[ stRpsIdx ][ i ], DeltaPocS0[ stRpsIdx ][ i ] and DeltaPocS1[ stRpsIdx ][ i ] are derived as follows:

NumNegativePics[ stRpsIdx ] = num_negative_pics		(7-63)

NumPositivePics[ stRpsIdx ] = num_positive_pics		(7-64)

UsedByCurrPicS0[ stRpsIdx ][ i ] = used_by_curr_pic_s0_flag[ i ]	(7-65)

UsedByCurrPicS1[ stRpsIdx ][ i ] = used_by_curr_pic_s1_flag[ i ]	(7-66)

–	If i is equal to 0, the following applies:

DeltaPocS0[ stRpsIdx ][ i ] = −( delta_poc_s0_minus1[ i ] + 1 )	(7-67)

DeltaPocS1[ stRpsIdx ][ i ] = delta_poc_s1_minus1[ i ] + 1	(7-68)

–	Otherwise, the following applies:

DeltaPocS0[ stRpsIdx ][ i ] = DeltaPocS0[ stRpsIdx ][ i − 1 ] − ( delta_poc_s0_minus1[ i ] + 1 )	(7-69)

DeltaPocS1[ stRpsIdx ][ i ] = DeltaPocS1[ stRpsIdx ][ i − 1 ] + ( delta_poc_s1_minus1[ i ] + 1 )	(7-70)

-

It is a requirement of bitstream conformance that DeltaPocS0[ stRpsIdx ][ i ] is not equal to DeltaPocLt[ j ], for any j in the range of 0 to num_long_term_pics, inclusive.

# Decoding process

## General decoding process

## NAL unit decoding process

## Slice decoding process

### Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit. This process may result in one or more reference pictures in the DPB being marked as "unused for reference" or "used for long-term reference".

NOTE 1 – The RPS is an absolute description of the reference pictures used in the decoding process of the current and future coded pictures. The RPS signalling is explicit in the sense that all reference pictures included in the RPS are listed explicitly.

A decoded picture in the DPB can be marked as "unused for reference", "used for short-term reference" or "used for long-term reference", but only one among these three at any given moment during the operation of the decoding process. Assigning one of these markings to a picture implicitly removes another of these markings when applicable. When a picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both).

When the current picture is an IRAP picture, all reference pictures currently in the DPB (if any) are marked as "unused for reference".

Short-term and long-term reference pictures are identified by their PicOrderCntVal values.

Five lists of picture order count values are constructed to derive the RPS. These five lists are PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr and PocLtFoll, with NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr and NumPocLtFoll number of elements, respectively. The five lists and the five variables are derived as follows:

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr and PocLtFoll are all set to be empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr and NumPocLtFoll are all set equal to 0.

- Otherwise, the following applies:

for( i = 0, j = 0, k = 0; i < NumNegativePics[ CurrRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ CurrRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ CurrRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ CurrRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(8-5)
for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ ) {
	
	
		
				
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j
		++ ] = PicOrderCntVal - DeltaPocLt[ i ]
	} else {
		PocLtFoll[ k
		++ ] = PicOrderCntVal - DeltaPocLt[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in clause 8.3.1.

NOTE 2 – A value of CurrRpsIdx in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a candidate short-term RPS from the active SPS for the current layer is being used, where CurrRpsIdx is the index of the candidate short-term RPS into the list of candidate short-term RPSs signalled in the active SPS for the current layer. CurrRpsIdx equal to num_short_term_ref_pic_sets indicates that the short-term RPS of the current picture is directly signalled in the slice header.

It is a requirement of bitstream conformance that the following applies to the value of NumPicTotalCurr:

- When the current picture contains a P or B slice, the value of NumPicTotalCurr shall not be equal to 0.

The RPS of the current picture consists of five RPS lists; RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetStFoll, RefPicSetLtCurr and RefPicSetLtFoll. RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetStFoll are collectively referred to as the short-term RPS. RefPicSetLtCurr and RefPicSetLtFoll are collectively referred to as the long-term RPS.

NOTE 3 – RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr contain all reference pictures that may be used for inter prediction of the current picture and one or more pictures that follow the current picture in decoding order. RefPicSetStFoll and RefPicSetLtFoll consist of all reference pictures that are not used for inter prediction of the current picture but may be used in inter prediction for one or more pictures that follow the current picture in decoding order.

The derivation process for the RPS and picture marking are performed according to the following ordered steps:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ )
	if( 
		there is a reference picture picX in the DPB with PicOrderCntVal
							 equal to PocLtCurr[ i ] 
			)
		RefPicSetLtCurr[ i ] = picX
		else
			RefPicSetLtCurr[ i ] = "no reference picture"
	
		
							
			
		
																											(8-6)
for( i = 0; i < NumPocLtFoll; i++ )
	if( 
		there is a reference picture picX in the DPB with PicOrderCntVal
							 equal to PocLtFoll[ i ] 
	)
		RefPicSetLtFoll[ i ] = picX
		
			
	
		
							
			
		
			else
		RefPicSetLtFoll[ i ] = "no reference picture"

- All reference pictures that are included in RefPicSetLtCurr or RefPicSetLtFoll are marked as "used for long-term reference".

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrBefore[ i ] )
		RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrAfter[ i ] )
		RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(8-7)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStFoll[ i ] )
		RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures in the DPB that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetStFoll are marked as "unused for reference".

NOTE 4 – There may be one or more entries in the RPS lists that are equal to "no reference picture" because the corresponding pictures are not present in the DPB. Entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr that is equal to "no reference picture".

NOTE 5 – A picture cannot be included in more than one of the five RPS lists.

It is a requirement of bitstream conformance that the RPS is restricted as follows:

- There shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr for which one or more of the following are true:

  - The entry is equal to "no reference picture".

  - The entry is a picture that has TemporalId greater than that of the current picture.

- There shall be no entry in RefPicSetLtCurr or RefPicSetLtFoll for which the difference between the picture order count value of the current picture and the picture order count value of the entry is greater than or equal to 224.

____________________