| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0416-v2 |
| --- | --- |

| Title: | Simplified RPS for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |  |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution it is proposed that a minimal reference picture set (RPS) is adopted into the VVC draft as a starting point. The proposal suggests that text based on the HEVC v5 RPS text is used, with removal of inter-RPS, HEVC long-term pictures and text related to syntax elements not present in the current VVC draft. In a second version of this proposal, a new long-term picture referencing mechanism is proposed.

Text from the following parts of the HEVC v5 specification is proposed to be included into the VVC draft:

- Definitions: Add definitions of DPB, long-term reference picture, reference picture, reference picture set (RPS) and short-term reference picture set

- SPS: Add syntax and semantics for 2 RPS SPS syntax element (there are 6 syntax elements related to RPS in HEVC)

- PPS: Add syntax and semantics for 3 reference picture lists syntax elements (3 in HEVC)

- Slice header: Add syntax and semantics for 9 slice header syntax elements (12 in HEVC)

- Reference picture lists modification: Add syntax and semantics for 4 syntax elements (4 in HEVC)

- RPS: Add syntax and semantics for 6 syntax elements that can be present in SPS and/or slice header (12 in HEVC)

- Decoding process: Add the decoding processes for picture (general decoding process), NAL units, reference picture set and reference picture lists

The proposed text was provided as an attachment with the contribution. Note that text proposed for picture order count is proposed in JVET-L0249.

The proponents believe inter-RPS is beneficial for reducing the bit cost, but do not propose to include it into the VVC draft at this point. The authors propose to keep inter-RPS in the software for further study.

The following restriction flags for turning off RPS features are identified by the proponents as candidates to be discussed:

- long_term_reference_pictures_disable_flag

- reference_picture_lists_modifications_disable_flag

- inter_RPS_disable_flag

# Introduction

Reference pictures in HEVC are handled by the so-called reference picture set (RPS). The RPS is a set of reference picture indicators and is present in each slice header in HEVC (except for IDR picture slices). This means that the decoder before decoding each slice can check whether the states of the reference pictures in the decoder are in sync with the encoder or not. The RPS in HEVC is also used for marking pictures as one of "unused for reference", "used for short-term reference" or "used for long-term reference".

# Benefits of RPS and comparison with MMCO

We believe that RPS provides the following benefits (and that MMCO does not provide any of these):

- Practically, any reference picture loss is detected in any bitstream

- Reference picture loss is detected at decoding of the very next picture after a loss

- Reference picture loss detection is done in the slice header and never during CABAC decoding

- The identity (POC or frame_num) of any lost reference picture will be known to the decoder

- Any picture reference structure can be used with minimum number of reference frames

- Reference picture issues can be determined easily when analyzing corrupt bitstreams

The authors further claim that the RPS design in well tested and familiar to implementors.

| Property | RPS | MMCO |
| --- | --- | --- |
| Virtually any reference picture loss is detected in any bitstream* | Yes | No |
| Reference picture loss is detected at decoding of the very next picture after a loss | Yes | No |
| Reference picture loss detection is done in the slice header and never during CABAC decoding | Yes | No |
| The identity (POC or frame_num) of any lost reference picture will be known to the decoder | Yes | No |
| Any picture reference structure can be used with minimum number of reference frames** | Yes | No |
| Reference picture issues can be determined easily when analyzing corrupt bitstreams | Yes | No |
| Design is well tested and familiar to implementors | Yes | Yes |

* One example of undetected reference picture loss in HEVC is if exactly all pictures in an entire POC cycle is lost and the encoder uses identical POC lsb values and picture reference structure for the first lost picture and the first picture that is received after the loss.

** See example in Annex A in this document

# Proposal

As a starting point in VVC, we propose a simplified RPS where long-term pictures are modified and Inter-RPS is removed from the HEVC design.

The syntax changes shown here is relative to HEVC. Proposed added text is shown by red text and removed text is shown by strikethrough red text. As shown, the RPS syntax is simplified significantly.

The proposed text is provided as an attachment with the contribution. Text with proposed long-term pictures is provided as v2 attachments and text without long-term pictures is provided as a v1 attachment.

The v2 version also contains syntax, semantics, and a process for reference picture lists. Decoding processes for NAL units and pictures (a general decoding process) are also included.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| st_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| ... |  |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nal_unit_type != IRAP_NUTIDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| st_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| delta_poc_msb_lt[ i ] | ue(v) |
| delta_poc_lsb_lt_minus1[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

| st_ref_pic_set( stRpsIdx ) { | Descriptor |
| --- | --- |
| if( stRpsIdx != 0 ) |  |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| if( stRpsIdx = = num_short_term_ref_pic_sets ) |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RefRpsIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| } else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

# Annex A – Example picture reference structure

The figure below shows an example reference structure that can not be used with MMCO at a minimum number of reference frames.

This reference picture structure uses 3 reference pictures and 1 current picture. Each picture only uses 2 reference pictures for inter prediction, but each picture must hold one extra picture from the other temporal layer that will be used for prediction by the next picture.

In order to have picture 0 and picture 2 available when decoding picture 4, picture 3 must have an MMCO command marking picture 1 as unavailable. Without MMCO commands, sliding window is used, and then picture 0 would be removed and picture 1 would be kept.

Now, if temporal layer 1 is removed (for example by a network node) there will be gaps in frame_num for all odd numbered pictures. “Non-existing” pictures will be created for these pictures and the sliding window process will be applied. That will result in having the “non-existing” picture 3 marking picture 0 unavailable. Picture 0 will then not be available for prediction when picture 4 is decoded. Since the encoder cannot make the decoding process be the same for the two cases 1) when all picture are decoded and 2) when only the lowest layer is decoded, the example coding structure cannot be used for temporal scalability with 3 reference pictures. The problem is caused by MMCO commands in a higher layer and that higher layer is removed, since then sliding window is assumed for the missing pictures. It is not possible for picture 4 no know what MMCO commands picture 3 had if picture 3 is not received.

It can be noted that the SVC specification text states that:

“With currTId being the value of temporal_id for the current access unit, the bitstream shall not contain data that result in the marking of pictures with temporal_id less currTId as "unused for reference" during this invocation of the process in subclause G.8.2.4.4.” Where G.8.2.4.4. is the “SVC adaptive memory control decoded reference picture marking process” section.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).