| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0417 |
| --- | --- |

| Title: | CE10: Results on Geometric Partitioning (Experiments 3.2.a – 3.2.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Max Bläser Johannes Sauer | Tel: Email: | +49 241 80 27677 blaeser@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This document relates to CE10 sub-experiment 3.2 on geometric block partitioning (GEO). It reports the results of geometric block partitioning as an additional coding tool, proposed in document JVET-J0023 and implemented into the BMS-2.0.1 software. According to the CE10 description given in JVET-K0144, the major difference to the previous experiment on GEO is the restriction to uni-directional motion compensation for inter-predicted GEO partitions. Three different configurations for GEO were tested. The reported BD-rate results are as follows:

- Geometric partitioning with only diagonal partitions enabled:

  - Compared to the VTM-2.0.1 reference, VTM-2.0.1 + GEO (Tool on) obtains an average Y BD-rate change of -0.04% for Random Access, and of -0.14% for Low Delay.

- Geometric partitioning with four “1/3rd – 2/3rd diagonal” partitions enabled:

  - Compared to the VTM-2.0.1 reference, VTM-2.0.1 + GEO (Tool on) obtains an average Y BD-rate change of -0.04% for Random Access, and of -0.15% for Low Delay.

- Geometric partitioning with no restrictions:

  - Compared to the VTM-2.0.1 reference, VTM-2.0.1 + GEO (Tool on) obtains an average Y BD-rate change of -0.10% for Random Access, and of -0.16% for Low Delay.

# Introduction

The experiment description is given in JVET-K0144. GEO was implemented on top of the QTBT-TT / MTT block coding structure. It is available as an additional coding tool for all block sizes larger or equal to 8 luma samples in width and height. The implementation follows the description given in JVET-J0023 closely: each segment of a GEO block may be inter- or intra-predicted. For inter-prediction, segment-based motion compensation is used with the HEVC related prediction and coding of motion vectors. In case of intra-predicted segments, a modified method of planar prediction is utilized as the single available intra mode.

In contrast to the previous experiment on GEO (JVET-K0146), bi-prediction is disabled for all sub-experiments. Thus, each GEO partition may only be inter-predicted uni-directionally. The three sub-experiments are distinguished by different restrictions on the flexibility of GEO:

Figure 1: Allowed GEO partitioning in sub-experiment 3.2.a (only diagonals).

In sub-experiment 3.2.a, GEO is configured to only allow a diagonal splitting of the block. No refinement coding of the partitioning is available. Geometric partitioning may be used for all blocks between the sizes 8×8 up to 128×128.

Figure 2: Allowed GEO partitioning in sub-experiment 3.2.b (four diagonals connecting the 1/3rd, 2/3rd positions).

In sub-experiment 3.2.b, more flexibility is allowed. GEO may use four different diagonal splits, which extent from the 1/3rd to the 2/3rd	positions and vice versa. No refinement coding of the partitioning is available. Geometric partitioning may be used for all blocks between the sizes 8×8 up to 128×128. Figure 2 exemplifies the four possible splitting options in case of square CUs / PUs.

Figure 3: Allowed GEO partitioning in sub-experiment 3.2.c (restrictions lifted for blocks larger than 16x16).

In sub-experiment 3.2.c, restrictions on GEO are mostly lifted. Refinement coding of the partitioning is available for blocks larger than 16×16. Smaller blocks may only use the diagonal templates. Geometric partitioning may be used for all blocks between the sizes 8×8 up to 128×128. Figure 3 exemplifies this setup in case of square CUs / PUs.

# Results

Note: In the previous version, BD coding results where reported with JEM_TOOLS macro on. However, crosschecks were performed with JEM_TOOLS macro off. Also, tool usage and bandwidth statistics were added to the reporting sheet. The impact on coding performance is minor.

Coding results have been obtained according to the Common test conditions given in JVET-K1010 and the methodology of JVET-K1005. Thus, GEO was tested in the following way:

- Tool on with VTM software against Tool off VTM reference, JEM_TOOLS macro off

## Sub-experiment 3.2.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.22% |  | -0.08% |  | 107% |  | 123% |  |
| Class A2 | -0.03% | -0.03% |  | -0.07% |  | 103% |  | 115% |  |
| Class B | 0.03% | -0.08% |  | 0.01% |  | 107% |  | 109% |  |
| Class C | -0.21% | -0.14% |  | -0.32% |  | 108% |  | 111% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.12% |  | -0.11% |  | 106% |  | 114% |  |
| Class D | -0.04% | -0.23% |  | -0.37% |  | 112% |  | 113% |  |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.03% | -0.17% | 108% | 108% |
| Class C | -0.10% | -0.22% | -0.08% | 113% | 110% |
| Class E | -0.02% | -0.07% | 0.01% | 108% | 111% |
| Overall | -0.14% | 0.04% | -0.07% | 108% | 110% |
| Class D | -0.37% | 0.39% | 0.12% | 110% | 111% |
| Class F |  |  |  |  |  |

## Sub-experiment 3.2.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.02% | -0.15% | 110% | 115% |
| Class A2 | -0.05% | -0.04% | 0.06% | 109% | 99% |
| Class B | 0.01% | -0.02% | -0.12% | 115% | 110% |
| Class C | -0.23% | -0.31% | -0.30% | 119% | 114% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.09% | -0.14% | 114% | 110% |
| Class D | -0.11% | -0.37% | -0.17% | 122% | 113% |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.04% | -0.19% | 115% | 108% |
| Class C | -0.10% | -0.41% | -0.10% | 122% | 114% |
| Class E | -0.08% | -0.31% | -0.11% | 125% | 113% |
| Overall | -0.15% | -0.15% | -0.19% | 116% | 111% |
| Class D | -0.36% | 0.02% | -0.30% | 128% | 114% |
| Class F |  |  |  |  |  |

## Sub-experiment 3.2.c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | -0.14% | 142% | 120% |
| Class A2 | -0.09% | -0.10% | -0.10% | 153% | 139% |
| Class B | -0.05% | -0.20% | -0.11% | 167% | 108% |
| Class C | -0.24% | -0.23% | -0.32% | 177% | 129% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.14% | -0.17% | 162% | 122% |
| Class D | -0.13% | -0.13% | -0.17% | 185% | 112% |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.01% | -0.39% | 206% | 121% |
| Class C | -0.14% | -0.28% | -0.29% | 230% | 143% |
| Class E | -0.10% | -0.43% | 0.47% | 213% | 124% |
| Overall | -0.16% | -0.21% | -0.36% | 220% |  | 133% |  |
| Class D | -0.39% | -0.49% | -0.38% | 225% | 126% |
| Class F |  |  |  |  |  |

## Evaluation

It is noted that in contrast to similar non-rectangular partitioning CE experiments 3.1 and 3.3, the encoder for all experiments 3.2.a – 3.2.c was configured to employ a fast GEO block skipping method which is based on a pixel variance calculation on the original. All blocks with σ ≤12.0∙ 2Bitdepth-8  are not tested for GEO. Further, GEO is disabled for all pictures of highest Tid (RA) / every second picture (LB). As the GEO mode usage (< 5% of all pixels affected by GEO) in this particular encoder setting is quite low, this might explain the low RD-impact of GEO.

# Acknowledgments

Simulations were performed with computing resources granted by RWTH Aachen University under project rwth0333.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.