| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0418-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0046 (CE4-related: On line buffer reduction for affine mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Huanbang Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of JVET-L0046 (CE4-related: On line buffer reduction for affine mode). The simulations were carried out based on the common test condition (CTC). The test results are matched for all points and the software matches the description in the proposal.

Introduction

This contribution JVET-L0046 proposes the following changes for the affine motion data inheritance from a neighboring PU located at the top CTU boundary, i.e.

- If the neighboring PU uses the 4-parameter model, or the neighboring PU has PU width 8, uses the 6-parameter model and bi-directional prediction, the 4-parameter model using the bottom-left and bottom-right control point motion vectors (CPMVs) is applied to the inheritance.

- Otherwise, if the neighboring PU uses the 6-parameter model and unidirectional prediction, the 6-parameter model using the bottom-left, bottom-right and top-left CPMVs is applied to the inheritance.

- Otherwise, if the neighboring PU uses the 6-parameter model and unidirectional prediction but its PU width is larger than 8, the 6-parameter model using the bottom-left, bottom-right CPMVs and bottom-middle two sub-block vectors is applied to the inheritance.

Simulation results

Table 1. Summary results of the proposed algorithm

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.07% | 99% | 101% |
| Class A2 | 0.08% | 0.00% | -0.03% | 100% | 101% |
| Class B | 0.02% | -0.02% | -0.03% | 99% | 100% |
| Class C | 0.00% | 0.11% | 0.06% | 98% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | -0.01% | 99% | 99% |
| Class D | 0.00% | 0.02% | -0.07% | 98% | 97% |
| Class F (optional) | -0.01% | -0.04% | -0.01% | 98% | 98% |
|  | Low Delay B Main 10 |  |  |  |  |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.11% | -0.30% | 100% | 103% |
| Class C | 0.04% | -0.08% | 0.04% | 99% | 103% |
| Class E | -0.06% | -0.12% | -0.51% | 100% | 101% |
| Overall | 0.01% | -0.10% | -0.24% | 100% | 102% |
| Class D | 0.00% | 0.33% | 0.23% | 99% | 101% |
| Class F (optional) | -0.04% | 0.56% | -0.03% | 100% | 101% |

Table 2. Summary results of the reference method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.14% | -0.04% | 100% | 102% |
| Class A2 | 0.02% | 0.01% | -0.03% | 100% | 101% |
| Class B | 0.02% | 0.02% | -0.08% | 99% | 101% |
| Class C | 0.01% | 0.14% | 0.08% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | -0.02% | 99% | 101% |
| Class D | 0.01% | -0.01% | -0.04% | 99% | 99% |
| Class F (optional) | -0.01% | 0.00% | 0.00% | 99% | 99% |
|  | Low Delay B Main 10 |  |  |  |  |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.02% | -0.15% | 100% | 102% |
| Class C | 0.03% | 0.05% | 0.01% | 99% | 103% |
| Class E | -0.02% | 0.01% | -0.34% | 100% | 102% |
| Overall | 0.01% | 0.01% | -0.15% | 100% | 102% |
| Class D | 0.06% | 0.43% | -0.40% | 99% | 101% |
| Class F (optional) | -0.04% | 0.21% | -0.05% | 99% | 101% |

References

- M. Zhou, “CE4-related: On line buffer reduction for affine mode”, JVET-L0046, Oct. 2018, Macao, CN.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.