| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0422 |
| --- | --- |

| Title: | CE13：In-loop filters disabled across face discontinuities and post-filtering of seam artifacts (Test 7.2.a) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuchang Huangfu Yule Sun Lu Yu | Tel: Email: | +86 57187953107 huangfuxc@zju.edu.cn sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This document reports experiment results of CE13.7.2.a. The proposed post-filtering method was implemented into 360Lib7.0 merged with BMS2.0.1. PHEC was set as anchor. Both anchor and post-filtering method were tested with VTM configuration. Results show that using HEC with in-loop filters disabled across face discontinuities and post-filtering can obtain an average gain of 0.59% for Luma component using E2EWSPSNR. The seam artifacts are significantly reduced after filtering.

Introduction

The detailed description of post-filtering method is given in JVET-K0183[1] and the experiment description is given in JVET-K1033[2]. Blocks located at discontinuous face boundaries are filtered using their spherical adjacent blocks. The filter we used is the same as deblocking filter in HEVC and the boundary strength (BS) is fixed to 1. 4 pixels near discontinuous face boundaries are filtered. In our experiment in-loop filters across face discontinuities were also disabled.

Results

Coding results of CE13.7.2.a

The experiment was tested according to the common test conditions for 360° video described in JVET-K1012[3]. The results are as follows:

| CE13.7.2a VS PHEC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -0.58% | -0.34% | 0.07% | -0.59% | -0.33% | 0.09% |
| Class S2 | -0.61% | -0.82% | -0.26% | -0.60% | -0.79% | -0.22% |
| Overall | -0.59% | -0.53% | -0.06% | -0.59% | -0.51% | -0.04% |

| CE13.7.2a VS HEC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -0.31% | -0.96% | -0.63% | -0.30% | -1.00% | -0.67% |
| Class S2 | -0.23% | -1.01% | -0.71% | -0.22% | -1.01% | -0.69% |
| Overall | -0.28% | -0.98% | -0.67% | -0.27% | -1.00% | -0.68% |

Class S1 includes 8K sequences and Class S2 includes 6K sequences.

Subjective results of CE13.7.2.a

ChairftRide, PHEC, Frame 30, QP 37

ChairftRide, CE13.7.2.a, Frame 30, QP 37

Trolley, PHEC, Frame 205, QP 37

Trolley, CE13.7.2.a, Frame 205, QP 37

Conclusion

In-loop filters disabled across face discontinuities and post-filtering can provide average gains of 0.59% for test sequences. From subjective results we can see that seam artifacts are significantly weakened using this method.

Reference

- X. Huangfu, Y. Sun, B. Wang, L. Yu, “AHG8：Face boundary filtering for 360° video,” JVET-K0183, July 2018.

- Philippe Hanhart, Jian-Liang Lin, Chirag Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” JVET-K1033, July 2018.

- P. Hanhart, J. Boyce, K. Choi, “Common test conditions and evaluation procedures for 360° video,” JVET-K1012, July 2018.

Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).