| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0423 |
| --- | --- |

| Title: | CE13-related: HEC with in-loop filters using spherical neighbors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuchang Huangfu Yule Sun Lu Yu | Tel: Email: | +86 57187953107 huangfuxc@zju.edu.cn sunyule@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

HEC with in-loop filters using spherical neighbors is studied in this document. Blocks located at discontinuous face boundaries are filtered using HEVC deblocking filter. SAO and ALF are disabled in face row discontinuous boundaries. Three cases of different boundary strength (BS) values are tested. There is no significant difference in coding performance between the three cases.

Introduction

As described in JVET-K1033 [1] blocks located at discontinuous face boundaries are filtered using their spherical adjacent blocks. HEVC deblocking filter is used and the BS is fixed to 1or 2. In this document, the coding performance of three cases of different boundary strength values is compared.

Case a:

BS is fixed to1;

Case b:

BS is fixed to2:

Case c:

BS is determined by the prediction modes of blocks:

Block P and block Q are blocks which are adjacent on the sphere.

In three cases, SAO and ALF are disabled in face row discontinuous boundaries.

Results and analysis

Coding performance of three cases

The experiment was tested according to the common test conditions for 360° video described in JVET-K1012[2] and PHEC is set as the anchor. The results are as follows:

|  | End-to-end WS-PSNR |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case a VS PHEC |  |  | Case b VS PHEC |  |  | Case c VS PHEC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class S1 | -0.58% | -0.29% | 0.07% | -0.59% | -0.44% | -0.08% | -0.57% | -0.39% | -0.03% |
| Class S2 | -0.59% | -0.84% | -0.27% | -0.60% | -1.10% | -0.48% | -0.59% | -0.92% | -0.38% |
| Overall | -0.59% | -0.51% | -0.06% | -0.59% | -0.70% | -0.24% | -0.58% | -0.60% | -0.17% |

|  | End-to-end WS-PSNR |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case a VS HEC |  |  | Case b VS HEC |  |  | Case c VS HEC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class S1 | -0.31% | -0.90% | -0.63% | -0.31% | -1.05% | -0.78% | -0.30% | -1.01% | -0.73% |
| Class S2 | -0.22% | -1.04% | -0.72% | -0.22% | -1.29% | -0.93% | -0.22% | -1.11% | -0.83% |
| Overall | -0.27% | -0.96% | -0.67% | -0.28% | -1.15% | -0.84% | -0.27% | -1.05% | -0.77% |

Class S1 includes 8K sequences and Class S2 includes 6K sequences.

Additional memory analysis

When blocks located at face boundaries are filtered using their spherical neighbors, additional memory for spherical adjacent blocks is needed. In the above three cases, different additional memory is needed.

Case a

Luma component of spherical adjacent blocks

Case b

Luma component of spherical adjacent blocks

Chroma components of spherical adjacent blocks

Case c

Luma component of spherical adjacent blocks

Chroma components of spherical adjacent blocks

Prediction mode of spherical adjacent blocks

Conclusion

In-loop filtering using spherical neighbors can solve the seam artifacts roughly. The coding performance of different boundary strength values is similar. Therefore we suggest judge these cases by subjective observation.

Reference

- Philippe Hanhart, Jian-Liang Lin, Chirag Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” JVET-K1033, July 2018.

- P. Hanhart, J. Boyce, K. Choi, “Common test conditions and evaluation procedures for 360° video,” JVET-K1012, July 2018.

Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).