| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0424r1 |
| --- | --- |

| Title: | CE1 tests 3.1.1, 3.1.2, 3.2.1, 3.2.2: Separate intra trees |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall, Frank Bossen 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com frank@bossentech.com |
| Source: | Sharp Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1 tests 3.1.1, 3.1.2, 3.2.1 and 3.2.2.  These tests consider the support for separate intra trees, or dual trees, within an inter slice.  The motivation for these tests is to allow for the use of the same intra-coding tools in both intra and inter slices.  The tests evaluate the performance of the proposed approach using the common test condition (CTC) as well as synthetically generated sequences.  During the creation of the CE tests, it was understood that the CTC sequences do not contain a large number of intra-blocks in inter-slices, and so these synthetically generated sequences were also included in the test to create sequences with a large number of intra blocks in inter slices.  The method used for creating these sequences was discussed during the CE test creation, and the purpose of the synthetic test is to better evaluate the coding efficiency improvement of supporting dual trees in both intra and inter-slices.

The following four configurations of dual tree signaling were considered:

- 3.1.1: CU level separate trees for intra-coded CUs in inter slices

- 3.1.2: Separate trees for intra-coded CUs in inter slices combined with support for both separate and shared trees in intra-slices.

- 3.2.1: Separate tree support at CTU level for intra-coded CTUs in inter slices.

- 3.2.2: Combination of 3.1.2 and 3.2.1.  For all slices, flag sent at CTU level.  If flag is true, then CTU is coded as intra CTU using separate trees.  If flag is false, then method of 3.1.2 is used.

The average BD Rate performance for the tests versus VTM-2.0.1 is as follows:

Common test conditions:

|  | All Intra |  |  | Random Access (RA) |  |  | Low Delay B (LDB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V | Y | U | V |
| 3.1.1 | -0.01% | -0.07% | -0.06% | -0.13% | -2.32% | -1.87% | -0.03% | -0.55% | -0.63% |
| 3.1.2 | 0.02% | -0.05% | -0.05% | -0.11% | -2.25% | -1.89% | 0.00% | -0.66% | -0.89% |
| 3.2.1 | -0.01% | -0.07% | -0.06% | 0.01% | 0.01% | 0.02% | -0.04% | -0.06% | -0.08% |
| 3.2.2 | 0.02% | -0.05% | -0.05% | -0.11% | -2.24% | -1.87% | 0.04% | -0.43% | -0.78% |

Synthetic sequences:

|  | RA synthetic sequences (RA-SYN) |  |  | LDB synthetic sequences (LDB-SYN) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V |
| 3.1.1 | -0.69% | -3.24% | -3.04% | -0.47% | -4.18% | -5.06% |
| 3.1.2 | -0.69% | -3.24% | -3.04% | -0.47% | -4.09% | -5.07% |
| 3.2.1 | 0.00% | 0.01% | 0.01% | 0.01% | 0.01% | 0.03% |
| 3.2.2 | -0.69% | -3.24% | -3.04% | -0.47% | -4.10% | -5.06% |

Revision 1 of the contribution provides updated encoder and decoder runtime data for Tests 3.1.1 and 3.1.2. Furthermore, the revision provides results for an alternative encoder configuration for Test 3.1.1.  In this non-normative change, separate tree testing is performed after regular testing.  The encoder then skips the separate tree testing if the shared-tree tests not yield a large fraction of intra-coded samples. The speedup is reported to reduce encoder runtime for the random access case from 122% to 107%.

# Introduction

In the current VVC draft [1], a CTU in an intra slice type may undergo separate tree, or dual tree,  partitioning. The goal of this contribution is to extend the dual tree concept to a block of intra samples in an inter slice.  This removes the current limitation that an encoder cannot use all of the intra-coding tools in an inter-slice that are available in an intra-slice.  It also has the additional benefit of harmonizing signaling across slice types.

## Separate intra tree signaling in Inter Slices

Figure 1 provides an overview of the core approach.  As can be seen in the Figure, CTUs in an inter-slice are first partitioned using a shared tree, as is done in the current VTM and BMS software.  However, for blocks coded using an intra mode in these CTUs, we propose to then allow the intra region to be further partitioned using a separate, or dual, tree.  The indication of the dual tree partition is either received in the bit-stream or inferred based on number of samples in the block.

Figure 1: Illustration showing separate tree signaling in inter slices

Signaling of the presence of a separate tree is accomplished with a combination of explicit signaling and inference.  Specifically, when the number of luma samples are large, then it is inferred that a dual tree is present.  Conversely, when the number of luma samples is small, then it is inferred that a dual tree is not present.  Otherwise, a flag in the bit-stream controls if a dual tree is present or not.  In the current proposal, the thresholds for what is a large and small number of luma samples are signaled in the SPS.  Of course, as the standard matures, these could become parameters defined in a profile or level. A flag is also included in the SPS to indicate whether separate intra trees are enabled for Inter Slices.

## Separate intra tree signaling in Intra Slices

In Test 3.1.2 and Test 3.2.2, we further apply the approach in Figure 1 to intra slices.  In the current VVC draft, when separate trees are enabled in an intra slice type, a block undergoes implicit quad-tree splits until both sides of the block are less than or equal to 64, at which point separate trees are inferred. For Test 3.1.2 and Test 3.2.2, we similarly split a block with implicit quad-tree splits until both sides of the block are less than or equal to 64. After reaching a 64x64 region, we then apply the approach in Figure 1.  Specifically, a shared tree signaling is performed.  After terminating the shared tree (by receiving a NO_SPLIT flag), the presence of a separate tree is signaled is a similar manner as in Figure 1.

# CE test motivation and description

The proposed approach has been tested previously in a core experiment.  And, the purpose of this test is to additionally evaluate the amount of coding gains that can be achieved with synthetically created sequences, where inter-prediction fails more often (compared to CTC). Briefly, the following four configurations of separate tree signaling were tested [2]:

| Test | Description |
| --- | --- |
| 3.1.1 | CU level separate trees for intra-coded CUs in inter slices |
| 3.1.2 | Separate trees for intra-coded CUs in inter slices combined with support for both separate and shared trees in intra-slices. |
| 3.2.1 | Separate tree support at CTU level for intra-coded CTUs in inter slices. |
| 3.2.2 | Combination of 3.1.2 and 3.2.1. For all slices, flag sent at CTU level. If flag is true, then CTU is coded as intra CTU using separate trees. If flag is false, then method of 3.1.2 is used. |

Synthetic sequence generation was discussed during the creation of the core experiment, and the resulting approach was used to generate the sequences by copying sample values from another sequence in a checker-board pattern. The location of the checkerboard pattern was modified for every GOP.  A visual description appears in figure below. Software to create the synthetic sequences was shared over the CE1 email list.

Figure 2: Checkerboard pattern used for each GOP when generating synthetic sequences

# Experimental Results

The coding efficiency of each test was evaluated with VTM-2.0.1 used as anchors. The synthetic sequences were evaluated for (a) class A1, A2, B random-access configuration; and (b) class B low delay B configuration. The summary results for each test is listed below. The detailed results can be found in the attached excel.

## Test 3.1.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class A1 | 0.00% | 0.04% | -0.01% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.01% | -0.03% | -0.12% | 104% | 104% |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.08% | 102% | 100% |  |  |  |  |  |
| Class C | -0.03% | -0.23% | -0.05% | 101% | 102% |  |  |  |  |  |
| Class E | -0.01% | -0.12% | -0.29% | 105% | 103% |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.06% | 102% | 102% |  |  |  |  |  |
| Class D | 0.02% | 0.02% | -0.26% | 104% | 103% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -4.00% | -2.10% | 126% | 103% | -0.81% | -2.92% | -1.55% | 137% | 106% |
| Class A2 | -0.06% | -2.14% | -0.90% | 124% | 102% | -0.49% | -1.91% | -1.00% | 151% | 106% |
| Class B | -0.13% | -2.26% | -2.64% | 121% | 102% | -0.75% | -4.24% | -5.16% | 144% | 105% |
| Class C | 0.07% | -1.26% | -1.45% | 120% | 100% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.13% | -2.32% | -1.87% | 122% | 102% | -0.69% | -3.24% | -3.04% | 144% | 105% |
| Class D | 0.15% | -0.71% | -0.15% | 116% | 99% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.64% | -1.12% | 112% | 102% | -0.47% | -4.18% | -5.06% | 152% | 104% |
| Class C | 0.06% | -0.36% | -0.32% | 111% | 102% |  |  |  |  |  |
| Class E | -0.22% | -0.65% | -0.22% | 104% | 101% |  |  |  |  |  |
| Overall | -0.03% | -0.55% | -0.63% | 109% | 101% | -0.47% | -4.18% | -5.06% | 152% | 104% |
| Class D | 0.05% | -0.32% | -0.24% | 109% | 101% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |

## Test 3.1.2

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class A1 | 0.02% | 0.11% | 0.00% | 101% | 97% |  |  |  |  |  |
| Class A2 | 0.00% | -0.06% | -0.14% | 101% | 100% |  |  |  |  |  |
| Class B | 0.03% | 0.05% | 0.10% | 101% | 97% |  |  |  |  |  |
| Class C | 0.02% | -0.22% | -0.07% | 100% | 99% |  |  |  |  |  |
| Class E | 0.01% | -0.14% | -0.24% | 99% | 98% |  |  |  |  |  |
| Overall | 0.02% | -0.05% | -0.05% | 100% | 98% |  |  |  |  |  |
| Class D | 0.14% | 0.22% | -0.14% | 101% | 102% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -3.91% | -2.28% | 126% | 103% | -0.81% | -2.89% | -1.54% | 137% | 104% |
| Class A2 | -0.03% | -2.11% | -0.92% | 121% | 102% | -0.49% | -1.89% | -1.00% | 151% | 104% |
| Class B | -0.11% | -2.16% | -2.62% | 121% | 102% | -0.75% | -4.27% | -5.17% | 144% | 103% |
| Class C | 0.09% | -1.23% | -1.41% | 121% | 101% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.11% | -2.25% | -1.89% | 122% | 102% | -0.69% | -3.24% | -3.04% | 144% | 103% |
| Class D | 0.17% | -0.52% | -0.19% | 117% | 101% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.77% | -1.31% | 111% | 102% | -0.47% | -4.09% | -5.07% | 150% | 101% |
| Class C | 0.07% | -0.38% | -0.63% | 110% | 100% |  |  |  |  |  |
| Class E | -0.12% | -0.85% | -0.54% | 103% | 102% |  |  |  |  |  |
| Overall | 0.00% | -0.66% | -0.89% | 108% | 101% | -0.47% | -4.09% | -5.07% | 150% | 101% |
| Class D | 0.05% | 0.27% | -0.20% | 108% | 101% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |

## Test 3.2.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class A1 | 0.00% | 0.04% | -0.01% | 79% | 91% |  |  |  |  |  |
| Class A2 | -0.01% | -0.03% | -0.12% | 97% | 103% |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.08% | 89% | 95% |  |  |  |  |  |
| Class C | -0.03% | -0.23% | -0.05% | 86% | 91% |  |  |  |  |  |
| Class E | -0.01% | -0.12% | -0.29% | 75% | 82% |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.06% | 85% | 92% |  |  |  |  |  |
| Class D | 0.02% | 0.02% | -0.26% | 83% | 88% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.00% | 92% | 92% | 0.01% | 0.02% | -0.01% | 88% | 95% |
| Class A2 | 0.02% | -0.12% | 0.03% | 91% | 91% | 0.00% | 0.02% | 0.02% | 92% | 94% |
| Class B | 0.01% | 0.03% | 0.07% | 88% | 92% | -0.02% | -0.01% | 0.01% | 87% | 90% |
| Class C | 0.02% | 0.03% | -0.03% | 107% | 106% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 94% | 95% | 0.00% | 0.01% | 0.01% | 88% | 92% |
| Class D | 0.02% | -0.26% | 0.04% | 97% | 98% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.14% | 79% | 87% | 0.01% | 0.01% | 0.03% | 77% | 80% |
| Class C | 0.00% | 0.03% | -0.14% | 85% | 90% |  |  |  |  |  |
| Class E | -0.17% | -0.32% | -0.37% | 84% | 89% |  |  |  |  |  |
| Overall | -0.04% | -0.06% | -0.08% | 82% | 89% | 0.01% | 0.01% | 0.03% | 77% | 80% |
| Class D | 0.02% | 0.00% | -0.73% | 86% | 88% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |

## Test 3.2.2

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class A1 | 0.02% | 0.11% | 0.00% | 84% | 93% |  |  |  |  |  |
| Class A2 | 0.00% | -0.06% | -0.14% | 95% | 104% |  |  |  |  |  |
| Class B | 0.04% | 0.05% | 0.10% | 83% | 89% |  |  |  |  |  |
| Class C | 0.02% | -0.22% | -0.07% | 89% | 92% |  |  |  |  |  |
| Class E | 0.01% | -0.14% | -0.24% | 74% | 82% |  |  |  |  |  |
| Overall | 0.02% | -0.05% | -0.05% | 85% | 91% |  |  |  |  |  |
| Class D | 0.14% | 0.22% | -0.14% | 85% | 89% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -3.89% | -2.25% | 104% | 90% | -0.81% | -2.89% | -1.53% | 121% | 100% |
| Class A2 | -0.02% | -2.09% | -0.91% | 101% | 87% | -0.48% | -1.89% | -1.00% | 129% | 98% |
| Class B | -0.10% | -2.15% | -2.60% | 112% | 93% | -0.74% | -4.27% | -5.16% | 122% | 93% |
| Class C | 0.10% | -1.23% | -1.40% | 105% | 90% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.11% | -2.24% | -1.87% | 106% | 90% | -0.69% | -3.24% | -3.04% | 124% | 96% |
| Class D | 0.17% | -0.52% | -0.18% | 103% | 89% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.53% | -1.34% | 89% | 87% | -0.47% | -4.10% | -5.06% | 114% | 82% |
| Class C | 0.08% | -0.35% | -0.57% | 99% | 90% |  |  |  |  |  |
| Class E | 0.01% | -0.39% | -0.10% | 88% | 89% |  |  |  |  |  |
| Overall | 0.04% | -0.43% | -0.78% | 92% | 89% | -0.47% | -4.10% | -5.06% | 114% | 82% |
| Class D | 0.05% | 0.28% | -0.19% | 93% | 86% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |

## Test 3.1.1 encoder speedup

Separate tree testing is moved to be after regular testing and is skipped if the regular splits do not yield a large fraction of samples that are intra-coded.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |  |  |  |  |
| Class A1 | 0.00% | 0.04% | -0.01% | 103% | 101% |  |  |  |  |  |
| Class A2 | -0.01% | -0.03% | -0.12% | 99% | 102% |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.08% | 103% | 101% |  |  |  |  |  |
| Class C | -0.03% | -0.23% | -0.05% | 102% | 103% |  |  |  |  |  |
| Class E | -0.01% | -0.12% | -0.29% | 100% | 102% |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.06% | 102% | 102% |  |  |  |  |  |
| Class D | 0.02% | 0.02% | -0.26% | 102% | 103% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -2.42% | -1.58% | 111% | 102% | -0.35% | -1.84% | -1.34% | 117% | 106% |
| Class A2 | -0.06% | -1.48% | -0.70% | 109% | 101% | -0.24% | -1.57% | -0.94% | 120% | 105% |
| Class B | -0.03% | -1.65% | -1.95% | 106% | 102% | -0.27% | -3.51% | -4.32% | 118% | 105% |
| Class C | 0.14% | -0.71% | -0.85% | 103% | 101% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.04% | -1.52% | -1.33% | 107% | 101% | -0.29% | -2.52% | -2.59% | 118% | 105% |
| Class D | 0.15% | -0.59% | 0.02% | 102% | 101% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 (Synthetic) |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.18% | -0.34% | 106% | 101% | -0.10% | -3.14% | -3.79% | 118% | 102% |
| Class C | 0.09% | -0.07% | -0.07% | 101% | 99% |  |  |  |  |  |
| Class E | -0.18% | -0.35% | -0.41% | 99% | 101% |  |  |  |  |  |
| Overall | 0.00% | -0.19% | -0.27% | 102% | 101% | -0.10% | -3.14% | -3.79% | 118% | 102% |
| Class D | 0.07% | -0.23% | -0.16% | 103% | 101% |  |  |  |  |  |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |  |  |  |  |

# Conclusion

The contribution reports the performance of supporting the separate tree, or dual tree, concept in inter slices, which is asserted to have the benefit of enabling support for the same set of intra-coding tools in both slice types.  Results are reported for both the common test conditions (CTC) as well as synthetically generated sequences, as requested during the creation of the CE. It was shown that for the synthetically generated sequences in random access configuration the BD rate is -0.7% (Y) / -3.2% (U) / -3%(V). Results are also reported for an encoder speedup for separate tree testing. We recommend the adoption of separate trees for inter slices in the next draft of VVC.

# References

[1] JVET-K1001 “Versatile Video Coding (Draft 2),” B. Bross, J. Chen, S. Liu; 11th JVET Meeting: Ljubljana, SI, 10–18 July 2018

[2] JVET-K1021 “Description of Core Experiment 1 (CE1): Partitioning,” J. Ma, F. Le Léannec, M. W. Park; 11th JVET Meeting: Ljubljana, SI, 10–18 July 2018

[3] JVET-K0353 “CE1: Shared-separate partition tree in QT+BT configuration (Tests 5.3.1 and 5.4.1),” K. Misra, W. Zhu, A. Segall; 11th JVET Meeting: Ljubljana, SI, 10–18 July 2018

[4] JVET-K0354 “CE1: Shared-separate partition tree in QT+BT+TT configuration (Tests 5.3.2 and 5.4.2),” K. Misra, W. Zhu, A. Segall; 11th JVET Meeting: Ljubljana, SI, 10–18 July 2018

[5] JVET-J0026 “Description of SDR and HDR video coding technology proposal by Sharp and Foxconn,” 10th JVET Meeting: San Diego, US, 10–20 Apr. 2018

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).