| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0427-v1 |
| --- | --- |

| Title: | CE15-related: Separate Palette Coding for Luma and Chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Roman Chernyak Sergey Ikonin Jianle Chen | Tel: Email: | +7-925-008-12-86 chernyak.roman@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

The contribution proposes a unification of Palette Coding tool for Single and Dual Trees.

# Proposed method

In the VVC 2 standard draft [1], separate tree approach may be used depend on codec configuration file. Thus, in general codec can operate with either Single CU, which includes one Luma CB and two Chroma CBs; or Dual CUs, which assumes separated Luma CU with one Luma CB and Chroma CU with two Chroma CB.

This contribution proposes a unified method of Palette Coding in VVC codec, if separate tree approach is possible. It is proposed always applying Palette Coding separately for Y component and for Cb-Cr jointly regardless of partitioning tree type. Figure 1 illustrates such approach.

Figure 1 - A diagram of partitioning trees possibilities and Palette Coding signaling

# Simulation results

Simulations were conducted on top of HEVC SCC Palette Coding implementation in VVC SW (CE15.2). To illustrate experimental results for proposed solution Common Test Conditions (CTC) [2] were used with disabled Dual Tree approach. Table 1 demonstrates the results.

Table 1 – Simulation results of proposed method under CTC w/o Dual Tree

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -9.12% | -9.05% | -10.20% | 123% | 90% |
| Class TGM 4:2:0 (optional) | -25.67% | -25.39% | -24.92% | 134% | 64% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -7.02% | -7.24% | -8.34% | 120% | 100% |
| Class TGM 4:2:0 (optional) | -12.78% | -13.89% | -13.02% | 129% | 91% |

In Table 1 an anchor is BMS2.0.1 in VTM configuration; a test is the anchor plus HEVC SCC Palette Coding implementation (CE15.2) with proposed modification.

For comparison purposes HEVC SCC Palette Coding implementation was also tested under CTC and CTC without dual tree. Table 2 and Table 3 correspondently demonstrates this results.

Table 2 – Simulation results of HEVC SCC Palette Coding under CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -11.51% | -9.19% | -9.23% | 125% | 94% |
| Class TGM 4:2:0 (optional) | -33.48% | -28.20% | -28.24% | 136% | 71% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -8.89% | -8.33% | -8.76% | 120% | 100% |
| Class TGM 4:2:0 (optional) | -17.06% | -16.01% | -15.69% | 129% | 91% |

Table 3 – Simulation results of HEVC SCC Palette Coding under CTC w/o Dual Tree

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -5.60% | -7.24% | -7.75% | 119% | 98% |
| Class TGM 4:2:0 (optional) | -18.46% | -19.60% | -19.71% | 128% | 84% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -4.02% | -5.83% | -6.28% | 121% | 102% |
| Class TGM 4:2:0 (optional) | -9.25% | -10.51% | -10.23% | 128% | 95% |

# Conclusion

Proposed modification unifies Palette Coding design in case if dual tree is possible regardless of certain type of partitioning tree and demonstrates coding gain in comparison to HEVC SCC Palette Coding.

# References

- B. Bross, J. Chen, S. Liu „Versatile Video Coding (Draft 2)“, JVET-K1001-v5, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, Joint Video Experts Team, doc. JVET-K1010, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).