| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0463 |
| --- | --- |

| Title: | Crosscheck of JVET-L0124: CE10.3.1.b Triangular prediction unit mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Max Bläser | Tel: Email: | +49 241 80 27677 blaeser@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report for JVET-L0124, concerning triangular prediction unit mode as described in experiment 3.1.b of CE10. The test results presented in JVET-L0124, including the additional information given by the proponents, are confirmed to be accurate.

# Introduction

The software provided by the proponents was reviewed and it is confirmed that the proposed changes to the CE10 code base are related to enabling triangular inter-picture prediction for the MERGE and SKIP mode. Further, the bandwidth restriction that is applied in the additional test is implemented as specified in JVET-L0124 (uni-prediction restriction for < 8x8 and further ATMVP and AFFINE restrictions). The coding performance was evaluated under the following testing conditions:

- Common testing conditions according to JVET-K1010

- RA and LB were tested for VTM configuration (BMS code base with JEM_TOOLS on)

- Anchor was the initial CE10 master branch (BMS code base with JEM_TOOLS on), commit hash

3d43a6ba412907b06ca78bf2520cdfd022497467

- Source code was compiled using ggc 7.2.0

- Simulations for RA were performed using parallel encoding and bitstream concatenation

Minor differences in relative encoder-runtimes are observed due to potential inaccuracies that occur when the computing cluster operates under changing workloads. The simulated anchor (tool-off) from the CE10 master branch is slightly different compared to the anchor reported by the proponents. The reported bitrates and PSNR values for the tool-on case however matches exactly.

# Simulation results

## CE10 3.1.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.51% | -0.90% | 111% | 103% |
| Class A2 | -0.51% | -0.83% | -0.83% | 108% | 103% |
| Class B | -0.46% | -0.68% | -0.55% | 121% | 100% |
| Class C | -0.93% | -1.22% | -1.26% | 126% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.55% | -0.82% | -0.86% | 117% | 102% |
| Class D | -0.57% | -0.83% | -1.04% | 126% | 101% |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.94% | -1.14% | -1.41% | 120% | 100% |
| Class C | -1.28% | -1.88% | -1.67% | 121% | 100% |
| Class E | -1.22% | -1.55% | -1.09% | 111% | 100% |
| Overall | -1.26% | -1.40% | -1.47% | 121% | 101% |
| Class D | -1.79% | -1.18% | -1.30% | 116% | 101% |
| Class F |  |  |  |  |  |

## CE10 3.1.b with worst-case memory bandwidth restriction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.46% | -0.91% | 111% | 103% |
| Class A2 | -0.41% | -0.77% | -0.47% | 109% | 102% |
| Class B | -0.32% | -0.51% | -0.35% | 120% | 98% |
| Class C | -0.69% | -0.96% | -0.97% | 125% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.43% | -0.67% | -0.65% | 117% | 99% |
| Class D | -0.09% | -0.44% | -0.46% | 127% | 100% |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.84% | -1.06% | -1.57% | 113% | 98% |
| Class C | -1.08% | -1.56% | -1.50% | 123% | 98% |
| Class E | -0.70% | -1.15% | -1.19% | 120% | 97% |
| Overall | -1.14% | -1.37% | -1.51% | 116% | 100% |
| Class D | -1.71% | -1.62% | -1.44% | 126% | 96% |
| Class F |  |  |  |  |  |

# Acknowledgments

Simulations were performed with computing resources granted by RWTH Aachen University under project rwth0333.