| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0486-v1 |
| --- | --- |

| Title: | CE6: Add DST-2/DCT-2 and switch AMT candidate depending on intra direction (CE6-1.7d and CE6-1.7e) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki (Sony) Karam Naser, Fabrice Le Leannec, Edouard Francois (Technicolor) | Tel: Email: | +81 50 3750 2782 masaru.ikeda@sony.com teruhiko.s@sony.com karam.naser@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor, Sony Corporation |  |  |

_____________________________

- Abstract

This contribution reports results of CE6-1.7d and CE6-1.7e. In this contribution, it is proposed that DST-2 and DCT-2 are added as AMT candidates and the transform subset is selected based on the intra prediction mode. Those tests are tested using BMS-2.0.1 with both 1) VTM configuration and 2) VTM configuration and inter MTS enabled.

It is reported that CE6-1.7d provides an average BD-rate luma loss of -0.21%, -0.07% and -0.10% with VTM configuration at AI, RA and LDB, respectively, and CE6-1.7e provides an average BD-rate luma loss of -0.21%, -0.08% and -0.08% with VTM configuration and inter MTS enabled co in AI, RA and LDB configurations, respectively.

It is reported that CE6-1.7e provides an average BD-rate luma loss of 0.08%, 0.11% and -0.01% with VTM configuration at AI, RA and LDB, respectively, and CE6-1.7e provides an average BD-rate luma loss of 0.08%, 0.07% and -0.22% with VTM configuration and inter MTS enabled co in AI, RA and LDB configurations, respectively.

Proposed method

In the contributions [1] [2], intra prediction mode depending AMT was proposed with the basis function of DST-4 and DCT-4 with which replaced DST-7 and DCT-8, respectively. The proposed method is that DST-2 and DCT-2 are added on top of DST-4 and DCT-4 as AMT candidates and the transform subset is selected based on the intra prediction mode with max AMT size restriction. The proposed method is tested on top of both DST-7/DCT-8 (current MTS) [3] and DST-4/DCT-4 (CE-6.1.7a) with max AMT size restriction as shown in Table 1.

| CE6.1.7-d | AMT intra: DST-7 / DCT-8 / DST-2 / DCT-2 AMT inter: DCT-8 / DST-7 Max AMT size: 32 |
| --- | --- |
| CE6.1.7-e | AMT intra: DST-4 / DCT-4 / DST-2 / DCT-2 AMT inter: DCT-4 / DST-4 Max AMT size: 32 |

Table 1 Test description of CE6-1.7d and CE6-1.7e

Experimental results

CE6-1.7d

Test-1: BMS-2.0.1 with VTM configuration

The following tables summarize the BD-rate results in AI, RA and LDB configurations.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.14% | -0.03% | 87% | #DIV/0! |
| Class A2 | -0.13% | -0.18% | -0.16% | 89% | #DIV/0! |
| Class B | -0.18% | -0.12% | -0.12% | 84% | #DIV/0! |
| Class C | -0.24% | -0.19% | -0.11% | 82% | #DIV/0! |
| Class E | -0.41% | -0.29% | -0.37% | 88% | #DIV/0! |
| Overall | -0.21% | -0.18% | -0.15% | 85% | #DIV/0! |
| Class D | -0.23% | -0.06% | -0.30% | 85% | #DIV/0! |
| Class F (optional) | -0.21% | -0.23% | -0.15% | 80% | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.11% | 0.15% | 110% | #DIV/0! |
| Class A2 | -0.05% | 0.00% | -0.10% | 108% | #DIV/0! |
| Class B | -0.08% | -0.11% | -0.02% | 106% | #DIV/0! |
| Class C | -0.10% | 0.05% | 0.06% | 121% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.07% | 0.00% | 0.02% | 111% | #DIV/0! |
| Class D | -0.10% | -0.05% | 0.43% | 104% | #DIV/0! |
| Class F (optional) | -0.14% | 0.09% | -0.05% | 87% | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.06% | -0.20% | 103% | #DIV/0! |
| Class C | -0.03% | -0.04% | -0.13% | 101% | #DIV/0! |
| Class E | -0.29% | 0.64% | -0.67% | 100% | #DIV/0! |
| Overall | -0.10% | 0.13% | -0.29% | 102% | #DIV/0! |
| Class D | -0.01% | -0.11% | 0.05% | 101% | #DIV/0! |
| Class F (optional) | -0.16% | 0.32% | -0.29% | 100% | #DIV/0! |

Table 2: BD-Rate results of CE6-1.7d with VTM configuration

Test-2: BMS-2.0.1 with VTM configuration and inter MTS enabled

Another test was conducted with inter MTS enabled. The parameters are as follows:

- EMT = 3

- EMTFast = 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.14% | -0.03% | #DIV/0! | #DIV/0! |
| Class A2 | -0.13% | -0.18% | -0.16% | #DIV/0! | #DIV/0! |
| Class B | -0.18% | -0.12% | -0.12% | #DIV/0! | #DIV/0! |
| Class C | -0.24% | -0.19% | -0.11% | #DIV/0! | #DIV/0! |
| Class E | -0.41% | -0.29% | -0.37% | #DIV/0! | #DIV/0! |
| Overall | -0.21% | -0.18% | -0.15% | #DIV/0! | #DIV/0! |
| Class D | -0.23% | -0.06% | -0.30% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.21% | -0.23% | -0.15% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | 0.15% | #DIV/0! | #DIV/0! |
| Class A2 | -0.05% | 0.12% | 0.00% | #DIV/0! | #DIV/0! |
| Class B | -0.10% | -0.14% | -0.08% | #DIV/0! | #DIV/0! |
| Class C | -0.11% | -0.09% | 0.01% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.04% | 0.01% | #DIV/0! | #DIV/0! |
| Class D | -0.11% | -0.01% | 0.38% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.12% | 0.03% | -0.16% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.19% | 0.16% | #DIV/0! | #DIV/0! |
| Class C | -0.03% | 0.16% | -0.04% | #DIV/0! | #DIV/0! |
| Class E | -0.20% | 0.90% | 0.16% | #DIV/0! | #DIV/0! |
| Overall | -0.08% | 0.20% | 0.09% | #DIV/0! | #DIV/0! |
| Class D | -0.07% | 0.24% | 0.39% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.17% | -0.15% | -0.40% | #DIV/0! | #DIV/0! |

Table 3: BD-Rate results of CE6-1.7d with VTM configuration and inter MTS enabled

CE6-1.7e

Test-1: BMS-2.0.1 with VTM configuration

The following tables summarize the BD-rate results in AI, RA and LDB configurations.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.60% | 0.60% | 0.68% | 97% | #DIV/0! |
| Class A2 | 0.33% | 0.08% | 0.02% | 98% | #DIV/0! |
| Class B | 0.17% | 0.05% | 0.14% | 97% | #DIV/0! |
| Class C | -0.32% | -0.29% | -0.15% | 87% | #DIV/0! |
| Class E | -0.27% | -0.28% | -0.39% | 99% | #DIV/0! |
| Overall | 0.08% | 0.02% | 0.06% | 95% | #DIV/0! |
| Class D | -0.42% | -0.24% | -0.60% | 86% | #DIV/0! |
| Class F (optional) | -0.25% | -0.21% | -0.26% | 89% | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.28% | 0.43% | 0.39% | 109% | #DIV/0! |
| Class A2 | 0.19% | 0.15% | 0.09% | 104% | #DIV/0! |
| Class B | 0.13% | 0.10% | 0.11% | 104% | #DIV/0! |
| Class C | -0.10% | 0.09% | 0.05% | 90% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.17% | 0.14% | 101% | #DIV/0! |
| Class D | -0.14% | -0.32% | 0.21% | 77% | #DIV/0! |
| Class F (optional) | -0.12% | -0.06% | -0.14% | 88% | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.04% | 0.24% | 99% | #DIV/0! |
| Class C | -0.06% | -0.15% | -0.23% | 101% | #DIV/0! |
| Class E | -0.06% | -0.68% | -0.70% | 99% | #DIV/0! |
| Overall | -0.01% | -0.24% | -0.15% | 100% | #DIV/0! |
| Class D | -0.06% | 0.23% | -0.22% | 100% | #DIV/0! |
| Class F (optional) | -0.19% | 0.32% | -0.41% | 108% | #DIV/0! |

Table 4: BD-Rate results of CE6-1.7e with VTM configuration

Test-2: BMS-2.0.1 with VTM configuration and inter MTS enabled

The following tables summarize the BD-rate results in AI, RA and LDB configurations.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.60% | 0.60% | 0.68% | #DIV/0! | #DIV/0! |
| Class A2 | 0.33% | 0.08% | 0.02% | #DIV/0! | #DIV/0! |
| Class B | 0.17% | 0.05% | 0.14% | #DIV/0! | #DIV/0! |
| Class C | -0.32% | -0.29% | -0.15% | #DIV/0! | #DIV/0! |
| Class E | -0.27% | -0.28% | -0.39% | #DIV/0! | #DIV/0! |
| Overall | 0.08% | 0.02% | 0.06% | #DIV/0! | #DIV/0! |
| Class D | -0.42% | -0.24% | -0.60% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.25% | -0.21% | -0.26% | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.27% | 0.16% | 0.38% | #DIV/0! | #DIV/0! |
| Class A2 | 0.23% | 0.02% | 0.11% | #DIV/0! | #DIV/0! |
| Class B | 0.10% | -0.07% | -0.11% | #DIV/0! | #DIV/0! |
| Class C | -0.23% | -0.01% | 0.01% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.01% | 0.06% | #DIV/0! | #DIV/0! |
| Class D | -0.33% | -0.45% | 0.01% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.16% | -0.11% | -0.20% | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | -0.16% | 0.20% | #DIV/0! | #DIV/0! |
| Class C | -0.36% | -0.02% | -0.15% | #DIV/0! | #DIV/0! |
| Class E | -0.23% | -0.57% | -0.70% | #DIV/0! | #DIV/0! |
| Overall | -0.22% | -0.22% | -0.14% | #DIV/0! | #DIV/0! |
| Class D | -0.45% | -0.14% | 0.15% | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.44% | -0.09% | -0.83% | #DIV/0! | #DIV/0! |

Table 5: BD-Rate results of CE6-1.7e with VTM configuration and inter MTS enabled

Conclusion

This contribution reports the performance of CE6-1.7d and 1.7e. It is proposed that DST-2 and DCT-2 are added as AMT candidates and the transform subset is selected based on the intra prediction mode. The results should be reviewed by CE6 Summary Report together with other related proposals. The results show that especially CE6-1.7e would be a good tradeoff between complexity and performance. Therefore, this contribution proposes to adopt the method into next working draft.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Reference

- K. Naser, F. Le Léannec, E. François, “CE6-related: Type4 only AMT” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0265,

- T. Tsukuba, M. Ikeda, T. Suzuki, “CE6-related: AMT with only Type2/Type4 DCT/DST” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0394

- J. Chen, Y. Ye, S. Kim., “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1002