| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0499-v2 |
| --- | --- |

| Title: | Crosscheck for JVET-L0087 (CE3-related: Boundary PDPC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 l.li@lge.com jin78.heo@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This document reports the crosscheck result for JVET-L0087: CE3-related: Boundary PDPC.

Introduction

The algorithm described in JVET-L0087 proposes apply PDPC only to top-left boundary samples to reduce operational complexity.

Experimental Results

The algorithm is evaluated on BMS 2.0.1 [1] under VTM configuration according to the common test conditions [2]. Both all intra and random access are tested. The summarized results are shown in Tables 1. The detailed results can be found in Excel files in the package.

One minor bit mismatch is found in sequence Tango with QP 27 of random access case. It is on the fourth digit after decimal point.

Table 1 TEST1: crosscheck result for changing DM from top-left to central area using BMS 2.0.1 with VTM config as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.06% | 0.01% | 99% | 97% |
| Class A2 | 0.01% | 0.03% | -0.01% | 100% | 98% |
| Class B | 0.03% | 0.07% | 0.11% | 99% | 98% |
| Class C | 0.00% | 0.06% | 0.06% | 100% | 99% |
| Class E | 0.05% | 0.04% | 0.06% | 99% | 98% |
| Overall | 0.03% | 0.05% | 0.06% | 99% | 98% |
| Class D | -0.01% | 0.22% | -0.03% | 99% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.17% | 100% | 99% |
| Class A2 | 0.03% | -0.07% | 0.07% | 100% | 100% |
| Class B | 0.01% | 0.06% | 0.03% | 100% | 100% |
| Class C | 0.00% | 0.12% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.07% | 100% | 100% |
| Class D | -0.04% | -0.18% | 0.17% | 100% | 100% |

Summary

The crosscheck results match in general besides one minor mismatch.

References

- BMS-2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS/tags/BMS-2.0.1

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.