| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0519 |
| --- | --- |

| Title: | CE4: Cross-check of contribution JVET-L0278 on CE4.2.5 (simplification of affine merge list construction and move ATMVP to affine merge list) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, A. Robert, F. LeLéannec | Tel: Email: | franck.galpin@technicolor.com antoine.robert@technicolor.com fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-L0178 related to CE4.2.5 (Simplification of affine merge list construction and move ATMVP to affine merge list).

It is reported that VTM results obtained during this cross-checking match the results described in JVET-L0278.

# Introduction

The tests were performed on top of VTM2.0.1. The SIMD level was set to SSE42. Other parameters were similar to the common base 4.2.1: md5 were turn on, output turn off.

# Results

## Test a

The results for the test a) described in [1] are show in the table below. They match the result described in [1].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.59% | -0.90% | 105% | 107% |
| Class A2 | -0.96% | -1.00% | -1.08% | 106% | 107% |
| Class B | -0.65% | -0.46% | -0.56% | 105% | 106% |
| Class C | -0.46% | -0.31% | -0.55% | 106% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.72% | -0.55% | -0.73% | 106% | 106% |
| Class D | -0.58% | -0.55% | -0.62% | 107% | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.28% | -0.49% | 107% | 103% |
| Class C | -0.23% | -0.16% | -0.27% | 105% | 103% |
| Class E | -0.46% | -0.10% | -0.30% | 106% | 105% |
| Overall | -0.33% | -0.20% | -0.37% | 106% | 103% |
| Class D | -0.31% | 0.33% | -0.48% | 105% | 106% |

## Test b

The results for the test b) described in [1] are show in the table below. They match the result described in [1].

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -0.63% | -0.92% | 104% | 105% |
| Class A2 | -0.98% | -0.90% | -1.08% | 105% | 106% |
| Class B | -0.69% | -0.61% | -0.63% | 105% | 106% |
| Class C | -0.54% | -0.49% | -0.56% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.76% | -0.64% | -0.76% | 105% | 106% |
| Class D | -0.72% | -0.64% | -0.77% | 106% | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.51% | -0.53% | 106% | 106% |
| Class C | -0.29% | -0.34% | -0.29% | 105% | 97% |
| Class E | -1.17% | -0.83% | -1.49% | 106% | 109% |
| Overall | -0.53% | -0.53% | -0.69% | 105% | 104% |
| Class D | -0.33% | 0.14% | 0.07% | 105% | 102% |

## Complexity analysis

As in the common base 4.2.1, the test for pure translational model does not appear in the candidate comparisons. For each spatial candidate, the equality of corners is tested. It requires 14 additional candidate comparisons. See JVET-L0521 for more details.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1024, 11th Meeting, Ljubljana, July 2018.

# Patent rights declaration(s)

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution