| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0552-v1 |
| --- | --- |

| Title: | Training initial CABAC states |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 frank@bossentech.com |
| Source: | Sharp Labs America, Inc. |  |  |

_____________________________

# Abstract

Initial CABAC states are retrained on the data set defined by common SDR test conditions. A BD rate improvement in the order of 0.2% on average for RA, LB and LP is reported using such retrained initial states. For class D, average gains in excess of 0.5% are reported. Additional experiments include retraining only subsets of the initial states, and recursively retraining a second time. It is suggested to further study the impact of such training on the development of the VVC standard.

# Problem Statement

New proposals may sometimes be difficult to compare, as it is unclear whether reported improvements come from better syntax or simply better initial CABAC states. Such distinction is particularly difficult when BD rate differences are small.

# Results

The following sections contain a number of experiments. In summary, several observations can be made:

- With VTM 2.0.1 retraining is most effective on contexts that are NOT related to residual and coefficient coding

- Retraining a second time shows a minor additional benefit

- Retraining can be even more effective when applied to CE experiments

- Using untrained 50/50 initial probabilities leads to substantial losses

## Retrain all contexts

The following results were obtained by training the initial states based on the bitstreams generated using common test conditions. There is thus a 100% overlap between training and test sets.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | 0.01% | 105% | 107% |
| Class A2 | -0.01% | -0.06% | -0.04% | 103% | 104% |
| Class B | -0.02% | -0.01% | 0.00% | 104% | 106% |
| Class C | -0.05% | -0.10% | 0.08% | 99% | 102% |
| Class E | -0.10% | -0.28% | -0.22% | 97% | 103% |
| Overall | -0.04% | -0.10% | -0.02% | 102% | 105% |
| Class D | -0.09% | -0.17% | -0.39% | 100% | 102% |
| Class F (optional) | -0.02% | -0.15% | -0.06% | 101% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.04% | -0.12% | 104% | 106% |
| Class A2 | -0.09% | -0.01% | 0.04% | 103% | 107% |
| Class B | -0.17% | -0.24% | -0.29% | 99% | 104% |
| Class C | -0.33% | -0.47% | -0.66% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.21% | -0.29% | 100% | 104% |
| Class D | -0.63% | -0.96% | -0.87% | 100% | 102% |
| Class F (optional) | -0.16% | -0.14% | -0.24% | 99% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.08% | -0.44% | 101% | 102% |
| Class C | -0.28% | -0.48% | -0.84% | 99% | 103% |
| Class E | -0.42% | -1.30% | -1.09% | 102% | 104% |
| Overall | -0.23% | -0.45% | -0.74% | 101% | 103% |
| Class D | -0.62% | -0.86% | -1.92% | 100% | 101% |
| Class F (optional) | -0.22% | 0.54% | -0.17% | 104% | 105% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | 0.28% | 0.04% | 104% | 107% |
| Class C | -0.29% | -0.42% | -0.75% | 98% | 109% |
| Class E | -0.25% | -0.78% | -1.12% | 87% | 100% |
| Overall | -0.20% | -0.22% | -0.52% | 97% | 106% |
| Class D | -0.52% | -1.12% | -2.42% | 96% | 100% |
| Class F (optional) | -0.05% | 0.10% | -1.01% | 103% | 107% |

## Retrain residual coding contexts

The following were obtained by retraining only contexts related to coding a residual.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | 0.02% | 103% | 106% |
| Class A2 | 0.00% | 0.01% | -0.02% | 101% | 104% |
| Class B | 0.00% | 0.00% | 0.02% | 105% | 106% |
| Class C | -0.03% | -0.04% | 0.02% | 102% | 104% |
| Class E | -0.04% | -0.11% | -0.14% | 104% | 105% |
| Overall | -0.01% | -0.04% | -0.02% | 103% | 105% |
| Class D | -0.04% | -0.07% | -0.25% | 98% | 100% |
| Class F (optional) | 0.00% | -0.15% | -0.03% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.22% | 0.02% | 103% | 105% |
| Class A2 | -0.01% | -0.12% | -0.06% | 104% | 106% |
| Class B | 0.00% | -0.01% | 0.04% | 99% | 103% |
| Class C | -0.04% | 0.08% | -0.13% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.05% | -0.03% | 100% | 103% |
| Class D | -0.03% | -0.34% | -0.23% | 98% | 101% |
| Class F (optional) | -0.06% | 0.12% | -0.11% | 98% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.09% | -0.44% | 106% | 104% |
| Class C | 0.02% | 0.26% | -0.33% | 100% | 103% |
| Class E | -0.33% | 0.74% | 0.15% | 102% | 104% |
| Overall | -0.08% | 0.31% | -0.25% | 103% | 104% |
| Class D | -0.02% | 1.16% | -0.78% | 98% | 100% |
| Class F (optional) | -0.20% | 0.17% | -0.26% | 100% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.14% | 0.10% | 103% | 107% |
| Class C | -0.03% | 0.36% | -0.60% | 99% | 110% |
| Class E | -0.05% | 0.85% | 0.05% | 87% | 99% |
| Overall | -0.03% | 0.39% | -0.15% | 97% | 106% |
| Class D | 0.03% | 0.24% | -0.94% | 95% | 99% |
| Class F (optional) | 0.11% | 0.07% | -1.15% | 103% | 106% |

## Retrain other contexts

Training of all the contexts not trained in the experiment above.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.05% | 103% | 104% |
| Class A2 | -0.01% | 0.03% | -0.05% | 104% | 105% |
| Class B | -0.01% | 0.00% | 0.04% | 105% | 106% |
| Class C | -0.05% | -0.12% | 0.09% | 103% | 104% |
| Class E | -0.04% | -0.16% | -0.18% | 102% | 104% |
| Overall | -0.02% | -0.05% | -0.02% | 104% | 105% |
| Class D | -0.08% | -0.10% | -0.46% | 100% | 102% |
| Class F (optional) | 0.00% | 0.01% | 0.07% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.11% | -0.05% | 105% | 108% |
| Class A2 | -0.06% | -0.12% | -0.08% | 102% | 106% |
| Class B | -0.14% | -0.15% | -0.06% | 99% | 104% |
| Class C | -0.31% | -0.46% | -0.40% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.22% | -0.15% | 100% | 104% |
| Class D | -0.61% | -0.78% | -0.52% | 99% | 101% |
| Class F (optional) | -0.10% | -0.25% | -0.25% | 98% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.01% | 0.10% | 102% | 103% |
| Class C | -0.25% | -0.63% | -0.64% | 100% | 104% |
| Class E | -0.52% | -1.52% | -0.71% | 101% | 103% |
| Overall | -0.24% | -0.59% | -0.35% | 101% | 103% |
| Class D | -0.51% | -0.79% | -1.53% | 100% | 100% |
| Class F (optional) | -0.02% | 0.17% | -0.15% | 101% | 105% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.15% | 0.31% | 103% | 108% |
| Class C | -0.20% | -0.36% | -0.51% | 98% | 109% |
| Class E | -0.23% | -1.98% | -0.60% | 88% | 101% |
| Overall | -0.15% | -0.55% | -0.19% | 97% | 107% |
| Class D | -0.39% | -1.46% | -1.23% | 95% | 100% |
| Class F (optional) | 0.08% | -0.03% | -0.19% | 100% | 107% |

## Second iteration of training

In this scenario, initial states are retrained based on the results of a first retraining.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | -0.02% | 104% | 106% |
| Class A2 | -0.01% | 0.00% | -0.04% | 104% | 105% |
| Class B | -0.01% | 0.01% | 0.02% | 105% | 107% |
| Class C | -0.06% | -0.20% | 0.00% | 101% | 104% |
| Class E | -0.11% | -0.21% | -0.21% | 104% | 105% |
| Overall | -0.04% | -0.09% | -0.04% | 104% | 105% |
| Class D | -0.13% | -0.19% | -0.42% | 101% | 102% |
| Class F (optional) | -0.03% | -0.08% | -0.05% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.20% | -0.13% | 106% | 108% |
| Class A2 | -0.08% | 0.00% | 0.06% | 103% | 107% |
| Class B | -0.17% | -0.23% | -0.14% | 99% | 104% |
| Class C | -0.42% | -0.51% | -0.62% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.25% | -0.23% | 101% | 104% |
| Class D | -0.75% | -0.83% | -0.77% | 98% | 101% |
| Class F (optional) | -0.15% | -0.10% | -0.32% | 98% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | 0.32% | -0.43% | 104% | 103% |
| Class C | -0.34% | -0.38% | -0.96% | 101% | 105% |
| Class E | -0.59% | -0.60% | -1.98% | 101% | 103% |
| Overall | -0.31% | -0.15% | -1.00% | 102% | 103% |
| Class D | -0.70% | -0.10% | -1.94% | 101% | 102% |
| Class F (optional) | -0.12% | 0.60% | -0.36% | 101% | 104% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | 0.75% | 0.38% | 101% | 106% |
| Class C | -0.33% | -0.49% | -0.71% | 97% | 110% |
| Class E | -0.37% | -0.26% | -1.06% | 86% | 99% |
| Overall | -0.25% | 0.09% | -0.35% | 95% | 106% |
| Class D | -0.54% | -0.46% | -1.80% | 97% | 101% |
| Class F (optional) | -0.07% | 0.48% | -0.80% | 104% | 107% |

## Default initializers

In this test default initializers (value 154) are used for all contexts instead of trained initializers. Default initializers correspond to a probability estimate of about 50/50.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.19% | 0.24% | 100% | 106% |
| Class A2 | 0.08% | 0.10% | 0.15% | 101% | 104% |
| Class B | 0.30% | 0.62% | 0.58% | 107% | 107% |
| Class C | 0.67% | 1.19% | 1.18% | 99% | 104% |
| Class E | 0.68% | 1.39% | 1.04% | 101% | 103% |
| Overall | 0.38% | 0.72% | 0.66% | 102% | 105% |
| Class D | 1.46% | 2.76% | 2.35% | 93% | 99% |
| Class F (optional) | 0.37% | 0.60% | 0.72% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.51% | 0.38% | 0.61% | 105% | 107% |
| Class A2 | 0.39% | 0.54% | 0.74% | 102% | 106% |
| Class B | 0.94% | 1.31% | 1.09% | 98% | 104% |
| Class C | 1.40% | 1.64% | 1.03% | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.87% | 1.06% | 0.91% | 99% | 103% |
| Class D | 2.17% | 2.02% | 2.06% | 96% | 101% |
| Class F (optional) | 0.94% | 0.93% | 0.85% | 97% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.38% | 3.37% | 1.98% | 101% | 102% |
| Class C | 2.40% | 3.32% | 1.78% | 97% | 104% |
| Class E | 3.70% | 4.10% | 2.80% | 97% | 103% |
| Overall | 2.30% | 3.54% | 2.12% | 99% | 103% |
| Class D | 3.74% | 6.08% | 3.13% | 96% | 102% |
| Class F (optional) | 2.33% | 2.60% | 1.98% | 100% | 103% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.21% | 3.43% | 2.00% | 97% | 107% |
| Class C | 2.18% | 3.11% | 1.82% | 92% | 109% |
| Class E | 3.57% | 4.53% | 3.69% | 83% | 99% |
| Overall | 2.12% | 3.60% | 2.36% | 92% | 105% |
| Class D | 3.51% | 6.26% | 3.69% | 92% | 101% |
| Class F (optional) | 2.23% | 2.30% | 1.58% | 101% | 106% |

## Impact on CEs

Gains can be more substantial when applied to CE experiments.

### Retraining applied to CE5.1.1

In this CE, retraining yields a higher gain than for VTM.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5.1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.16% | 0.01% | 96% | 97% |
| Class A2 | -0.02% | -0.04% | -0.03% | 84% | 86% |
| Class B | -0.06% | -0.19% | -0.26% | 91% | 93% |
| Class C | -0.11% | -0.46% | -0.31% | 73% | 75% |
| Class E | -0.17% | -0.59% | -0.39% | 92% | 93% |
| Overall | -0.08% | -0.28% | -0.21% | 86% | 88% |
| Class D | -0.23% | -0.98% | -0.88% | 89% | 88% |
| Class F (optional) | -0.06% | -0.28% | -0.08% | 80% | 83% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE5.1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.46% | -0.39% | 81% | 89% |
| Class A2 | -0.17% | -0.31% | -0.22% | 75% | 83% |
| Class B | -0.36% | -0.57% | -0.70% | 82% | 86% |
| Class C | -0.57% | -0.96% | -1.07% | 83% | 87% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.60% | -0.64% | 81% | 86% |
| Class D | -0.97% | -1.03% | -1.18% | 90% | 90% |
| Class F (optional) | -0.29% | -0.51% | -0.45% | 85% | 89% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5.1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.37% | -1.27% | 83% | 95% |
| Class C | -0.60% | -0.73% | -1.43% | 72% | 77% |
| Class E | -0.65% | -1.79% | -1.82% | 79% | 85% |
| Overall | -0.50% | -0.84% | -1.46% | 78% | 86% |
| Class D | -1.08% | -1.09% | -2.30% | 76% | 79% |
| Class F (optional) | -0.43% | -0.35% | -1.04% | 80% | 86% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over CE5.1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.13% | -0.81% | 81% | 87% |
| Class C | -0.54% | -0.40% | -1.30% | 88% | 89% |
| Class E | -0.74% | -1.73% | -2.02% | 88% | 90% |
| Overall | -0.48% | -0.62% | -1.28% | 85% | 88% |
| Class D | -0.94% | -0.64% | -2.51% | 76% | 79% |
| Class F (optional) | -0.44% | -0.01% | -0.34% | 79% | 85% |

### Retraining applied to CE5.1.3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.12% | -0.02% | 99% | 100% |
| Class A2 | -0.02% | 0.06% | -0.04% | 105% | 101% |
| Class B | -0.06% | -0.10% | -0.08% | 101% | 102% |
| Class C | -0.10% | -0.15% | -0.29% | 102% | 102% |
| Class E | -0.15% | -0.28% | -0.45% | 98% | 102% |
| Overall | -0.07% | -0.08% | -0.17% | 101% | 102% |
| Class D | -0.21% | -0.54% | -0.44% | 95% | 98% |
| Class F (optional) | -0.05% | -0.16% | -0.11% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.18% | -0.22% | 101% | 102% |
| Class A2 | -0.08% | -0.11% | 0.04% | 101% | 102% |
| Class B | -0.22% | 0.00% | -0.36% | 99% | 100% |
| Class C | -0.42% | -0.52% | -0.55% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.20% | -0.30% | 100% | 101% |
| Class D | -0.71% | -0.49% | -0.49% | 101% | 99% |
| Class F (optional) | -0.14% | -0.24% | -0.28% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | 0.54% | -0.03% | 102% | 103% |
| Class C | -0.38% | -0.12% | -0.59% | 98% | 101% |
| Class E | -0.44% | 1.48% | 1.09% | 104% | 103% |
| Overall | -0.29% | 0.56% | 0.06% | 101% | 102% |
| Class D | -0.77% | 0.31% | -0.56% | 96% | 97% |
| Class F (optional) | -0.18% | -0.27% | -0.27% | 98% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | 0.49% | 0.20% | 99% | 106% |
| Class C | -0.41% | -0.32% | -0.50% | 97% | 101% |
| Class E | -0.20% | 1.26% | 0.26% | 99% | 101% |
| Overall | -0.23% | 0.41% | -0.02% | 99% | 103% |
| Class D | -0.72% | 0.66% | -1.00% | 102% | 103% |
| Class F (optional) | -0.01% | 0.77% | 0.27% | 99% | 101% |