| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0594 |
| --- | --- |

| Title: | Crosscheck of JVET-L0139 (CE3-related : Simplified MDMS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-L0139. It is confirmed that the cross-check results are identical with the proponent’s results.

Summary of Proposal JVET-L0139

JVET-L0139[1] proposes a modified derivation for multiple DM modes signalling (MDMS). The main point of the proposal is to reduce the number point access and context models from five to two shown in Figure 1. In the proposed method, five chroma intra candidate modes are constructed with the two DM points (BL, and TR as shown in Figure 1).

Figure 1 Example of corresponding sub-blocks for a chroma CB in I slice.

The chroma intra candidate modes list is generated as follows

- If BL = TR and BL is either Planar or DC      : BL, !BL, Vertical, Horizontal, and Mode 66

- If BL = TR and BL is neither Planar nor DC    : BL, BL-1, BL+1, Planar, DC

- If BL != TR and BL+TR < 2 			 : BL, TR, Vertical, Horizontal, and Mode 66

- If BL != TR and both BL and TR are not Planar and DC : BL, TR, Planar, DC, BL-1

- Else 					: BL, TR, Planar or DC, max(BL,TR)-1, max(BL,TR)+1

Figure 2 shows the flow chart of the proposed method.

Figure 2 Flow chart of the proposed method.

Source Code Analysis

The source code provided by the proponents was carefully analysed. It is confirmed that the implementation matches the description in JVET-L0139.

Simulation results

Simulations were conducted for AI/RA configurations and all test sequences specified in the CTC. The cross-check results are provided in the accompanying Excel documents. The cross checker could exactly reproduce the results provided by the proponent and reported in JVET-L0139. The measured encoder and decoder run time ratios are similar to those reported in JVET-L0139.

References

- J. Choi, J. Heo, S. Yoo, L. Li, J. Choi and J. Lim, “CE3-related : Simplified MDMS”, JVET-L0139 JVET 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018.

Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.