| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0597 |
| --- | --- |

| Title: | CE6-related: cross-check report of JVET-L0304 on Multiplication Free Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | E. Francois K. Naser | Tel: Email: | karam.naser@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-L0304 on Multiplication Free Transform. The cross-checker could exactly reproduce the results from the two proposed tests reported in JVET-K0304 for AI and RA configurations. The measured encoder and decoder run time ratios are reported to be of same order as the anchor (VTM2.0.1). It is also reported that the source code implementation was checked and is conform to the description.

# Summary of Proposal JVET-L0304

The proposal JVET-L0304 consists in replacing any multiplication in the MTS transforms by additions and shifts. Different versions are provided by the proponent, with different maximum number of the allowable terms for the shifts and additions.

2 versions were tested by the cross-checker: allowing 3 terms of shifts or allowing 4 terms of shifts.

# Source Code Analysis

The source code was analysed and  is confirmed to be in line with the description of the proposal.

# Simulation Results

Results were checked for AI and RA configurations. They are summarized in following tables and in attached xls files. Perfect matching with proponent results was obtained. Encoding and decoding runtime seem to be similar to the anchors one.

The version with 4 terms of shift has no impact on coding performance, while some loss is observed with the version with 3 terms shift.

3 terms of shifts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | -0.03% | 0.02% | 102% | 101% |
| Class A2 | 0.18% | -0.04% | -0.06% | 96% | 97% |
| Class B | 0.11% | 0.02% | 0.06% | 98% | 98% |
| Class C | 0.02% | -0.01% | 0.17% | 96% | 97% |
| Class E | 0.09% | -0.03% | -0.16% | 100% | 100% |
| Overall | 0.11% | -0.01% | 0.02% | 98% | 98% |
| Class D | 0.03% | 0.24% | -0.11% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.02% | 0.02% | 102% | 102% |
| Class A2 | 0.05% | -0.09% | -0.02% | 103% | 95% |
| Class B | 0.05% | 0.05% | 0.11% | 102% | 99% |
| Class C | 0.05% | 0.15% | 0.12% | 100% | 97% |
| Overall | 0.05% | 0.04% | 0.07% | 102% | 98% |
| Class D | 0.02% | -0.20% | 0.23% | 100% | 99% |

4 terms of shifts

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | -0.03% | 102% | 104% |
| Class A2 | 0.00% | 0.00% | -0.04% | 96% | 98% |
| Class B | 0.01% | 0.02% | 0.00% | 99% | 97% |
| Class C | -0.01% | 0.01% | 0.13% | 96% | 96% |
| Class E | -0.01% | 0.01% | -0.10% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class D | 0.01% | 0.18% | -0.01% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.07% | 0.06% | 99% | 103% |
| Class A2 | 0.02% | -0.16% | -0.06% | 99% | 96% |
| Class B | 0.00% | -0.09% | 0.20% | 100% | 99% |
| Class C | 0.03% | 0.14% | 0.08% | 99% | 97% |
| Overall | 0.01% | -0.01% | 0.09% | 99% | 98% |
| Class D | 0.00% | -0.10% | 0.10% | 100% | 100% |