| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0612 |
| --- | --- |

| Title: | Crosscheck of JVET-L0427: CE-15 related: Separate Palette Coding for Luma and Chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay 9276 Scranton Road, Suite 300 San Diego, CA 92121 USA | Tel: Email: | +1 (858) 210 4813 saurav.bandyopadhyay @interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

The contribution reports the cross-check for the results reported in the input document: JVET-L0427: CE-15 related: Separate Palette Coding for Luma and Chroma components [1]. The cross-check contribution reports the results of the proposed separate palette coding on top of the BMS-2.0.1 for the All-Intra Main 10, Random Access configurations for CE-15 test conditions. The source code as well as the configuration files for performed test were provided by the proponent Huawei. The cross-check confirms the results reported by the proponents, the run-times are comparable.

# Summary of JVET-L0427: Separate Palette Coding for Luma, Chroma

JVET-L0427 proposes an unified approach of Palette Coding in VVC codec with a separate tree approach. The proposed method is applied separately for Y component and for Cb/Cr jointly. BMS-2.0.1 software is used to provide BD-rate over VTM anchor. The cross-check confirms the results reported by the proponents.

# Simulation results

Simulations results reported in this contribution are based on the BMS2.0.1 software with Dual Tree approach disabled used as anchor. The results are generated using configuration for JVET-L0427.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (VTM, Dual-Tree disabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.04% | 0.08% | 115% | 104% |
| Class A2 | 0.12% | 0.04% | 0.09% | 115% | 104% |
| Class B | 0.07% | 0.00% | 0.07% | 115% | 104% |
| Class C | 0.08% | 0.00% | 0.11% | 115% | 106% |
| Class E | 0.11% | 0.01% | 0.17% | 117% | 107% |
| Overall | 0.08% | 0.02% | 0.10% | 115% | 105% |
| Class D | 0.12% | 0.07% | 0.06% | 114% | 104% |
| Class F (optional) | -9.12% | -9.05% | -10.20% | 124% | 88% |
| Class TGM 4:2:0 (optional) | -25.67% | -25.39% | -24.92% | 131% | 59% |

All Intra Main 10

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (VTM, Dual-Tree disabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.02% | -0.03% | 110% | 102% |
| Class A2 | 0.08% | 0.12% | 0.01% | 112% | 101% |
| Class B | 0.11% | 0.13% | 0.09% | 114% | 102% |
| Class C | 0.16% | -0.05% | -0.02% | 117% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.05% | 0.02% | 114% | 102% |
| Class D | 0.19% | 0.32% | 0.11% | 120% | 104% |
| Class F (optional) | -7.02% | -7.24% | -8.34% | 119% | 97% |
| Class TGM 4:2:0 (optional) | -12.78% | -13.89% | -13.02% | 126% | 85% |

Random Access Main 10

# References

[1] R. Chernyak, S. Ikonin, J. Chen, “CE15-related: Separate Palette Coding for Luma and Chroma components”, JVET-L0427, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.