| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0613 |
| --- | --- |

| Title: | Cross-check of contribution JVET-L0300 on Generic Motion Vector Difference Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yan Zhang, Wei-Jung Chien | Tel: Email: | yzh@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-L0300 related to generic motion vector difference coding.

In the summary of the simulation result, 0.0002%, -0.0001% and -0.0002% coding difference for Y, U, V channels are observed for Random Access config. It is discovered that the PSNR value of the last decimal position is different for some QP points of specific test sequences. The reason is that the PrintHexPSNR option is not enabled when running the cross-check simulation which is recommended to be turned on in the Common Test Condition document [1]. Apart from that, the cross-check simulation results match with the test results.

# Introduction

In JVET-L0300, instead of separately coding the x and y components of the Motion Vector Difference (MVD) and the corresponding positive/negative sign, it is proposed to jointly code the two MVD components by alternatively sending the MVD layer and index information [2]. By doing this, the worst-case context bins used could be reduced by half (4 bins to 2 bins) and sign bit of the MVD components can be omitted when coding MVD. The implementation matches with the description of JVET-L0300: the Greater Than 0 (GRT0) and the Greater Than 1 (GRT1) flags are context coded; the remaining layer ID (Layer - 2) is coded using Exponential Golomb code; The index is coded with truncated binary code.

# Test Result

The cross-check test is performed on top of BMS2.0.1 with VTM2.0.1 configurations. The result is shown in the following table, and it matches the result shown in JVET-L0300 except for the above-mentioned observations. Runtime in the reported result is not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.00% | 102% | 92% |
| Class A2 | 0.03% | 0.04% | -0.01% | 103% | 93% |
| Class B | -0.01% | 0.05% | -0.07% | 92% | 88% |
| Class C | -0.03% | -0.03% | -0.05% | 85% | 79% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.04% | 94% | 87% |
| Class D | 0.03% | -0.21% | -0.14% | 87% | 74% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over vtm2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.09% | -0.02% | 90% | 86% |
| Class C | 0.00% | -0.26% | -0.05% | 90% | 85% |
| Class E | -0.04% | -0.50% | -0.21% | 91% | 92% |
| Overall | -0.01% | -0.17% | -0.08% | 91% | 87% |
| Class D | 0.04% | -0.01% | -0.28% | 82% | 72% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution

# Reference

[1]	Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li and Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 Jul. 2018

[2]	Seethal Paluri, Mehdi Salehifar and Seung Hwan Kim, “CE4-related: Generic Vector Coding of Motion Vector Difference”, Document of Joint Video Experts Team (JVET), JVET-L0300, 12th Meeting: Macao, CN, 3–12 Oct. 2018