| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0629 |
| --- | --- |

| Title: | Cross-check of contribution JVET-L0185 on Luma 2xN and Nx2 Block Partitions Support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yan Zhang, Han Huang | Tel: Email: | yzh@qti.qualcomm.com hanhuang@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-L0185 related to Luma 2xN and Nx2 Block Partitions Support.

It is reported that the result obtained during this cross-checking matches the result described in JVET-L0185.

# Introduction

In JVET-L0185[1], it is proposed to further decrease the minimum luma width and height of a Coding Block (CB) to 2 pixels. With such design, 2xN and Nx2 luma partitions will be supported. In the extreme case, a 2x2 luma partition could be arrived. This technique is only applied to I-Slice. Since the partition tree for luma and chroma channels are separated, the chroma partition will not be influenced.

From the implementation, the mode storage unit is modified from 4x4 to 2x2 sample. Correspondingly, the deblocking filter is also changed to 2-line processing instead of 4-line processing. For the Multiple Transform Selection part, only DCT2 is applied for CB with size of 2xN or Nx2 luma samples. The implementation matches the description in JVET-L0185.

# Test Result

The cross-check test is performed on top of BMS2.0.1 with VTM2.0.1 configuration. The result is shown in the following table, and it matches the result shown in JVET-L0185. Runtime in the reported result is not accurate.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1ctc |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.05% | 0.00% | 108% | 117% |
| Class A2 | -0.05% | -0.04% | -0.10% | 113% | 112% |
| Class B | -0.13% | -0.08% | -0.01% | 111% | 114% |
| Class C | -0.58% | -0.27% | -0.19% | 111% | 106% |
| Class E | -0.33% | -0.21% | -0.24% | 109% | 115% |
| Overall | -0.24% | -0.14% | -0.10% | 110% | 112% |
| Class D | -0.77% | -0.23% | -0.61% | 113% | 103% |
| Class F (optional) | -4.73% | -3.69% | -3.80% | 113% | 117% |

# Patent rights declaration(s)

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution

# Reference

- Jicheng An, Yu-Chen Sun and Jian Lou, “AHG11 & CE1-related: Luma 2xN and Nx2 Block Partitions Support”, 12th Meeting: Macao, CN, 3–12 Oct. 2018.