| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0636_v2 |
| --- | --- |

| Title: | CE14: Crosscheck of CE14.3 (JVET-L326) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jacob Ström Per Wennersten Jack Enhorn Du Liu Kenneth Andersson Rickard Sjöberg Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46702672192 {jacob.strom, per.wennersten, jack.enhorn, du.liu, kenneth.r.andersson, rickard.sjoberg}@ ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The crosscheckers have confirmed BD-rate results for CE14.3a and CE14.3b (JVET-L0326). Runtimes have also been investigated and are found to be matching. A code review has also taken place, especially investigating the look-up table used to approximate the transfer function used to filter the Hadamard coefficients. The crosscheckers claim that this results in discontinuities in the transfer function. They further claim that this leads to quantization effects in intensity ramps. The updated version (JVET-L0656) has also been investigated. The crosscheckers claim that this leads to similar quantization effects.

# Introduction

The BD-rate figures computed by the authors for both CE14.1a and CE14.1b match with the BD-rate numbers provided by the proponent, and with run times, as seen in the result section.

In CE14.3 (JVET-L0326), samples are filtered by transforming a 2x2 area using the Hadamard transform. The resulting coefficients (except DC) are processed with a transfer function Fx as shown below,

Fx=x3x2+σ.              (Eqn 1)

The filtered coefficients are then inverse Hadamard transformed. This process is repeated for all 2×2 areas containing the sample and the result is averaged. For more details, see JVET-L0326.

In JVET-L0326, the transfer function in Equation 1 is approximated by a look-up table. Only positive values are tabulated, and for values larger than 127 the approximation Fx=x is used instead. Furthermore, not every value between 0 and 127 is tabulated. Instead subsampling is used, so that a total of 32 values are used (CE14.3a) or 16 values are used (CE14.3b).

Fig. 1: Original LUT (blue) compared to LUT of 14.3b, lowest qp range18-23

Taking the lowest qp-range (18-23) as an example, Figure 1 shows the transfer function with using the subsampled LUT of 14.3 (red) compared to a non-subsampled version (blue). As can be seen, this creates discontinuities at regular intervals.

These discontinuities will cause quantization effects in the filtered output. Consider the following slow brightness ramp of slope -5.5:

250   245   239   234   228   223   217   212 ...
250   245   239   234   228   223   217   212
250   245   239   234   228   223   217   212
...

After applying Hadamard, feeding the AC coefficients through the transfer function (LUT) and going back, this has become

249   246   238   235   227   224   216   213 ...
249   246   238   235   227   224   216   213 
249   246   238   235   227   224   216   213 
...

Figure 2 plots the intensities of the above scan line before (blue) and after (red) processing. As can be seen in the figure, the ramp has become more wobbly from the processing.

Figure 2. Effect of processing a brightness ramp of slope -5.5.

Discontinuities such as the one described above are present for both 14.3a and 14.3b for all tables. To avoid these discontinuities one could at most subsample the LUT once, i.e., having 64 items in the LUT.

Another characteristic of the transfer function in both 14.3a and 14.3 is that the LUT is only used for values up to a threshold value around 127. Above this, the approximation F(x) = x is used.

This creates another discontinuity. As can be seen in Figure 3, where the approximation is plotted for the highest qp-range (48-63), this discontinuity can be very strong.

Figure 3. Discontinuity in the transfer function of the highest qp range of 14.3a. The discontinuity goes from F(123)=35 to F(124)=124, and is therefore 89 levels high.

This particular discontinuity affects ramps with a slope of close to half the discontinuity. An example is the following brightness ramp, with slope -61.5:

961   900   838   777   715   654   592   531   469   408   346   285   223   162   100...
961   900   838   777   715   654   592   531   469   408   346   285   223   162   100
961   900   838   777   715   654   592   531   469   408   346   285   223   162   100
...

After transforming to Hadamard, using the transfer function on the AC coefficients and inverse transforming we get

950   911   827   788   704   665   581   542   458   419   335   296   212   173    89...
950   911   827   788   704   665   581   542   458   419   335   296   212   173    89
950   911   827   788   704   665   581   542   458   419   335   296   212   173    89
...

This creates clearly visible banding artifacts in the ramp, as can be seen in the 14x14 sample blocks of Figure 4. To avoid these discontinuities one would have to continue the LUT until the difference between F(x) and x is smaller than 4, which happens at 43 for the LUT corresponding to the smallest qp-range, 83 for next range, at 753 at for the following range, and never for table 3 and 4. To avoid both artifacts one would therefore need LUTs of sizes 22 (43/3), 42, 377, 1024 and 1024.

Figure 4. Brightness ramp of slope -61.5 before and after Hadamard processing.

# Conclusion

The Hadamard processing as described in CE14.3a and CE14.3b (JVET-L0326) has reduced complexity by subsampling the look-up table. This has introduced two types of discontinuities; small discontinuities in many parts of the transfer function, and a large discontinuity where the switch between LUT and F(x)=x takes place. These discontinuities affect certain brightness ramps in the image and can cause clearly visible artifacts. To avoid all discontinuities one would need LUT sizes of 22, 42, 377, 1024 and 1024 items, in total 2489 items. These items would need to be 11 bits for the last two qp-ranges.

# Crosscheck of JVET-L0656

The proponents of CE14.3 have uploaded a late contribution (JVET-L0656) where the filter from CE14.3b has been changed. The main difference is that the threshold where F(x) switches from using the LUT to using Fx=x has been moved from 123 to 239.

The proponents claim that this lowers the discontinuity by a factor of two (Section 3). We were not able to verify that. Instead we find that the size of the discontinuity has increased from 87 to 96.

The proponents show in Figure 5 that this solves the quantization issue with respect to a ramp of  slope 61.5, but that is unsurprising: Since the discontinuity has moved from 123 to 239, the new ramp with artifacts is going to be for a slope 239/2 = 119.5.

In fact this large discontinuity can still provide deviations of up to 37 intensity levels.

Furthermore, moving the large discontinuity from 123 to 239 has the effect of spreading out the 16 LUT values even further. There function F(x) is now constant for 16 values in a row, interrupted by smaller discontinuities of the form shown in Figure 1, but now with a magnitude of up to 18. This can lead to deviations of intensity of up to 7 intensity levels. Even a deviation of just 4 intensity levels will be equivalent to one intensity step in 8 bits.

Given that 10-bit data is important for applications such as HDR, we do not think that errors up seven intensity levels are appropriate. We therefore think that the current updated version in JVET-L0656 is not appropriate to include in a mandatory part of the pixel pipeline.

# Results

## All Intra 14.3a

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.29% | 0.24% | 110% | 113% |
| Class A2 | -0.35% | 0.31% | 0.32% | 116% | 113% |
| Class B | -0.42% | 0.29% | 0.32% | 109% | 112% |
| Class C | -0.83% | 0.23% | 0.42% | 107% | 111% |
| Class E | -0.54% | 0.28% | 0.19% | 112% | 117% |
| Overall | -0.48% | 0.28% | 0.31% | 110% | 113% |
| Class D | -0.61% | 0.47% | 0.01% | 106% | 109% |

## Random access 14.3a

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.23% | -0.25% | 105% | 103% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.69% | -0.02% | -0.27% | 106% | 103% |
| Class C | -0.75% | 0.10% | 0.14% | 106% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.67% | -0.14% | 0.53% | 103% | 100% |

## Low Delay 14.3a

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.74% | 0.33% | 0.52% | 101% | 103% |
| Class E | -0.75% | 0.31% | 1.09% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.90% | 0.75% | 0.28% | 107% | 114% |

## All Intra 14.3b

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.28% | 0.28% | 113% | 112% |
| Class A2 | -0.33% | 0.27% | 0.29% | 109% | 111% |
| Class B | -0.40% | 0.31% | 0.39% | 105% | 111% |
| Class C | -0.82% | 0.20% | 0.32% | 107% | 110% |
| Class E | -0.54% | 0.34% | 0.25% | 113% | 113% |
| Overall | -0.47% | 0.28% | 0.32% | 109% | 111% |
| Class D | -0.65% | 0.43% | 0.22% | 107% | 110% |

## Random access 14.3b

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.36% | -0.24% | 105% | 106% |
| Class A2 | -0.92% | -0.49% | -0.37% | 106% | 106% |
| Class B | -0.69% | -0.11% | 0.12% | 104% | 104% |
| Class C | -0.75% | 0.14% | 0.10% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -0.17% | -0.06% | 105% | 105% |
| Class D | -0.69% | -0.30% | 0.49% | 102% | 105% |

## Low Delay B 14.3b

|  | Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.57% | 0.49% | 0.69% | 108% | 104% |
| Class C | -0.76% | 0.31% | 0.42% | 101% | 104% |
| Class E | -0.67% | -0.66% | -0.48% | 99% | 101% |
| Overall | -0.66% | 0.14% | 0.31% | 104% | 103% |
| Class D | -0.88% | 0.92% | 0.31% | 105% | 105% |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).