| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0638-v2 |
| --- | --- |

| Title: | CE5-related: Retraining of context initialization values for CE5.1.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Christian Bartnik, Paul Haase, Stefan Matlage, Jan Stegemann, Detlev Marpe, Heiko Schwarz, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

New context model initialization values are proposed for tests CE5.1.4.1 and CE5.1.4.2 (naming is according to the CE5 summary report JVET-L0025). Both tests use the state-based probability estimator of JVET-K0430 using an 8 and a 12 bit state variable per context model. CE5.1.4.1 doesn’t use custom window sizes, CE5.1.4.2 uses custom window sizes. Only the initialization values for the states are updated in this proposal (not the custom window size parameters). For RA and LB, experimental results show that the version without custom window sizes benefits more from retrained context model initialization values than the version with custom window sizes.

# Test conditions and experimental results

The same test conditions as for CE5.1.4.1 and CE5.1.4.2 were used. Simulation results are currently incomplete and will be updated as soon as possible.

## Coding gains over CE5.1.4.1 and CE5.1.4.2

For both tests the version with retrained initialization values is compared against the version without retrained initialization values.

CE5.1.4.1 - No custom window sizes: New initialization values against old initialization values

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5.1.4.1_old |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | -0,03% | -0,11% | 105% | 103% |
| Class A2 | -0,02% | -0,09% | -0,07% | 103% | 103% |
| Class B | -0,06% | -0,14% | -0,14% | 103% | 100% |
| Class C | -0,05% | -0,38% | -0,37% | 103% | 98% |
| Class E | -0,09% | -0,64% | -0,44% | 104% | 97% |
| Overall | -0,05% | -0,25% | -0,22% | 104% | 100% |
| Class D | -0,13% | -0,94% | -0,84% | 104% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE5.1.4.1_old |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,19% | -0,26% | -0,27% | 110% | 107% |
| Class A2 | -0,19% | -0,19% | 0,02% | 109% | 105% |
| Class B | -0,41% | -0,43% | -0,57% | 109% | 101% |
| Class C | -0,64% | -0,60% | -1,14% | 107% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0,38% | -0,39% | -0,54% | 109% | 103% |
| Class D | -0,99% | -1,08% | -1,70% | 107% | 103% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5.1.4.1_old |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,57% | -0,42% | -0,84% | 107% | 98% |
| Class E | -0,48% | -1,52% | -2,26% | 116% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,90% | -0,65% | -2,18% | 107% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

CE5.1.4.2 – Custom window sizes: New initialization values against old initialization values

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5.1.4.2_old |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | -0,14% | -0,08% | 91% | 96% |
| Class A2 | -0,03% | 0,02% | -0,03% | 94% | 98% |
| Class B | -0,03% | -0,09% | -0,08% | 94% | 97% |
| Class C | -0,05% | -0,29% | -0,22% | 96% | 99% |
| Class E | -0,10% | -0,38% | -0,28% | 95% | 94% |
| Overall | -0,05% | -0,17% | -0,14% | 94% | 97% |
| Class D | -0,15% | -0,69% | -0,43% | 98% | 97% |
| Class F (optional) | 0,02% | -0,08% | 0,15% | 97% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE5.1.4.2_old |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,12% | -0,10% | -0,15% | 89% | 94% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0,20% | -0,13% | -0,20% | 89% | 90% |
| Class C | -0,37% | -0,62% | -0,47% | 91% | 92% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,70% | -0,86% | -0,59% | 92% | 92% |
| Class F (optional) | -0,05% | -0,33% | -0,06% | 89% | 89% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5.1.4.2_old |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,34% | -0,24% | -0,06% | 90% | 95% |
| Class E | -0,17% | -0,41% | 0,68% | 82% | 89% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,57% | 0,02% | -0,65% | 91% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Gains over CEM1 (according to JVET-K1025)

CE5.1.4.1 - No custom window sizes: New initialization values against CEM1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,22% | -0,05% | 107% | 102% |
| Class A2 | -0,09% | -0,20% | -0,18% | 105% | 103% |
| Class B | -0,02% | -0,17% | -0,06% | 105% | 101% |
| Class C | -0,03% | -0,46% | -0,49% | 105% | 98% |
| Class E | -0,06% | -0,54% | -0,46% | 105% | 99% |
| Overall | -0,03% | -0,31% | -0,24% | 105% | 100% |
| Class D | -0,10% | -1,16% | -0,97% | 106% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,26% | -0,31% | -0,42% | 111% | 106% |
| Class A2 | -0,35% | -0,16% | -0,12% | 109% | 106% |
| Class B | -0,48% | -0,56% | -0,64% | 110% | 102% |
| Class C | -0,65% | -0,96% | -1,22% | 108% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0,45% | -0,54% | -0,65% | 109% | 104% |
| Class D | -1,02% | -1,33% | -1,46% | 109% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,60% | -0,25% | -0,80% | 108% | 96% |
| Class E | -0,47% | -1,18% | -2,30% | 118% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,86% | -0,48% | -2,24% | 109% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

CE5.1.4.2 - Custom window sizes: New initialization values against CEM1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,08% | -0,16% | -0,27% | 101% | 98% |
| Class A2 | -0,04% | -0,14% | -0,02% | 102% | 101% |
| Class B | -0,08% | -0,52% | -0,37% | 103% | 99% |
| Class C | -0,11% | -0,85% | -0,77% | 104% | 99% |
| Class E | -0,15% | -0,88% | -0,73% | 103% | 96% |
| Overall | -0,09% | -0,53% | -0,44% | 103% | 98% |
| Class D | -0,22% | -1,39% | -1,23% | 104% | 101% |
| Class F (optional) | 0,01% | -0,33% | -0,09% | 103% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,39% | -0,09% | -0,54% | 102% | 105% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0,59% | -0,45% | -0,58% | 101% | 99% |
| Class C | -0,79% | -1,32% | -1,46% | 102% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1,27% | -1,80% | -1,60% | 103% | 100% |
| Class F (optional) | -0,35% | -0,54% | -0,41% | 102% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CEM1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,71% | -0,45% | -1,19% | 101% | 92% |
| Class E | -0,59% | -0,94% | -0,82% | 100% | 93% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1,04% | -0,47% | -2,65% | 102% | 96% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Gains over VTM-2.0.1

CE5.1.4.1 - No custom window sizes: New initialization values against VTM-2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,12% | -1,48% | -1,50% | 107% | 100% |
| Class A2 | -0,74% | -1,55% | -1,50% | 105% | 103% |
| Class B | -1,00% | -1,54% | -1,54% | 105% | 99% |
| Class C | -0,96% | -1,60% | -1,57% | 105% | 99% |
| Class E | -0,90% | -1,60% | -1,66% | 105% | 97% |
| Overall | -0,95% | -1,56% | -1,55% | 106% | 100% |
| Class D | -0,92% | -1,64% | -1,80% | 106% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,94% | -1,08% | -1,33% | 102% | 101% |
| Class A2 | -0,73% | -0,69% | -0,86% | 102% | 100% |
| Class B | -1,01% | -1,35% | -1,05% | 103% | 98% |
| Class C | -1,10% | -1,54% | -1,27% | 102% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0,96% | -1,22% | -1,13% | 102% | 99% |
| Class D | -1,26% | -1,80% | -1,61% | 103% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -1,07% | -1,05% | -0,70% | 102% | 95% |
| Class E | -1,24% | -1,41% | -2,40% | 102% | 94% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1,15% | -0,09% | -1,46% | 103% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

CE5.1.4.2 - Custom window sizes: New initialization values against VTM-2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,21% | -1,42% | -1,72% | 101% | 96% |
| Class A2 | -0,70% | -1,50% | -1,35% | 102% | 100% |
| Class B | -1,07% | -1,89% | -1,84% | 103% | 97% |
| Class C | -1,05% | -1,99% | -1,85% | 104% | 100% |
| Class E | -0,99% | -1,93% | -1,92% | 103% | 95% |
| Overall | -1,01% | -1,78% | -1,75% | 103% | 98% |
| Class D | -1,03% | -1,87% | -2,06% | 105% | 101% |
| Class F (optional) | -0,77% | -1,34% | -1,09% | 105% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,07% | -0,87% | -1,45% | 93% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -1,12% | -1,24% | -1,00% | 95% | 95% |
| Class C | -1,24% | -1,90% | -1,52% | 96% | 93% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1,52% | -2,27% | -1,75% | 98% | 97% |
| Class F (optional) | -0,72% | -1,11% | -0,93% | 95% | 91% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -1,18% | -1,25% | -1,09% | 95% | 91% |
| Class E | -1,36% | -1,23% | -0,92% | 87% | 88% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1,32% | -0,08% | -1,82% | 96% | 95% |
| Class F (optional) | -0,63% | 0,37% | -0,38% | 91% | 94% |

# Conclusion

When comparing both CE tests (CE5.1.4.1 and CE5.1.4.2) with new initialization values to CEM1, the advantage in coding gain of CE5.1.4.2 (with custom window sizes) over CE5.1.4.1 (without custom window sizes) is only 0.06% for AI, XX for RA, and XX for LB.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).