| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0647-v2 |
| --- | --- |

| Title: | BoG report on 360º video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG on 360 Video met on 5 Oct 2018 in two sessions, with informal subjective viewing conducted in between. The BoG also met on 7 Oct 2018. The BoG plans to meet again to discuss CE planning.

The BoG recommends:

- JVET-L0231: adopt to the VTM, for horizontal geometric padding of inter-prediction references for ERP/PERP (and other single face projection variants)

  - Also update 360 CTC to enable in the PERP anchor.

- JVET-L0238: adopt the proposed changes to 360Lib and the 360 CTC to add support for different chroma types.

  - The 360Lib document should also describe this more clearly, as it will impact client-end operation.

  - Is there any effect on the HEVC SEI messages for cube map? Issue should be raised to JCT-VC and OMAF.

The BoG suggests:

- VTM include support for sub-CTU tile sizes, to allow more flexible alignment of tiles with cube faces, which could be used to disable in-loop filters at tile face row boundaries.

- Continue CE13, with same coordinators

- Discussion about if 360º video specific tools can be included in a “Main” profile, or if a “360” profile might be defined.

# Contribution review

## CE13: Coding tools for 360° omnidirectional video (21)

JVET-L0033 CE13: Summary report on coding tools for 360° omnidirectional video [P. Hanhart, J.-L. Lin, C. Pujara]

Several different anchors were available for use, depending on the test case, including HEC, PHEC (Padded HEC), and PERP. The PHEC anchor by default has 4 luma samples of padding around face rows.

### Test 13.1 Padding and blending

Different sizes, 4 and 8 luma samples around face rows, of PHEC padding were studied. Padding caused objective bitrate penalty, as expected by amount of padding.

### Test 13.2 Intra prediction

Disabling intra prediction at discontinuities and intra prediction using spherical neighbors were studied. Very small objective gains were observed, but expect more impact on subjective quality.

### Test 13.3 Inter prediction

Spherical geometry padding of reference pictures for inter prediction was studied, using both the simpler cube face unpacking and more complicated geometric projection. Moving content benefits the most from this test case.

Test 13.3.1.c uses cube face unmapping, which is simpler for the decoder than the projection approaches, but more complex. It is agnostic to the exact projection of each cube face. Tests 13.3.2 and 13.3.3 use projection which require decoder knowledge and targeting of the particular projection method used, e.g. HEVC vs EAC vs CMP.

Test 13.3.2 had the biggest coding gain, averaging 1.61% for Lanczos filter and 1.6% for bilinear interpolation, with larger gains on some of the moving sequences.

Test 13.3.3 was for the PERP format, unlike HEC, which was used in most other tests. For ERP or PERP formats, face unpacking and geometric projection are both the same mathematical operation, so geometric projection is simpler than for the cube map based formats. It showed a modest gain of 0.22%, but expected to have more subjective impact.

### Test 13.4 In-loop filters

Both disabling in-loop filtering at the discontinuous boundaries and using spherical neighbors were tested. Disabling the filters is simpler. De-blocking, SAO, and ALF were all considered. Deblocking had the biggest objective impact, with disabling gaining 0.57% on average and spherical neighbors gaining 0.62%.

Test 13.4.1.c modifies the ALF to be more like the picture boundaries

Test 13.4.2 fetches different samples (spherical neighbors)  for the in-loop filtering

CE-related contribution JVET-L0423 disables ALF and SAO at face discontinuities, and uses spatial neighbors for deblocking but with a modification to the strength calculation. Also modifies the boundary strength, to avoid the need to access the extra info from spatial neighbors – QP, MV, etc. The similarity of the  MVs of the spatial neighbors don’t make sense anyway, given the rotation and translation.

### Test 13.5 Post-filtering

Dropped

### Test 13.6 projection format adaptation

HEC with adaptive frame packing and PHEC w/ pre-rotation were studied.

No subjective viewing, since wasn’t combined with an in-loop filter solution, so the deblocking artifacts jump around.

### Test 13.7 Combinations

Test 13.7.1 represents the best that could be done to avoid seam artifacts w/o a decoder change, assuming tile/cube face alignment, with padding and in-loop filter disabling at face discontinuities.

Test 13.7.4 combines the simplest forms of 3 tools, for in-loop filter, intra prediction, and inter prediction, with disabling of in-loop filters and intra prediction at face discontinuities, and spatial geometric neighbors with cube face unpacking. It got a coding gain of 1.29%.

Test 13.7.8 combines the most complex forms of tools in the same 3 categories - or in-loop filter, intra prediction, and inter prediction. In-loop filters and intra prediction using spatial neighbors, and projection based geometric padding of reference frames for inter prediction. It has 2 variants, (a) using a Lancos filter or (b) using bilinear interpolation for the projection. Both a and b had a 1.98% gain, demonstrating that the simpler bilinear interpolation could be used.

Will be interesting to subjectively view: 13.7.1, 13.7.4, and 13.7.8b

It was noted that the decoder runtimes can be reduced by 4% for tests 7.4 and 7.8 because of implementation improvement.

JVET-L0075 CE13: Hybrid Cubemap with Pre-rotation (Test 6.2) [C. Pujara, A. Konda, A. Singh, R. Gadde, W. Choi, K. Choi, K.P. Choi(Samsung)]

Analyzed first picture in the sequence. Looked at 10 degree increments of yaw rotation, calculated gradiants. Tried to align gradients with horizontal and vertical.

There was some loss for some sequences, including Landing, because the first frame was not very representative of the full sequence. A more sophisticated encoder algorithm, which included check of motion, could avoid losses.

Similar to the rotation SEI message in HEVC – would recommend carrying that forward.

JVET-L0211 CE13: Results on CE13.3.2, CE13.4.3 and CE13.7.7 [J. Sauer, M. Bläser (RWTH Aachen University)

JVET-L0228 CE13: HEC with 8 samples padding around face row (Test 1.1.a) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0229 CE13: HEC with intra prediction disabled across face discontinuities (Test 2.1) [P. Hanhart, Y. He, Y. Ye (InterDigital), C.-H. Shih, J.-L. Lin, C.-C. Ju (MediaTek)]

JVET-L0230 CE13: HEC with face row based geometry padding using projection with bilinear interpolation (Test 3.1.b) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0231 CE13: PERP with horizontal geometry padding of reference pictures (Test 3.3) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

Reviewed on 7 Oct 2018.

This contribution reports simulation results of CE13 test 3.3. For inter prediction, horizontal geometry padding of reference pictures is used over conventional repetitive padding at the left and right boundaries for the padded equirectangular projection (PERP). Results based on end-to-end WS-PSNR show that PERP with horizontal geometry padding of reference pictures provides {Y, U, V} BD-rate savings of {-0.22%, -0.29%, -0.38%} over PERP for VTM configuration at a similar complexity as repetitive padding. More importantly, it is reported that horizontal geometry padding significantly reduces the seam artifact issue for the ChairLift sequence.

At least 4 proposals have made proposals with geometry padding D0061, D0067, D0075, E0057.

Works for any single face projection - ERP and PERP or EAP, cylinder. For PERP, requires that the decoder be aware of the amount of padding used, so that it can be accounted for when calculating the reference sample locations.

Objective coding gain on moving sequences: ChairLift 0.73%, SkateboardInLot 0.53%, BranCastle 0.52% .vs PERP.

For moving camera sequences, the subjective quality in the informal viewing showed improvement over the best non-normative change solution studied in the CE.

BoG recommends to adopt JVET-L0231 to the VTM, and to update 360 CTC to enable in the PERP anchor.

JVET-L0232 CE13: HEC with deblocking and ALF disabled across face discontinuities (Test 4.1.c) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0233 CE13: HEC with adaptive frame packing (Test 6.1) [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0234 CE13: Combined test 7.5 [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0235 CE13: Combined test 7.6 [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0236 CE13: Combined test 7.8.b [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0345 CE13: Intra prediction using spherical neighbors (Test 2.2) [C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0346 CE13: Face row based geometry padding of reference pictures (Test 3.1.a and Test 3.1.c) [C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0347 CE13: De-blocking filter disabled across face discontinuities (Test 4.1.a) [S.-Y. Lin, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0348 CE13: In-loop filters disabled across face discontinuities (Test 4.1.b and Test 4.1.d) [S.-Y. Lin, L. Liu, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0349 CE13: In-loop filters using spherical neighbors (Test 4.2) [S.-Y. Lin, L. Liu, J.-L. Lin, H.-C. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0350 CE13: Padding and in-loop filters disabled across face discontinuities (Test 7.1) [S.-Y. Lin, L. Liu, J.-L. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0351 CE13: Intra prediction and in-loop filters disabled across face discontinuities, and unfolding-based padding (Test 7.3 and 7.4) [C.-H. Shih, S.-Y. Lin, L. Liu, J.-L. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0352 CE13: Intra prediction and in-loop filters using spherical neighbors, and geometry padding (Test 7.8.a) [C.-H. Shih, S.-Y. Lin, L. Liu, J.-L. Lin, S.-K. Chang, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-L0422 CE13: In-loop filters disabled across face discontinuities and post-filtering of seam artifacts (Test 7.2.a) [X. Huangfu, Y. Sun, L. Yu (Zhejiang Univ.)] [late]

## CE13 related – Coding tools for 360° omnidirectional video (4)

Contributions in this category were discussed XXday XX Oct XXXX–XXXX (chaired by XXX).

JVET-L0166 CE13-related: Subjective Quality Improvement for RSP [A. Singh (Samsung)] [late] [miss]

Providing information, not asking for action.

JVET-L0212 CE13-related: Results for experiments as CE13.3.2, CE13.4.3 and CE13.7.7 with PHEC and impact of rotation on the coding performance of PHEC [J. Sauer, M. Bläser (RWTH Aachen University)]

This document reports two experiment groups. 
In the first group 3 experiments which have been made similar to the CE13 sub-experiments experiments CE13.3.2, CE13.4.3 and CE13.7.7 are reported. Here, the same tools have been used on top of PHEC, while in the CE13 sub-experiments they were applied on top of HEC. It reports the results of geometry padding and adapted in-loop filters for 360° video as proposed initially in document JVET-J0023, implemented into the BMS-2.0.1 software. The VTM configuration was used for the anchor and experiments. 
CE13.3.2 investigates HEC with face based geometry padding of reference pictures. Using PHEC instead of HEC and compared to the BMS-2.0.1 reference, an average Y E2E WS-PSNR BD-rate loss of 0.95% for Random Access is obtained.
CE13.4.3 investigates HEC with in-loop filters using spherical neighbors. Using PHEC instead of HEC and compared to the BMS-2.0.1 reference, an average Y E2E WS-PSNR BD-rate loss of 2.10% for Random Access is obtained.
CE13.7.7 is the combination of CE13.3.2 and CE13.4.3. Using PHEC instead of HEC and compared to the BMS-2.0.1 reference, an average Y E2E WS-PSNR BD-rate loss of 0.95% for Random Access is obtained.
The second group of experiments investigates the impact of rotation on the coding performance of PHEC and padded EAC. Padding was added in a manner identical to PHEC to EAC. Both projection formats have been evaluated for “Roll” rotation ranging from 0 degrees to 315 degrees in steps of 45 degrees. Yaw and Pitch were kept constant to 0 degrees. PHEC outperforms padded EAC in most cases, but it should be noted that the performance difference does depend on the rotation. 
Further, the impact of rotation on the coding performance is evaluated, by comparing rotated PHEC against not rotated PHEC. There are losses of up to 4.78 % Y E2E WS-PSNR BD-rate, depending on the applied rotation. Of the investigated orientations, the original orientation is the best performing.

Not asking for any specific action.

JVET-L0237 CE13-related: Adaptive frame packing using chroma sample location type 1 [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0423 CE13-related: HEC with in-loop filters using spherical neighbors [Xuchang Huangfu, Yule Sun, Lu Yu (Zhejiang Univ.) [late]

## Not CE13 related

JVET-L0238 AHG8: Chroma sample location type support for 360Lib [P. Hanhart, Y. He, Y. Ye (InterDigital)]

The 360Lib supports projection format conversion between different chroma formats. However, when the 4:2:0 chroma format is used, several issues have been identified in the 360Lib and 360° CTC related to the chroma sample location type:

- With the configuration files used in the CTC, chroma sample location type 2 is assumed for the input and output, whereas the 360° video sequences in CTC are in type-0.

- The cross-component linear model (CCLM) tool in VTM-2.0 assumes chroma sample location type 0 when downsampling the reconstructed chroma samples. If the input video has a chroma sample location type different from the one assumed in CCLM mode, an average coding loss in the range of 1% to 2% is observed on the chroma components (JVET-L0239).

- The input and output chroma sample location types cannot be specified independently.

- Only chroma sample location types 0 and 2 are supported, but not always correctly.

- Input/output using chroma sample location type 0 is only supported by means of resampling between type-0 and type-2.

- Only WS-PSNR supports different chroma location types, other metrics only support type-0.

- When a multi-face project format such as the cubemap (CMP) or a cubemap-based projection formats is used, different rotations are applied to different faces, causing the chroma sample location type to be inconsistent among different faces within a frame packed picture.

This contribution proposes to fix these problems by modifying the 360Lib software and 360Lib configuration files as follow:

- Change the default chroma sample location type to type-0.

- Add parameters to specify the chroma sample location types of the input and output independently.

- Add support for all four chroma sample location types in 360Lib.

- Modify the mapping functions in 360Lib such that mathematically correct mapping and resampling is performed between all chroma sample location types in a single step.

- Modify the metric calculations in 360Lib such that mathematically correct calculation is performed considering the chroma types of the two inputs.

- Modify the 360Lib software such that multi-face project format has a consistent chroma sample location type within the entire frame packed picture.

It is reported that, with the proposed modification to the 360Lib, the average coding performance can be improved by -0.35% and -0.26% for the U and V components, respectively, based on end-to-end WS-PSNR for PERP encoding. For HEC encoding, the average coding gain is -1.47% and -1.03%for the U and V components, respectively.

360Lib always uses type 2 chroma format. But the content is type 0.

rotations for cube faces has weird interactions with chroma type locations

Rec 2020 requires type 2

type 1 is robust to rotations

CCLM assumes type 0. This is a general VVC issue, not just for 360.

Proposes to modify 360Lib to properly handle chroma sample location types, and to update the CTC.

BoG recommends to adopt the proposed changes to 360Lib and the 360 CTC to add support for different chroma types. The 360Lib document should also describe this more clearly, as it will impact client-end operation.

Is there any effect on the HEVC SEI messages for cube map? Issue should be raised to JCT-VC and OMAF.

# Subjective Viewing Observations

Informal subjective viewing was performed for most of the CE tests. QP37 was used for most tests, using two vertices for the sequences: ChairliftRide, Harbor, SkateboardInLot, Balboa, Broadway.  Evil viewport vertex has 3 seams, 2 of which are discontinuous face edges, and 1 of which is continuous face edge.

13.7.8.a and 13.7.8.b were noticeably better than others on the most challenging moving sequences, especially Chairlift.

For Test 13.7.8 Chairlift, the 2 discontinuous edges were improved through the tools, and the continuous edge seam artifact was still noticeable.

13.7.8 vs 13.7.4 showed improvement, but had multiple differences.

The deblocking filter artifact was the most significant, and overshadowed any other types of improvements if not addressed.

Padding amount (4 vs 8 pixels) didn’t matter much. 4 pixels was enough to get rid of the seam lines caused by deblocking.

Padding alone was insufficient in many cases, with seam artifacts visible (at QP37).

Padding plus disabling loop filter removed the “lines” at the discontinuous face edges, but discontinuities were still frequently noticeable for moving content.

QP37 was used in all tests. Would be useful to have some tests with lower QP values also, because overall quality was often poor.

Using spherical neighbors in in-loop filters improved continuity vs. disabling in-loop filters. Deblocking had the biggest effect. ALF had some effect on chroma.

Test 3.3 ERP geometric padding for ERP showed clear improvement for chairlift sequence.Some lower QP values were also shown.

In future tests, should address the in-loop filters, because filtering across discontinuous edges led to obvious subjective artifacts that blinded us to any other impacts.

# Discussion

Would be useful to do some analysis of implementation complexity of the proposed tools. Some analysis had previously be in done in JVET-E0057.

Should there be a separate profile for 360 video? Should raise in the track.

If a cube face was aligned with a tile or tile group, could disable in-loop filtering at tile/face boundaries. Would be desirable to have tile size granularity smaller than the CTU size.

BoG could meet again after track HLS discussion about tiles.

Question: How will a tile design handling ALF at tile boundaries when in-loop filtering at tile boundaries is disabled?

For inloop filtering, would be interesting to also see results for lower QP values, not just QP37.

For Test 3.3 ERP geometric padding (in JVET-L0231), would need some HLS, probably at the sequence layer, to indicate that the horizontal geometry padding is enabled. May also want to signal that the content is ERP format.  Some discussion about if the BoG should recommend action for Test 13.3.3, to be revisited.

BoG will meet again after HLS tile discussion in track.

## 7 Oct meeting discussion

L0231 presentation

BoG suggests some finer-grain tile size inclusion in VTM, to allow more flexible alignment of tiles with cube faces, which could used to disable in-loop filters at tile face row boundaries.

If finer grain tile sizes are not adopted, proposals could still be made to disable in-loop filters at tile face row boundaries. Such proposals would preferably include a full specification, including signaling of cube face locations, signaling to enable the “disable deblocking filter” tool, and decoding process to implement.

BoG recommends continuing CE13, with same coordinators. BoG will plan to meet again to discuss what should be tested.

BoG suggests some discussion in the track about if 360 tools can be included in a “Main” profile, or if a “360” profile might be defined.

Should we modify anchor definition to use tile/face alignment and disbling in-loopfilter at tile boundaries? The CTC anchor or the CE anchor? JVET has adopted tiles at this meeting. Changing the anchor relies on the VTM SW having tile support. Sometimes HLS SW is not available in the first release of a new VTM, so dangerous to require this in the CTC anchor. A CE anchor can include HEC w/ tiles and in-loop filter disabling.

CE may want to study 2 tiles and 6 tiles for HEC, corresponding to face rows and faces, respectively, and study overall impact, assuming tile VTM SW is available. Impact goes beyond in-loop filter disabling, as other prediction dependencies are broken.

CE could compare the L0231 approach with cube-based formats such as HEC.