| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0664-v1 |
| --- | --- |

| Title: | CE2-related: Test results of disabling 5x5 ALF for luma component |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VTM-2, adaptive loop filter can be applied using 5x5 or 7x7 filter shapes to luma component. The filter shape is controlled by signalling a flag alf_luma_type_flag at slice header. This contribution presents the results of disabling 5x5 filter for luma component, i.e. only 7x7 filters can be applied. Test results reportedly show 0.00%, 0.02%, 0.06% BD-rate loss for luma component for AI, RA and LDB configurations respectively.

# Experimental Results

The tests are performed using VTM-2.0.1 with 5x5 ALF luma filter disabled following common test configuration. The anchor is VTM-2.0.1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 106% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 107% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class A2 | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Class B | 0.02% | 0.01% | 0.00% | 101% | 102% |
| Class C | 0.03% | 0.04% | 0.04% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.04% | -0.02% | 0.01% | 98% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.06% | -0.10% | 101% | 105% |
| Class C | 0.08% | -0.14% | -0.23% | 99% | 95% |
| Class E | 0.08% | 0.26% | -0.09% | 99% | 100% |
| Overall | 0.06% | 0.00% | -0.14% | 100% | 100% |
| Class D | 0.09% | 0.32% | -0.37% | 101% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |