| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0670 |
| --- | --- |

| Title: | Simplified DMVR for inclusion in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Anand Meher Kotra, Biao Wang, Han Gao, Jianle Chen (Hauawei), Sriram Sethuraman (Ittiam), | Tel: Email: | semih.esenlik@huawei.com sriram.sethuraman@ittiam.com |
| Source: | Huawei Technologies Co.,Ltd., Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

This contribution document reports the result of the combination of CE9 tests to reduce the complexity of DMVR in BMS2.1. It is asserted that the proposed DMVR is algorithm provides significant reduction in complexity while increasing the coding gain. -1.67% luma BD-rate gain and -1.68% and -1.80% chroma BD-rate gain is achieved with 1% increase in encoding time and 6% increase in decoding time compared to BMS anchor with VTM encoder configuration.

The contribution reports the combination of following CE tests:

- Reference sample padding for eliminating memory BW increase (CE9.2.2)

- Integer based DMVR to eliminate intermediate interpolation filters and buffers (CE9.2.1)

- Use refined MV from top and top-left CU (CE9.1.4)

- Parametric error surface based sub-pel refinement (CE9.2.5)

- Disable DMVR for small blocks and subsampled MRSAD (CE9.2.9f)

- Early-termination based on MV difference between merge candidates(CE9.2.13a)

The proposed modifications are independently tested in CE9.

# Introduction

Decoder Side Motion Vector Refinement (DMVR) was first introduced in 4th JVET meeting in Chengdu [1]. It was later included in JEM software in the 5th JVET meeting and extensively tested since. Finally it was included in the BMS software in 10th JVET meeting in San Diego. In 11th JVET meeting in Ljubljana usage of refined motion vectors in the spatial MV prediction loop was disabled to address hardware design limitation of DMVR.

Present contribution introduces further complexity reduction measures for DMVR in order to address remaining hardware implementation challenges.

# Technical Description

## General design of proposed DMVR

According to the proposed DMVR design the following steps are applied:

- Step 0: The initial motion vectors are obtained as indicated by the merge index.

- Step 1: Compute block averages for search points (line-by-line summation can be applied first from which block averages for each search point can be obtained)

- Step 2: 5 point MRSAD cost calculation is applied and the motion vector is refined according to best cost point.

- Step 3: 8 point MRSAD cost calculation is applied and the motion vector is refined according to best cost point.

- Step 4: If the motion vectors are not changed in the step4, an error surface equation is solved in order to obtain the final refined motion vector.

- Step 5: Obtain the final motion compensated prediction according to the final refined motion vector using 8-tap DCTIF.

It is noted that the proposed DMVR design uses motion vector difference mirroring, which means that between the refined MV pair (MVL0’, MVL1’) and the initial MV pair (MVL0, MVL1) the following equation holds:

MVL0’ - MVL0 = MVL1 - MVL1’

## DMVR application criteria

DMVR is applied under the following conditions:

- Merge mode is applied

- Affine and ATMVP is not applied

- Current block is bi-predicted

- The following equation holds for reference pictures: POC_current – POC0 = POC1 - POC_current

- DMVR is disabled if ((w <= 8 AND h ==4) OR (h <= 8 AND w ==4))

## Usage of refined motion vectors

The refined motion vectors are used in deblocking operation and in the temporal motion vector prediction.

The spatial motion vector prediction use the refined motion vectors from top and top-left CTUs. If the candidate does not belong to top or top-left CTU, non-refined motion vectors are used in spatial MV prediction.

Requirement CE9 are satisfied also for wavefront parallelism as refined MV from the top-right CTU is not used in spatial MV prediction.

It is reported that the line memory requirement is increased only by 28bits per CTU in order to maintain the non-refined motion vectors of top-right CTU.

It is reported that no pipeline delay is incurred with the usage of refined motion vectors from the top-CTU row.

## Integer based DMVR to eliminate intermediate interpolation filters

In CE test 9.2.1, the 8-tap interpolation filter is eliminated and instead integer reference samples are used for MRSAD comparison process of DMVR. The integer reference samples are obtained by rounding the initial motion vector to the nearest integer sample coordinate before DMVR. This means that no interpolation filtering is necessary until the final refined motion vectors are obtained. Only at this step (after the final refined MV is obtained), 8-tap interpolation filtering is applied to obtain the final motion compensation results. It is noted that the final motion vector can still have fractional sample resolution, since the MV rounding operation is reverted before final motion compensated prediction.

The process is elaborated in the following figure:

Figure 1: Application of rounding before MRSAD search and inversion of the rounding operation in order to obtain refined MV.

## Memory BW requirement

In the proposed method padding is applied to luma and chroma reference samples before the final motion compensated prediction is obtained. Therefore the memory BW requirement of DMVR is same as regular bi-prediction.

## Parametric error surface based sub-pixel offsets estimation

Half-pixel based DMVR refinement stage is eliminated and replaced with parametric estimation of the refined motion vectors. The MRSAD costs calculated in the steps 3 and 4 are used to further refine the motion vectors.

- The center position cost and the costs at (-1,0), (0,-1), (1,0) and (0,1) positions from the center are used to fit a 2-D parabolic error surface equation of the form

Ex,y=Ax-x02+By-y02+C

where (x0, y0) corresponds to the position with the least cost and C corresponds to the minimum cost value. By solving the 5 equations in 5 unknowns, (x0, y0) is computed as:

x0=(E-1,0-E1,0)/(2(E-1,0+E1,0-2E0,0))

y0=(E0,-1-E0,1)/(2(E0,-1+E0,1-2E0,0)

For 1/16th-pel accuracy, just 4-bits in the absolute value of the quotient needs to be computed, which lends itself to a fast shifted subtraction based implementation of the 2 divisions required per CU.

- The computed (x0, y0) are added to the integer distance refinement MV to get the sub-pixel accurate refinement delta MV.

It is reported that 20 additions and 14 shift and 14 comparison operations are necessary to obtain sub-pixel accurate refined MV in this stage.

## Disabling DMVR for small CU

DMVR is disabled for small blocks that are 4x4 and 8x4 and 4x8 in size.

## Subsampled MRSAD

The MRSAD is calculated for every second row of the block to reduce complexity.

## MVD based early termination

The MV difference between the selected merge candidate and any of the previous ones in the same merge list is less than a pre-defined threshold (that is, ¼-, ½- and 1-pixel-wide intervals for CUs with less than 64 pixels, less than 256 pixels and at least 256 pixels, respectively).

# Complexity Analysis

The worst case complexity presented according to the CE9 evaluation criteria as follows:

| Description of processing steps | 1. Calculate block averages 2 perform 5 point MRSAD computation (integer refinement) 3.a. perform 8 point MRSAD computation (integer refinement) 3.b. Perform Error Surface calculation 4. Perform Final motion compensation |
| --- | --- |
| Data dependency within 128x128 region | None |
| Intermediate Buffers | 25x4bytes for MRSAD results |
| Usage of refined MVs | refined MV from top and top-left CTU is used in spatial MV prediction, Motion Compensation, deblocking and TVMP/ATMVP. |
| Number of operations for interpolation* | Adds: 2576, Mult: 2944 (no increase compared to bi-pred) |
| Number of operations for MRSAD* | 1536 |
| Additional operations* | DMVR condition: 5 if checks MRSAD comparisons: 13 comparisons Final MC averaging: 64 additions MVD early exit: 20 addition, 30 comparisons 20 additions, 14 shifts and 14 condition checks for error surface |
| Smallest block applying DMVR | 8x8 |
| On Chip memory accesses | 1 read for step 1 1 read for steps 2 1 read for steps 3.a and 3.b |
| External memory BW increase* | 0% (same as bi-pred) |

*The numbers are presented for 8x8 reference block which is the smallest block applying DMVR.

Table 1: Computational complexity assessment of the proposal

It is notes that 2 stage MRSAD search can be performed in a single stage. In this case 25 MRSAD checks are necessary, in this case 2954 additions are necessary per 8x8 block.

# Experimental results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1 - with VTM config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.05% | -1.74% | -2.10% | 103% | 103% |
| Class A2 | -2.47% | -2.57% | -2.65% | 101% | 103% |
| Class B | -1.36% | -1.39% | -1.44% | 102% | 107% |
| Class C | -1.19% | -1.34% | -1.40% | 102% | 110% |
| Class E |  |  |  |  |  |
| Overall | -1.67% | -1.68% | -1.80% | 102% | 106% |
| Class D | -1.06% | -1.23% | -1.26% | 102% | 110% |

Table 2: Experimental results of proposal

# Conclusion

This contribution document reports the results of combination of tests from CE9. The proposed modifications reportedly reduces the complexity of DMVR considerably with -1.67% luma, -1.68% and -1.80% chroma coding gain compared to VTM2.0.1 with 1% increase in encoding time and 106% in decoding time. It is proposed to be included into VVC.

# Reference

[1] “Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching”, X. Chen, J. An, J. Zheng, doc. JVET-D0029, October 2016

[2] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, doc. JVET-K1001, August 2018

[3] “ Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, S. Esenlik, Y.-W. Chen, F. Chen, doc. JVET-K1029, August 2018

[4] “ CE9: Test 9.2.6 (combines CE9.2.15/9.2.16 with elements of 9.1.4 and 9.2.5)”, S. Sethuraman, doc. JVET-L0173, October 2018

[5] “CE9: Report on the results of tests CE9.2.15 and CE9.2.16”, S. Esenlik, A.M. Kotra, B. Wang, H. Gao, J. Chen, C. Chen, W. Chen, M. Karczewicz, H. Liu, L. Zhang, K. Zhang, D. Luo, X. Xiu, Y. He, Y. Ye, doc. JVET-L0163, October 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).