| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0 |
| --- | --- |

| Title: | Crosscheck for CE3-related JVET-L0164 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jean-Marc THIESSE Didier NICHOLSON David GOMMELET | Email: Email: Email: Address: | jean-marc.thiesse@vitec.com didier.nicholson@vitec.com david.gommelet@vitec.com VITEC 99 rue Pierre Sémard 92320 Chatillon, France |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report for the JVET-L0164 contribution [1]. This contribution is related to the CE3 on Intra Prediction which proposes a new intra coding mode named DIMD (Decoder-side Intra Mode Derivation). This new intra mode allows an implicit signalling of intra angular modes without substituting its traditional explicit signalling. Both solutions are combined to bring better rate distortion performances. In the implicit signalling case, the angular mode is deduced, both at the encoding and decoding side, using a template of neighboring pixels to the current CU. Therefore, it can be used with any number of angular modes without additional signalling cost. In the JVET-L0164 contribution, the results are presented with 65 and 129 angular modes.

This document confirms that the crosscheck results and timing match those provided by the proponents. Authors think the contribution has a potential and can be further improved.

- Introduction

The source code was directly provided by the author and integrated on the VTM2.0.1 reference software. The code integrates several macros to allow different test configurations. For the first, or default, configuration, the macros need to be set with the following values:

ATM_DIMD				1
USE_129_MODES			1
DIMD_DISABLE_TOP_CTU		1
DIMD_TEMPLATE_SIZE		2

The new intra prediction mode is activated with the ATM_DIMD macro and using 129 angular modes as indicated by the macro USE_129_MODES. DIMD_TEMPLATE_SIZE macro indicates the template size to compute the DIMD intra mode and DIMD_DISABLE_TOP_CTU macro is an additional constraint on this template, such that pixels above the current CTU are discarded. Indeed, using samples above the CTU may have a significant complexity impact in regard to line buffering for the decoder.

In the second test, only 67 angular modes are used for the DIMD intra mode. Therefore,

USE_129_MODES			0 
This test presents the results when the DIMD mode is constrained to the actual angular mode of VVC.

Finally, in the third test, the template constraint on neighboring pixels at the top of the CTU is removed with

DIMD_DISABLE_TOP_CTU		0
This test is useful to evaluate the impact of the template constraint. Note that is is performed using the 129 angular modes.

The testing conditions defined in JVET-K1023 [2] were used with one configuration: All Intra (AI). The source code was compiled using MSVC-19.15, the encoding jobs ran on different Intel(R) Core(TM) i7 platforms and all decoding jobs on an Intel(R) Core(TM) i7-4600u. Therefore encoding times should be treated carefully.

The summary results are presented in the following section, while detailed results are available on the excel sheets attached to this contribution. We confirm that the following results match the ones obtained by the proponents.

- Experimental results

Mode detailed results are available in attached excel sheets.

Table 1 - Results with USE_129_MODES set to 1 DIMD_DISABLE_TOP_CTU set to 1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,25% | -0,16% | -0,14% | 120% | 102% |
| Class A2 | -0,19% | -0,16% | -0,19% | 135% | 101% |
| Class B | -0,22% | -0,11% | -0,15% | 111% | 101% |
| Class C | -0,31% | -0,19% | -0,10% | 125% | 102% |
| Class E | -0,44% | -0,31% | -0,39% | 135% | 102% |
| Overall | -0,28% | -0,18% | -0,18% | 123% | 102% |
| Class D | -0,32% | -0,11% | -0,37% | 126% | 106% |
| Class F | -0,35% | -0,38% | -0,26% | 125% | 103% |

Table 2 - Results with USE_129_MODES set to 0 DIMD_DISABLE_TOP_CTU set to 1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,24% | -0,19% | -0,19% | 101% | 102% |
| Class A2 | -0,14% | -0,08% | -0,12% | 107% | 101% |
| Class B | -0,17% | -0,06% | -0,03% | 101% | 103% |
| Class C | -0,24% | -0,18% | -0,07% | 96% | 103% |
| Class E | -0,37% | -0,34% | -0,26% | 104% | 103% |
| Overall | -0,22% | -0,16% | -0,12% | 101% | 102% |
| Class D | -0,33% | -0,02% | -0,26% | 103% | 105% |
| Class F | -0,33% | -0,27% | -0,26% | 96% | 104% |

Table 3 - Results with USE_129_MODES set to 1 DIMD_DISABLE_TOP_CTU set to 0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,28% | -0,11% | -0,18% | 108% | 103% |
| Class A2 | -0,22% | -0,22% | -0,21% | 107% | 101% |
| Class B | -0,25% | -0,15% | -0,07% | 99% | 103% |
| Class C | -0,33% | -0,20% | -0,08% | 107% | 104% |
| Class E | -0,51% | -0,28% | -0,36% | 105% | 104% |
| Overall | -0,31% | -0,19% | -0,16% | 105% | 103% |
| Class D | -0,33% | -0,08% | -0,32% | 98% | 108% |
| Class F | -0,39% | -0,45% | -0,36% | 100% | 105% |

- Software inspection

The code was reviewed, it is confirmed that each macro is adequately implemented and can be used to perform the tests described in section 1. The proponents add the ability to try 129 modes including all Angular directions.

- Summary

Cross-check of JVET-L0164 is performed and results match with the ones provided by the proponents. The complexity at the decoding side has very slightly increased despite using a more complex process to retrieve the DIMD intra angular mode.

- Acknowledgement

The research leading to this work has been supported by the FUI23 EFIGI project, BPIFrance and Region Ile de France.

- References

- E. Mora, A. Nasrallah, M. Raulet, “CE3-related: Decoder-side Intra Mode Derivation”, JVET-L0164, Macao, October 2018

- G. Van Der Auwera, J. Heo, and A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-K1023, Ljubljana, July 2018