| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau SAR, CN, 3–12 Oct. 2018 | Document: JVET-L0689-r1 |
| --- | --- |

| Title: | CE3-related: Comparison of Intra mode coding between L0222 and L0154 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Anand Meher Kotra, Semih Esenlik, Han Gao, Jianle Chen | Tel: Email: | biao.wang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This contribution evaluates the benefit of 6-MPM approach compared to 3-MPM that is used in VTM 2.0.1. In VTM2.0.1, left and above neighboring blocks shown as Fig.1 is used. In a number of proposal using 6 MPM approaches (such as L0219, L0220, and L0222), the positions of left and above neighboring blocks have been changed, shown as Fig. 2. In L0154 test 1, it showed that the position change of left and above blocks might bring some coding gain. Therefore, in order to align the comparison, in this contribution the 6 MPM approach is compared to L0154 test1 instead of VTM2.0.1, where the positions of left and above are the same. Experimental results of the proposal used in L0222 achieves following results:

Over VTM Anchor with constraint (AI, RA): − 0.26% Luma BD rate saving and -0.11% for RA.

# Introduction

For those proposals using 6-MPM approach, this contribution tries to evaluate the benefit of using 6-MPM with a more consolidate baseline, namely, L154 test1. In this way, the position of their neighboring blocks of left and above are aligned, as shown in Fig. 2.

L

A

Fig. 1 Neighbour intra modes considered in VTM 2.0

L

A

Fig. 2 Neighbour intra modes in L0154 and L0222

# Experimental Results

The table below shows the coding gain and processing time impact of L0222, compared to L0154 test1. The results of decoding/encoding time are run in different company’s cluster, therefore, the difference is huge. However, the coding gain can be well compared.

- VTM simulations

  - Anchor: L0154 Test1

  - Test: L0222

|  | All Intra Over MPM with 3 MPM position aligned |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.20% | -0.16% | 176% | 166% |
| Class A2 | -0.24% | -0.15% | -0.17% | 181% | 167% |
| Class B | -0.23% | -0.11% | -0.15% | 180% | 167% |
| Class C | -0.26% | -0.17% | -0.26% | 143% | 162% |
| Class E | -0.37% | -0.23% | -0.23% | 160% | 168% |
| Overall | -0.26% | -0.17% | -0.19% | 167% | 166% |
| Class D | -0.16% | -0.01% | -0.15% | 144% | 165% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

|  | Random Access Over MPM with 3 MPM position aligned |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.14% | -0.12% | 127% | 166% |
| Class A2 | -0.12% | -0.03% | -0.10% | 129% | 154% |
| Class B | -0.11% | -0.13% | -0.21% | 167% | 221% |
| Class C | -0.10% | -0.07% | -0.23% | 148% | 221% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.10% | -0.17% | 145% | 194% |
| Class D | 0.01% | 0.37% | -0.07% | 141% | 216% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

This contribution reports the results for L0222 compared to L0154 test1, where the positions of neighboring blocks of left and above are aligned. The results show using 6-MPM approach brings coding gain of −0.26% for AI, and -0.11% for RA.

# Reference

[1]“Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, A. Alshin et. al., doc. JVET-J0024, April 2018.

[2] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu doc. JVET-K1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-K1023, September 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).