| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0690 |
| --- | --- |

| Title: | CE4-related: Affine clean-up and constrained affine inheritance for local and line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Hongtao Wang, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, it’s proposed to use the same sub-block motion vector field for the motion compensation, merge/skip/AMVP list derivation, deblocking, and storage of the temporal motion vector predictors (TMVPs). The control point motion vectors (CPMVs) are used for the affine motion data inheritance (i.e. for merge mode and affine AMVP mode) if the candidate block is in the current CTU. However, the sub-block motion vectors are used for the affine motion data inheritance if the candidate block is in the above CTU. Therefore, no additional line buffer is required to store the CPMVs. It’s also proposed to use subblock MV at middle location for affine data inheritance from above CTU to avoid division by non-power of 2. In addition, it’s proposed to constrain the candidate block size to be at least 16x16 for affine motion data inheritance. So that the local buffer for CPMVs storage can be reduced. Finally, it’s also proposed to use left/bottom-left neighboring MV as the third CPMV for affine motion data inheritance from above CTU as in JVET-L0273. The BD-rate differences are 0.0x% in RA, -0.0x% in LD-B on top of VTM-2.0.1, and are 0.0x% in RA, -0.0x% in LD-B on top of VTM-2.0.1 plus already adopted affine tools.

# Introduction

The inherited affine motion prediction uses CPMVs of a neighboring affine coded block to derive the predicted CPMVs of current block. Therefore the CPMVs need to be stored for the inherited affine motion prediction. In the current VTM2.0.1, the CPMVs are stored mixed with the sub-block motion vectors. The CPMVs are stored in the top-left, top-right and bottom-left (if a PU uses 6-parameter affine motion models) sub-block locations of the sub-block motion vector field, overwriting the computed sub-block vectors for those locations. So, no additional buffer is used for the CPMV storage. However, a separate sub-block motion vector field is generated again for motion compensation. As discussed in [1], for a cost-sensitive decoder implementation, the sub-block motion vector field cannot be overwritten by CPMVs before it’s consumed by the motion compensation. Two methods are proposed in [1] to address this issue. One is to store CPMVs in a separate buffer, the other is use the same mixed MV field of CPMVs and sub-block vectors are used for motion compensation. The first method requires 768 more bytes than the second method in total memory size. The second method introduces nonnegligible loss in coding some sequences.

# Proposal

In this contribution, the following are proposed:

- Affine mode clean-up. The CPMVs are stored in a separate buffer. The CPMVs are only used in the affine motion data inheritance. The sub-block MVs are used for motion compensation, merge/skip/AMVP list derivation, deblocking, and storage of the temporal motion vector predictors. And the sub-block based motion compensation for ATMVP and affine mode are aligned.

- Affine motion data inheritance from above CTU. If the candidate PU for affine motion data inheritance is in the above CTU line, the subblock MVs in the linebuffer instead of the CPMVs are used for the affine MVP derivation. In this way, the CPMVs are only stored in local buffer.

- Use subblock MV at middle location for affine data inheritance from above CTU. Since the subblock MV represent the motion at the center of the subblock, the distance of two corner subblock MVs at the bottom of a candidate PU is neiW-4, wherein neiW is the width of the candidate PU. To avoid division by (neiW-4), which may not be power of 2, the subblock MV at the middle location is used. Such that the distance is neiW/2.

- Use left/bottom-left neighboring MV as the third CPMV for affine motion data inheritance from above CTU as in JVET-L0273[2]. However, it’s only applied to bi-prediction. If the candidate PU is 6-parameter affine and is bi-predition, then the third CPMV for the 6-parameter affine MVP is first derived by the left/bottom-left neighboring MV. If the left/bottom-left neighboring MV is not available, then use the 6-parameter affine MVP is degraded to 4-paramer affine MVP.

- Size constrain for the affine motion data inheritance as in JVET-L0281[3].  A candidate PU is available for affine motion data inheritance only when its width and height are both larger than or equal to 16. By the size constrain, the local buffer size is reduced from 6,144 (128*128/64*24) bytes to 1,536(128*128/256*24) bytes, wherein roughly 24 bytes for a set of CPMVs.

# Test results

Tests were performed by using the VTM-2.0.1 software and VTM-2.0.1 plus the already adopted affine tools in this meeting, and the common test conditions of random access and low delay B were followed.

When integrated on top of VTM-2.0.1, the summary results are provided in Table 1, the BD-rate differences are 0.0x% in RA, -0.0x% in LD-B.

When integrated on top of VTM-2.0.1 plus the already adopted affine tools, the summary results are provided in Table 2, the BD-rate differences are 0.0x% in RA, -0.0x% in LD-B.

Table 1 Summary results on top of VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.05% | 0.04% | -0.02% | 99% | 101% |
| Class C | 0.04% | 0.12% | -0.02% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | 0.10% | -0.10% | 99% | 100% |
| Class F (optional) | 0.05% | 0.04% | 0.06% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.04% | -0.01% | -0.07% | 98% | 100% |
| Class E | -0.16% | -0.55% | -0.17% | 99% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.12% | 0.55% | -0.11% | 100% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2 Summary results on top of VTM-2.0.1 plus adopted affine tools

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | -0.04% | 0.03% | 100% | 102% |
| Class C | 0.01% | 0.10% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | -0.01% | 0.03% | 98% | 98% |
| Class F (optional) | 0.02% | 0.00% | 0.02% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.04% | -0.10% | 0.04% | 100% | 103% |
| Class E | -0.13% | -0.13% | -0.16% | 100% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | 0.33% | -0.09% | 95% | 92% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

[1] M. Zhou, “CE4-related: Combined tests of JVET-L0046 and JVET-L0047”, JVET-L0048, the 12th JVET meeting, Macao, CN, 3–12 Oct. 2018.

[2] H. Huang, W.-J. Chien, Y. Han, Y. Zhang, M. Karczewicz, “CE4: Test results of CE4.1.10 and CE4.1.13”, JVET-L0273, the 12th JVET meeting, Macao, CN, 3–12 Oct. 2018.

[3] H. Huang, W.-J. Chien, M. Karczewicz, “CE4-related: Size constrain for inherited affine motion prediction”, JVET-L0281, the 12th JVET meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).