The derivation process for inherited control point motion vectors is modified as follows and highlighted in green.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block,

- a luma location ( xNb, yNb ) specifying the top-left sample of the neighbouring luma coding block relative to the top-left luma sample of the current picture,

- two variables nNbW and nNbH specifying the width and the height of the neighbouring luma coding block,

- the number of control point motion vectors numCpMv.

Output of this process are the luma affine control point vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1.

The variable isAboveCTU is derived as isAboveCTU = (yNb +nNbH==yPb && yPb % CtbSizeY == 0) ? 1 : 0.

If isAboveCTU is equal to 1 and inter_pred_idc[xNb][yNb] is equal to 3, the variable is6ParaAvail is set equal to 0, otherwise is6ParaAvail is set equal to 0.

If isAboveCTU is equal to 0, the following applies:

- The variables log2NbW and log2NbH are derived as follows:

log2NbW = Log2( nNbW )		(8-269)

log2NbH = Log2( nNbH ) 		(8-270)

- The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ xNb ][ yNb ][ 0 ] << 7		(8-271)

mvScaleVer = cpMvLX[ xNb ][ yNb ][ 1 ] << 7 		(8-272)

dHorX = ( cpMvLX[ xNb + nNbW − 1 ][ yNb ][ 0 ] − cpMvLX[ xNb ][ yNb ][ 0 ] ) << ( 7 − log2NbW )	(8-273)

dVerX = ( cpMvLX[ xNb + nNbW − 1 ][ yNb ][ 1 ] − cpMvLX[ xNb ][ yNb ][ 1 ] )  << ( 7 − log2NbW )	(8-274)

- The variables dHorY and dVerY are derived as follows:

  - If MotionModelIdc[ xNb ][ yNb ] is equal to 2, the following applies:

dHorY = ( cpMvLX[ xNb ][ yNb + nNbH − 1 ][ 0 ] − cpMvLX[ xNb ][ yNb ][ 0 ] ) << ( 7 − log2NbH ) 	(8-275)

dVerY = ( cpMvLX[ xNb ][ yNb + nNbH − 1 ][ 1 ] − cpMvLX[ xNb ][ yNb ][ 1 ] ) << ( 7 − log2NbH ) 	(8-276)

  - Otherwise (MotionModelIdc[ xNb ][ yNb ] is equal to 1), the following apply,

dHorY = − dVerX		(8-277)

dVerY = dHorX			(8-278)

Otherwise the following applies:

The variables log2NbW and log2NbH are derived as follows:

log2NbW = Log2( nNbW/2 )		(8-269)

log2NbH = Log2( nNbH/2 ) 		(8-270)

The variable yNbB is set equal to yNb+nNbH-1.

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = MvLX[ xNb ][ yNbB ][ 0 ] << 7		(8-271)

mvScaleVer = MvLX[ xNb ][ yNbB ][ 1 ] << 7 		(8-272)

dHorX = ( MvLX[ xNb + nNbW/2 ][  yNbB][ 0 ] − MvLX[ xNb ][ yNbB ][ 0 ] ) << ( 7 − log2NbW )	(8-273)

dVerX = ( MvLX[ xNb + nNbW/2 ][  yNbB][ 1 ] − MvLX[ xNb ][ yNbB ][ 1 ] )  << ( 7 − log2NbW )	(8-274)

The variables dHorY and dVerY are derived as follows:

- If MotionModelIdc[ xNb ][ yNb ] is equal to 2 and is6ParaAvail is equal to 1, the following applies:

dHorY = ( MvLX[ xNb ][ yNbB ][ 0 ] − MvLX[ xNb ][ yNbB – nNbH/2 ][ 0 ] ) << ( 7 − log2NbH ) 	(8-275)

dVerY = ( MvLX[ xNb ][ yNbB ][ 1 ] − MvLX[ xNb ][ yNbB – nNbH/2 ][ 1 ]  ) << ( 7 − log2NbH ) 	(8-276)

- Ohterwise (MotionModelIdc[ xNb ][ yNb ] is equal to 1), the following apply,

dHorY = − dVerX		(8-277)

dVerY = dHorX			(8-278)

If isAboveCTU is equal to 1, the variable xNb is set equal to xNb +2

The luma affine control point motion vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1 are derived as follows:

- The first two control point motion vectors cpMvLX[ 0 ] and cpMvLX[ 1 ] are derived as follows:

cpMvLX[ 0 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb − xNb ) + dHorY * (isAboveCTU ? 0 : yCb − yNb ) )	(8-279)

cpMvLX[ 0 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb − xNb ) + dVerY * ( isAboveCTU ? 0 : yCb − yNb ) )	8-280)

cpMvLX[ 1 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb + cbWidth − xNb ) + dHorY * ( isAboveCTU ? 0 : yCb − yNb ) )	 (8-281)

cpMvLX[ 1 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb + cbWidth − xNb ) + dVerY * ( isAboveCTU ? 0 : yCb − yNb ) ) 	(8-282)

- If numCpMv is equal to 3 and is6ParaAvail is equal to 1, the third control point vector cpMvLX[ 2 ] is derived as follows:

cpMvLX[ 2 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb − xNb ) + dHorY * ( yCb + cbHeight − isAboveCTU ? 0 : yNb ) )	(8-283)

cpMvLX[ 2 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb − xNb ) + dVerY * ( yCb + cbHeight − isAboveCTU ? 0 : yNb ) )	(8-284)

- Otherwise if numCpMv is equal to 3, the third control point vector cpMvLX[ 2 ] is derived in the following ordered steps:

- The sample locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb − 1, yCb + cbHeight ), respectively.

- The availability flag availableFlagLX[ 2 ] is set equal to 0 and both components of cpMvLX[ 2 ] are set equal to 0.

- The following applies for ( xNbBL, yNbBL ) with BL being replaced by A1 and A0:

- The availability derivation process for a coding block as specified in clause  6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, the luma location ( xNbY, yNbY ) set equal to ( xNbBL, yNbBL ) as inputs, and the output is assigned to the coding block availability flag availableBL.

- When availableBL is equal to TRUE and availableFlagLX[ 2 ] is equal to 0, the following applies:

- If PredFlagLX[ xNbBL ][ yNbBL ] is equal to 1, and DiffPicOrderCnt( RefPicListX[ RefIdxLX[ xNbBL ][ yNbBL ] ], RefPicListX[ refIdxLX ] ) is equal to 0, availableFlagLX[ 2 ] is set equal to 1 and the following assignments are made:

cpMvLX[ 2 ] = MvLX[ xNbBL ][ yNbBL ]		(8-xxx)

- Otherwise, when PredFlagLY[ xNbBL ][ yNbBL ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicListY[ RefIdxLY[ xNbBL ][ yNbBL ] ], RefPicListX[ refIdxLX ] ) is equal to 0, availableFlagLX[ 2 ] is set equal to 1 and the following assignments are made:

cpMvLX[ 2 ] = MvLY[ xNbBL ][ yNbBL ]		(8-xxx)

- When availableFlagLX[ 2 ] is equal to 1, the rounding process for motion vectors as specified in clause 8.3.2.14 is invoked with mvX set equal to cpMvLX[ 2 ], rightShift set equal to 2, and leftShift set equal to 2 as inputs and the rounded cpMvLX[ 2 ] as output.

- If availableFlagLX[ 2 ] is equal to 1, the following applies:

cpMvLX[ 2 ][ 0 ] = ( mvScaleHor + dHorX * ( xCb − xNb ) + dHorY * ( yCb + cbHeight − yNb ) )	(8-xxx)

cpMvLX[ 2 ][ 1 ] = ( mvScaleVer + dVerX * ( xCb − xNb ) + dVerY * ( yCb + cbHeight − yNb ) )	(8-xxx)

- The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to cpMvLX[ cpIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded cpMvLX[ cpIdx ] as output, with X being 0 or 1 and cpIdx = 0 .. numCpMv − 1.